Source:	Samsung Electronics Co., Ltd.

Title:	ITT4RT: On text for media configuration

Agenda Item:	15.2

Document for:	Discussion and Agreement

# Introduction

In the current draft CR (S4-201538), a multiple number of SDP attributes have been defined, each used to indicate a different type of video or image stream for ITT4RT.

Depending on the use case of the ITT4RT session to be established, certain combinations of attributes (video/image streams) may be allowed, whilst other combinations may not be logical as a negotiation outcome.  This document provides text to clarify these multiple attribute negotiation offers and outcomes

In one example, ITT4RT-Tx clients that support both 360-degree projected video and 360-degree fisheye video may include both 3gpp_360video and 3gpp_fisheye attributes as alternatives in the SDP offer, but an ITT4RT-Rx client shall include only one attribute (either 3gpp_360video or 3gpp_fisheye, based on support or selection) in the SDP answer.

Likewise, 3gpp_overlay, 3gpp_360bg attributes may be included with either 3gpp_360video or 3gpp_fisheye attributes as a negotiation outcome, as both spherical overlays and 360-degree backgrounds can be applied to either 360-degree projected or fisheye video.

# Changes

## *** Start change ***

# X.5	Media configuration

## X.5.1	General

Based on the architecture described in clause X.2, an SDP framework for immersive video and immersive voice/audio exchange for ITT4RT is presented to negotiate codec support, SEI messages for decoder rendering metadata, as well as RTP/RTCP signaling necessary for viewport dependent processing.

The SDP attributes 3gpp_360video, 3gpp_fisheye, 3gpp_overlay, 3gpp_360bg shall be used to indicate respectively a 360-degree projected video stream, a 360-degree fisheye video stream, a spherical overlay, and a 360-degree background image (or series of images or video). ITT4RT-Tx clients that support both 360-degree projected video and 360-degree fisheye video may include both 3gpp_360video and 3gpp_fisheye attributes as alternatives in the SDP offer, but an ITT4RT-Rx client willing to receive 360-degree video shall include only one attribute (either 3gpp_360video or 3gpp_fisheye, based on support or selection) in the SDP answer. 3gpp_overlay and 3gpp_360bg attributes may be included in the SDP answer independent on whether projected or fisheye video is selected, since spherical overlays and 360-degree background images are applicable to both types of 360-degree video streams. The detailed definition and usage of these SDP attributes are presented in the clauses below.

## *** End of changes ***

# Proposal

We propose to include the updated changes to the dCR text as specified in section 2 into the next version of the dCR.