Source:	Qualcomm Incorporated, Intel

Title:	Basic solution on conditional overlays

## Document for:	Agreement

## Agenda Item:	11.5

# 1. Introduction

In [1] it was clarified how video overlays are to be transported for ITT4RT using RTP streams as follows:

X.6.4.1 Video Media

Any visual media that is defined with the ‘m=video’ line, includes the attribute id and does not have the attribute a=3gpp_360video in the SDP may be rendered as an overlay by an ITT4RT-Rx client…

The authors of this document agree that using traditional RTP is a good approach to transporting the video overlays and appreciate the clarification.  Now that this has been clarified SA4 can also document and investigate solutions for providing the conditional overlay functionality based on the RTP transport approach.

In the rest of this paper the authors provide an alternative to enabling the conditional overlay functionality proposed in [1].

# 2. Proposal for a basic solution

Solution using RTP Pause/Resume

A solution for the support of conditional overlay functionality is described below.

- The overlay sender and receiver use SDP to negotiate the region of interest to be sent in the overlay RTP stream, e.g., using optional parameters <azimuth, azimuth_range, elevation, elevation_range> (as defined for the RTCP solution) in the a=3gpp_overlay attribute under the relevant media line corresponding to the related video.

- Using the a=rtcp-fb ccm pause attribute and parameter values as specified in [2] and [3], the overlay sender indicates the capability to support receiving and acting on PAUSE and RESUME requests targeted for RTP streams it sends.  The optional parameter setting of a=rtcp-fb ccm pause config=3 could be used by the overlay sender to indicate that it will only receive and react to PAUSE and RESUME requests but will not send these requests.

- Using the a=rtcp-fb ccm pause attribute and parameter values as specified in [2] and [3], the overlay receiver indicates the capability to support sending PAUSE and RESUME requests targeted for RTP streams it receives.  The optional parameter setting of a=rtcp-fb ccm pause config=2 could be used by the overlay receiver to indicate that it will only send PAUSE and RESUME requests and but does not support receiving these requests.

- The a=3gpp_overlay attribute may be used to indicate that an overlay stream is offered as a conditional overlay through the usage of a dedicated parameter conditional in the syntax of the SDP attribute. This would complement the inclusion of a=rtcp-fb ccm pause   to indicate in the offer-answer exchange the intention to send the overlay as a conditional overlay (from sender’s perspective) and to control its delivery using the RTCP-FB pause resume messages (from receiver’s perspective). The conditional parameter may also point into a related ROI to which the content of this overlay corresponds.

- Once the ITT4RT session begins, the overlay receiver can evaluate the conditions under which it wishes to receive the overlay.  Using the procedures specified in clause 8 of [3], the overlay receiver sends the appropriate PAUSE or RESUME request for that overlay to the sender.

- Upon receiving the PAUSE or RESUME request, the overlay sender can pause or resume transmitting the overlay in an RTP stream as requested.

Examples of conditions:

- When the overlay receiver determines that the current viewport is overlapping the overlay region by more than 50%, the overlay receiver will send a PAUSE request to the overlay sender.  If the overlay receiver later determines that the overlay will have an overlap smaller than 50%, then the receiver can send a RESUME request to the sender.

- When the user decides that she wants to see the full viewport without any overlays (i.e., the currently displayed slide in the overlay is not very relevant to the user or there is something significantly more important in the viewport), she can pause the transmission of the overlay at the sender by having her overlay receiver send a PAUSE request to the sender.

Advantages:

- RTP Stream Pause and Resume functionality has been specified in RFC 7728.

- Support of RTP Stream Pause and Resume functionality has been mandated in 3GPP TS 26.114 for MSMTSI terminals.

- Having the overlay RTP receiver pause the stream by a request avoids ambiguous operation at the overlay RTP receiver when the overlay RTP sender autonomously pauses the RTP stream based on some condition at the overlay sender.

- Having the overlay receiver directly control the pausing and sending of overlays enables receiver implementation of richer and more flexible “conditions” than can be standardized and communicated in a message to the overlay sender.  The PAUSE and RESUME messages enable this without requiring cancellation of an overlay or having to change conditions at the overlay sender.

- This provides a simple transport-level solution that avoids having to define and specify application-level functionality and signalling.

- While RTP Stream Pause and Resume may not be applicable for overlays that do not use RTP, it still provides the key benefit of being able to pause video overlays which consume the most resources.  Conditional overlay functionality is less relevant for lower bandwidth media such as images.

# 3. Proposal

Incorporate the text in clause 2 into the Permanent Document.

# 4. References

[1] S4aM200610 Proposed changes to draft CR – Overlays

[2] RFC 5104 Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)

[3] RFC 7728 RTP Stream Pause and Resume

SDP negotiation would be needed if the overlay region has to be changed.  However, it is unclear why the region of an overlay would have to change much during an ITT4RT session – e.g., view of the whiteboard, etc…. If support of such dynamics is truly needed then RTCP-FB messages could be used to also update or describe the region.