3GPP TSG-SA4 Meeting #112-e	S4-210292

# Online, 1st – 10th Februay 2021                                                         revision of S4-20174

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0007 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on OpenAPI Implementation of m1 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 27th January 2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is with the accordance with the new 3GPP procedures for documenting OpenAPI interfaces on 3GPP Forge. It contains the implementation of the m1 interface in the branch m1-dev |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR documents the implementation of the m1 interface by providing the corresponding YAML files. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.1, C.2, C.3, C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# C.1	General

This annex is based on the OpenAPI 3.0.0 specification [23] and provides corresponding representations of all APIs defined in the present specification.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Second Change

# C.2		Data Types applicable to several APIs

For the purpose of referencing entities in the Open API file defined in this annex, it shall be assumed that this Open API file is contained in a physical file named "TS26512_CommonTypes.yaml".

openapi: 3.0.0 servers: - url: '{apiRoot}/' variables: apiRoot: default: https://example.com description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501 info: description: Documentation of the 3GPP 5GMS Common Data Types. version: 1.0.0 title: 3GPP 5GMS Common Types tags: - name: 3GPP TS 26.512 description: 5G Media Streaming Common Types externalDocs: description: Find out more url: 'http://www.3gpp.org' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26512.htm' paths: /: description: 'This only defines a set of common reusable components' components: schemas: PathRewriteRule: type: object properties: originPathPattern: type: string distributionPath: type: string GeofencingLocators: type: object properties: locatorType: type: string enum: - cell - city locators: type: array items: type: string Pattern: type: object properties: pattern: type: string QoSSpecification: type: object properties: marBwUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' marBwDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxPacketLossRateDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm' maxPacketLossRateUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm' maxSuppBwDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxSuppBwUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' ApplicationSessionContext: type: object properties: afAppId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId' sliceInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai' dnn: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' ChargingSpecification: type: object properties: sponId: type: string sponStatus: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/SponsoringStatus' gpsi: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi' Percentage: type: number DurationSec: type: integer DateTime: type: string IpPacketFilterSet: type: object required: - direction properties: srcIp: type: string dstIp: type: string protocol: type: integer srcPort: type: integer dstPort: type: integer toSTc: type: string flowLabel: type: integer spi: type: integer direction: type: string ServiceDataFlowDescription: type: object properties: flowDescription: $ref: '#/components/schemas/IpPacketFilterSet' domainName: type: string M5QoSSpecification: type: object required: - marBwDlBitRate - marBwUlBitRate - mirBwDlBitRate - mirBwUlBitRate properties: marBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' marBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' desLatency: type: integer desLoss: type: integer TypedLocation: type: object required: - locationIdentifierType - location properties: locationIdentifierType: $ref: '#/components/schemas/CellIdentifierType' location: type: string OperationSuccessResponse: type: object required: - success properties: success: type: boolean reason: type: string CellIdentifierType: type: string enum: [ CGI, ECGI, NCGI ] SdfMethod: type: string enum: [5Tuple, 2Tuple, typeOfServiceMarking, flowLabel, domainName] ProvisioningSessionType: type: string enum: [downlink, uplink]

Third Change

# C.3	OpenAPI representation of the M1 APIs

## C.3.1	ProvisioningSessions API

openapi: 3.0.0 servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501 info: description: Documentation of the 3GPP M1 Provisioning interface. version: 1.0.0 title: 3GPP M1 interface Provisioning Sessions API tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1 Provisioning Sessions externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions: get: summary: 'List of all Provisioning Sessions' description: '' parameters: - name: provisioningSessionId in: query required: false schema: type: string description: A unique identifier of the Provisioning Session operationId: 'listProvisionings' responses: '200': description: List of Provisioning Sessions content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' post: summary: 'Create a new Provisioning Session' description: '' requestBody: description: initial configuration of new Provisioning Session required: true content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' parameters: [] operationId: 'Create a new Provisioning Session' responses: '201': description: Created /provisioning-sessions/{provisioningSessionId}: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: A unique identifier of the Provisioning Session get: summary: 'Get Provisioning Session' description: '' parameters: [] operationId: 'getProvisioningById' responses: '200': description: list of Provisioning sessions content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' put: summary: 'Update a Provisioning Session' description: '' parameters: [] operationId: '' responses: default: description: Default response delete: summary: '' description: '' parameters: [] operationId: '' responses: default: description: Default response externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: ProvisioningSession: type: object required: - id properties: id: type: integer format: int64 securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.2	ServerCertificatesProvisioning API

openapi: 3.0.0 servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501 info: description: Documentation of the 3GPP M1 Provisioning interface Certificate version: 1.0.0 title: 3GPP M1 interface tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1 Server Certificate externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/certificates: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the the Provisioning' post: summary: 'Create a new security certificate' description: '' responses: '200': description: 'The generated certificate' headers: Location: description: 'Location of the newly created server certificate' schema: type: string content: 'application/x-pem-file': schema: type: string get: summary: 'retrieve previously uploaded server certificate' parameters: - name: certificateId in: path required: true schema: type: string description: 'A unisque identifier of the server certificate' responses: '200': description: 'the server certificate' content: 'application/x-pem-file': schema: type: string externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: Certificate: type: object required: - "id" properties: id: type: string securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.3	ContentPreparationTemplatesProvisioning API

## C.3.4	ContentProtocolsDiscovery API

## C.3.5	ContentHostingProvisioning API

openapi: 3.0.0 servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501 info: description: Documentation of the 3GPP M1d Provisioning interface. version: 1.0.0 title: 3GPP M1d interface tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Content Hosting Configuration externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configurations: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning' get: summary: 'retrieve an existing Content Hosting Configuration' description: 'retrieve an existing Content Hosting Configuration' parameters: - name: contentHostingConfigurationId in: query required: false schema: type: string description: A unique identifier of the Content Hosting Configuration responses: '200': description: The Content Hosting Configuration content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' post: summary: 'create a Content Hosting Configuration' description: 'Used to retrieve an existing Content Hosting Configuration' responses: '201': description: Created headers: Location: description: 'Location of the newly created Content Hosting Configuration' schema: type: string content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/{contentHostingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: A unique identifier of the Provisioning Session - name: contentHostingConfigurationId in: path required: true schema: type: string description: A unique identifier of the Content Hosting Configuration get: summary: 'retrieve an existing Content Hosting Configuration' description: 'retrieve an existing Content Hosting Configuration' responses: '200': description: The identified Provisioning content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' put: summary: '' description: '' responses: default: description: '' delete: summary: '' description: '' responses: default: description: Default response /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/{contentHostingConfigurationId}/purge: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: A unique identifier of the Provisioning - name: contentHostingConfigurationId in: path required: true schema: type: string description: A unique identifier of the Content Hosting Configuration post: summary: 'Purge the content of the cache' description: '' requestBody: description: 'The RegEx pattern for resources to purge from the cache' required: true content: application/json: schema: $ref: '#/components/schemas/Pattern' responses: '200': description: 'Content was purged successfully' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: ContentHostingConfiguration: type: object required: - id - origin properties: id: type: integer format: int64 ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfiguration: type: array items: $ref: '#/components/schemas/DistributionConfiguration' IngestConfiguration: type: object properties: name: type: string path: type: string pull: type: boolean protocol: type: string enum: - dashifpush entryPoint: type: string xml: name: IngestConfiguration DistributionConfiguration: type: object properties: contentPreparationTemplateId: type: string targetDomain: type: string originDomain: type: string pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cacheConfiguration: type: array items: $ref: '#/components/schemas/CachingConfiguration' geoFencing: type: array items: $ref: '#/components/schemas/Geo' urlSignature: type: object properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: type: integer xml: name: DistributionConfiguration PathRewriteRule: type: object properties: originPathPattern: type: string distributionPath: type: string CachingConfiguration: type: object properties: urlPatternFilter: type: string cachingDirectives: type: object properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.6	ConsumptionReportingProvisioning API

openapi: 3.0.0 servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501 info: description: Documentation of the 3GPP M1d Provisioning interface Consumption Reporting. version: 1.0.0 title: 3GPP M1d interface tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Consumption Reporting externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/consumption-reporting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning' post: summary: 'Activate the consumption reporting procedure' description: 'Activate the consumption reporting procedure' responses: '200': description: 'the consumption reporting is successfully activate' get: summary: 'Retrieve the consumption reporting configuration' description: 'retrieve the consumption reporting configuration' responses: '200': description: 'a description of the consumption reporting configuration' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: ConsumptionReportingConfiguration: type: object properties: reportingInterval: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29122_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: type: number format: float minimum: 0.0 maximum: 100.0 locationType: $ref: '#/components/schemas/Geo' securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.7	MetricsReportingProvisioning API

## C.3.8	PolicyTemplatesProvisioning API

openapi: 3.0.0 servers: - url: '{apiRoot}/3gpp-m1/v1' variables: apiRoot: default: https://example.com description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501 info: description: Documentation of the 3GPP M1 Policy Template version: 1.0.0 title: 3GPP M1 Policy Template interface tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1 Provisioning Interface externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/policy-templates: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning Session' post: summary: 'Create a new Policy Template' description: 'create a new Policy Template' responses: '201': description: 'the newly created Policy Template' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' /provisioning-sessions/{provisioningSessionId}/policy-templates/{policyTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning Session' - name: policyTemplateId in: path required: true schema: type: string description: 'A unique identifier of the Policy Template' get: summary: 'retrieve a description of the Policy Template' description: 'retrieve a description of the Policy Template' responses: '200': description: 'a description of the Policy Template' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' put: summary: 'modify the configuration of a Policy Template' description: '' requestBody: content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '200': description: 'update a policy template' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' delete: summary: 'delete a Policy Template' description: 'delete a Policy Template' responses: '200': description: 'a Policy Template is deleted' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26512.htm' components: securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

Fourth Change

# C.4	OpenAPI representation of the M5 APIs

## C.4.1	ServiceAccessInformation API

openapi: 3.0.0 info: description: Documentation of the 3GPP M5 Media Session Handling API to aquire Service Access Information version: "1.0.0" title: M5 Media Session Handling to aquire Service Access Information. contact: email: Thorsten.Lohmar@ericsson.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' servers: - url: '{apiRoot}/3gpp-m5d/v1/service-access-information' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. paths: /{provisioningSessionId}/: get: operationId: 'ServiceAccessInformation_Get' tags: - ServiceAccessInformation parameters: - name: provisioningSessionId description: string identifying the service access information resource in: path required: true schema: type: string responses: '201': description: 'NA Session resource is created' headers: Location: description: 'Contains the URI of the newly created NA Sessionresource' required: true schema: type: string content: application/json: schema: $ref: '#/components/schemas/ServiceAccessInformationResource' '400': description: 'invalid input, object invalid' components: schemas: ServiceAccessInformationResource: type: object required: - provisioningSessionId - provisioningSessionType properties: provisioningSessionId: type: string provisioningSessionType: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/ProvisioningSessionType' StreamingAccess: type: object properties: mediaPlayerEntry: type: string format: uri ClientConsumptionReportingConfiguration: type: object required: - serverAddresses - locationReporting - samplePercentage properties: reportingInterval: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/DurationSec' serverAddresses: $ref: '#/components/schemas/ServerAddresses' locationReporting: type: boolean samplePercentage: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/Percentage' DynamicPolicyInvocationConfiguration: type: object required: - serverAddresses - validPolicyTemplateIds - sdfMethods properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' validPolicyTemplateIds: type: array items: type: string sdfMethods: type: array items: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/SdfMethods' externalReferences: type: array items: type: string ClientMetricsReportingConfiguration: type: array items: type: object required: - serverAddresses - samplePercentage - urlFilters - metrics properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' dataNetworkName: type: string reportingInterval: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string metrics: type: array items: type: string NetworkAssistanceConfiguration: type: object required: - serverAddress properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' # Common Data Type (26.512) ServerAddresses: type: array items: type: string format: uri

## C.4.2	ConsumptionReporting API

openapi: 3.0.0 info: description: Documentation of the 3GPP M5 Media Session Handling API version: "1.0.0" title: M5 Media Session Handling contact: email: Thorsten.Lohmar@ericsson.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' servers: - url: '{apiRoot}/3gpp-m5d/v1/consumption-reporting/{aspId}' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. aspId: default: MyAsp description: as defined 3GPP TS 26.512. tags: - name: 3GPP TS 26.512 description: 5G Media Session Handling API paths: /: post: summary: Upload a consumption report requestBody: description: Upload a consumption report. required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReport' responses: '201': description: 'A Consumption report is uploaded' components: schemas: ConsumptionReport : type: object required: - mediaPlayerEntry - reportingClientId - consumptionReportingUnits properties: mediaPlayerEntry: type: string reportingClientId: type: string consumptionReportingUnits: type: array items: $ref: '#/components/schemas/ConsumptionReportingUnit' ConsumptionReportingUnit: type: object properties: mediaConsumed: type: string startTime: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/DateTime' duration: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/DurationSec' Locations: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/TypedLocation'

## C.4.3	MetricReporting API

openapi: 3.0.0 info: description: Documentation of the 3GPP M5 Media Session Handling API version: "1.0.0" title: M5 Media Session Handling contact: email: Thorsten.Lohmar@ericsson.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' servers: - url: '{apiRoot}/3gpp-m5d/v1/metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. provisioningSessionId: default: MyProvSess description: as defined 3GPP TS 26.512. metricsReportingConfigurationId: default: MyQoEConf description: as defined 3GPP TS 26.512. tags: - name: 3GPP TS 26.512 description: 5G Media Session Handling API paths: /: post: summary: Upload a metrics report requestBody: description: Upload a metrics report. required: true content: application/xml: schema: type: string responses: '201': description: 'A metric report is uploaded'

## C.4.4	DynamicPolicies API

openapi: 3.0.0 info: description: Documentation of the 3GPP M5 Media Session Handling API version: "1.0.0" title: M5 Media Session Handling contact: email: Thorsten.Lohmar@ericsson.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' servers: - url: '3gpp-m5/v1/dynamicpolicies/' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. tags: - name: 3GPP TS 26.512 description: 5G Media Session Handling API paths: /policies/: get: tags: - DynamicPolicy summary: Forbidden due to privacy responses: '400': description: bad input parameter post: tags: - DynamicPolicy summary: Create a new Dynamic Policy resource responses: '201': description: 'dynamic policy resource is created' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' headers: Location: description: 'Contains the URI of the created Dynamic Policy resource, according to the structure: {apiRoot}/policies/{dynamicPolicyId}' required: true schema: type: string '401': description: Unauthorized /policies/{dynamicPolicyId}: get: tags: - DynamicPolicy summary: 'Provides all properties of a dynamic policy resources' parameters: - name: dynamicPolicyId description: 'string identifying the dynamic policy resource' in: path required: true schema: type: string responses: '200': description: 'Properties of dynamic policy resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '400': description: bad input parameter '401': description: Unauthorized put: tags: - DynamicPolicy summary: Adds / overwrites a dynamic policy resource description: Adds / overwrites a dynamic policy resource parameters: - name: dynamicPolicyId description: string identifying the dynamic policy resource in: path required: true schema: type: string requestBody: description: Setting policy parameters. required: true content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '201': description: item created '400': description: 'invalid input, object invalid' '401': description: Unauthorized '409': description: an existing item already exists patch: tags: - DynamicPolicy summary: modifies a dynamic policy description: Modifies a dynamic policy resource parameters: - name: dynamicPolicyId description: string identifying the dynamic policy resource in: path required: true schema: type: string requestBody: description: Setting policy parameters. required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '201': description: item created '400': description: 'invalid input, object invalid' '401': description: Unauthorized '409': description: an existing item already exists delete: tags: - DynamicPolicy summary: modifies a dynamic policy description: Deletes a dynamic policy resource parameters: - name: dynamicPolicyId description: string identifying the dynamic policy resource in: path required: true schema: type: string responses: '204': description: No Content (Successful deletion of the existing subscription) '400': description: 'invalid input, object invalid' '401': description: Unauthorized '409': description: an existing item already exists components: schemas: DynamicPolicy: type: object required: - dynamicPolicyId - policyTemplateId - serviceDataFlowDescription - provisioningSessionId properties: dynamicPolicyId: type: string policyTemplateId: type: string serviceDataFlowDescriptions: type: array items: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/ServiceDataFlowDescription' provisioningSessionId: type: string qosSpecification: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/M5QoSSpecification' enforcementMethod: type: string enforcementBitRate: type: integer

## C.4.5	NetworkAssistance API

openapi: 3.0.0 info: description: Documentation of the 3GPP M5 Media Session Handling API version: "1.0.0" title: M5 Media Session Handling contact: email: Thorsten.Lohmar@ericsson.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' servers: - url: '{apiRoot}/3gpp-m5d/v1/network-assistance/' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. tags: - name: 3GPP TS 26.512 description: 5G Media Session Handling API paths: /: post: tags: - NetworkAssistance summary: Create a new Network Assistance Session resource responses: '201': description: 'network assistance session resource is created' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' headers: Location: description: 'Contains the URI of the created Network Assistance Session resource' required: true schema: type: string '401': description: Unauthorized /{naSessionId}: get: tags: - NetworkAssistance summary: 'Provides all properties of a Network Assistance Session resources' parameters: - name: naSessionId description: 'string identifying the Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Properties of network assistance session resource' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '400': description: bad input parameter '401': description: Unauthorized put: tags: - NetworkAssistance summary: Adds / overwrites a network assistance session resource description: Adds / overwrites a network assistance session resource parameters: - name: naSessionId description: string identifying the network assistance session resource in: path required: true schema: type: string requestBody: description: Setting policy parameters. required: true content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '201': description: item created '400': description: 'invalid input, object invalid' '401': description: Unauthorized '409': description: an existing item already exists patch: tags: - NetworkAssistance summary: modifies a network assistance session resource description: Modifies a network assistance session resource parameters: - name: naSessionId description: string identifying the network assistance session resource in: path required: true schema: type: string requestBody: description: Setting policy parameters. required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '201': description: item created '400': description: 'invalid input, object invalid' '401': description: Unauthorized '409': description: an existing item already exists delete: tags: - NetworkAssistance summary: deletes a network assistance session resource description: Deletes a network assistance session resource parameters: - name: naSessionId description: string identifying the network assistance session resource in: path required: true schema: type: string responses: '204': description: No Content (Successful deletion of the existing resource) '400': description: 'invalid input, object invalid' '401': description: Unauthorized '409': description: an existing item already exists /{naSessionId}/recommendation: get: tags: - NetworkAssistance summary: 'Provides all properties of a Network Assistance Session resources' parameters: - name: naSessionId description: 'string identifying the Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Properties of network assistance session resource' content: application/json: schema: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/M5QoSSpecification' '400': description: bad input parameter '401': description: Unauthorized /{naSessionId}/boostRequest: post: tags: - NetworkAssistance summary: 'Provides all properties of a Network Assistance Session resources' parameters: - name: naSessionId description: 'string identifying the Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Properties of network assistance session resource' content: application/json: schema: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/OperationSuccessResponse' '400': description: bad input parameter '401': description: Unauthorized components: schemas: NetworkAssistanceSession: type: object required: - naSessionId properties: naSessionId: type: string serviceDataFlowDescription: type: array items: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/ServiceDataFlowDescription' policyTemplateId: type: string requestedQoS: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/M5QoSSpecification' recommendedQoS: $ref: 'TS26512_CommonTypes.yaml#/components/schemas/M5QoSSpecification' notficationURL: type: string