3GPP TSG SA WG4 #114e	S4-210738

# E-meeting, 18th – 29th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.116 | CR | <CR#> | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial positioning of the chroma samples for TV Video Profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Tencent, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For for BT.2020 and BT.2100, the relative spatial positioning of the chroma samples is different than the default and for BT.709. Signaling this difference is important and if not set, may lead to unnecessary problems shifts in the chroma presentation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Carry forwarded the recommendation of the HEVC specification for 3GPP media profiles using BT.2020 and BT.2100 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambigous specification leading to interop problems. Potential visual quality problems in the presentation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.5.3.5, 4.5.4.5, 4.5.5.5, 4.5.6.5, 4.5.7.5, 4.5.8.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

AVC	Advanced Video Coding

CVS	Coded Video Sequence

DASH	Dynamic Adaptive Streaming over HTTP

EOTF	Electro-Optical Transfer Function

FFS	For Further Study

HD	High Definition

HDR	High Dynamic Range

HLG	Hybrid Log Gamma

HRD	Hypothetical Reference Decoder

HEVC	High Efficiency Video Coding

MBMS	Multicast Broadcast Multimedia Service

MPD	Media Presentation Description

NAL	Network Abstraction Layer

PPS	Picture Parameter Set

OETF	Opto-Electronic Transfer Function

PQ	Perceptual Quantization

PSS	Packet Switch Streaming

RAP	Random Access Point

SDR	Standard Dynamic Range

SEI	Supplemental Enhancement Information

SPS	Sequence Parameter Set

TV	Television

UHD	Ultra High Definition

VCL	Video Coding Layer

VUI	Video Usability Information

=====  CHANGE  =====

#### 4.5.3.5	Colour information

A Bitstream conforming to the H.265/HEVC Full HD Operation Point shall use either Recommendation ITU-R BT.709 [3] colorimetry or Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance.

-	BT.709 [3] shall be signalled by setting colour_primaries to the value 1, transfer_characteristics to the value 1 and matrix_coeffs to the value 1.

-	If BT.2020 [4] is used,

-	it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

A Receiver conforming to the H.265/HEVC Full HD Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 [3] and ITU-R BT.2020 [4] colorimetry. Such a Receiver should support ITU-R BT.2020 [4] signalling and provide an appropriate mapping of the signal to the supported colour space of the device.

NOTE:	Colour spaces are not associated to any particular spatial resolution.

=====  CHANGE  =====

#### 4.5.4.5	Colour information

A Bitstream conforming to the H.265/HEVC UHD Operation Point shall use Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance.

-	If BT.2020 [4] is used,

-	it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set, the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

A Receiver conforming to the H.265/HEVC UHD Operation Point shall be capable of decoding Bitstreams that use ITU-R BT.2020 [4] colorimetry. Such a Receiver should support ITU-R BT.2020 [4] signalling and provide an appropriate mapping of the signal to the supported colour space of the device.

=====  CHANGE  =====

#### 4.5.5.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC Full HD HDR Operation Point that uses PQ HDR shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to the value 16,

-	matrix_coeffs shall be set to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

This signalling implies that BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

=====  CHANGE  =====

#### 4.5.6.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC UHD HDR Operation Point that uses PQ HDR shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to the value 16,

-	matrix_coeffs shall be set to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

This signalling implies that Recommendation BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

=====  CHANGE  =====

#### 4.5.7.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC Full HD HDR HLG Operation Point shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to either the value 18, or to the value 14. In the latter case, the Bitstream shall also contain the alternative_transfer_characteristics SEI message. The alternative_transfer_characteristics SEI message shall be inserted at each RAP, and its parameter preferred_transfer_characteristics shall be set to the value 18.

-	matrix_coeffs shall be set to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set	the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

This signalling implies that BT.2020 [4] colorimetry in non-constant luminance and Hybrid Log Gamma (HLG) opto-electronic transfer function (OETF)  as defined in Recommendation ITU-R BT.2100 [11] are in use.

=====  CHANGE  =====

#### 4.5.8.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC UHD HDR HLG Operation Point shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to either the value 18, or to the value 14. In the latter case, the Bitstream shall also contain the alternative_transfer_characteristics SEI message. The alternative_transfer_characteristics SEI message shall be inserted at each RAP, and its parameter preferred_transfer_characteristics shall be set to the value 18.

-	matrix_coeffs shall be set to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

This signalling implies that Recommendation BT.2020 [4] colorimetry in non-constant luminance and Hybrid Log Gamma (HLG) opto-electronic transfer function (OETF)  as defined in Recommendation ITU-R BT.2100 [11] are in use.