3GPP TSG-S4 Meeting #114-e	S4-210768

# 19-28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.803 | CR |  | rev |  | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User-generated live streaming walkthrough |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EMSA |  |  |  |  |  | Date: |  |  | 11 May 2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provides mapping of the user-generated live streaming use case to the EMSA call flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

Annex <A> (informative):
Selected Use Case Mapping

# A.X	User-generated live streaming

# A.X.1	Service setup

The user-generated live streaming use case covers scenarios where various processing may occur depending on the requested class of service. In many sessions, the service may occur in the cloud or at an edge close to the capturing device. In this use case, the Application Provider defines various classes of service.

Each class of service (Service Class) provides a subset of specific features from the list below:

- Quality improvement such as

  - Upscaling

  - Light correction

  - Stabilization

  - Audio quality improvement

- Enriching and adding interactivity features such as

  - Dubbing and captioning

  - Overlaying and tagging

  - Indexing and key feature detection

  - Navigation improvements

- Multirate encoding of content

  - Content-aware encoding

  - Low latency

  - Just in time encoding and transcoding

  - Multi-path encoding for on-demand content

- Splicing and ad insertion

  - Splicing content and manifest signaling

  - Ad insertion queues and information

  - Language setting and other service metadata

- Applying content protection and DRM

For each service class (SR), the Application provider defines a Service Class Requirements (SCR) for 5GMS AS running the service, which may include one or more of the following aspects:

- EAS type

- Hardware resources:

  - Compute

  - Graphical compute

  - Memory

  - Storage

- Connectivity

  - Bandwidth

  - Latency

  - Maximum request rate

  - Maximum response time

- Availability

- Functional support

  - Accelerated encoders/transcoders

  - Quality improvement functions

  - Content enriching and adding interactivity

  - Splicing and ad-insertion tools

  - Stabilization

For each user, the Application Provider maintains three objects:

- Service Class (SC)

- Service Class Requirements (SCR)

- The audience Scale and Geographical Distribution (SGD)

While SC and SCR are static, the SGD gets updated with each use of service.

Figure A.X-1 shows the high-level call flow for this use-case, which is based on the client-driven edge discovery of 6.3.2.

Figure A.X-1: User-generated live streaming walkthrough

The steps are:

- The Application Provider requests provisioning a session for user-generated live stream service.

- The 5GMS-Aware Application requests the start of service from the Application Provider.

- The Application Provider, using the user’s SC, SCR, and SGD in its database and the current user location, provides the 5GMS-Aware Application an EAS Profile (as defined in TS23.558 section 8.2.4).

- The 5GMS-Aware Application requests locating the server that can provide the class of service the user is signed up for.

- A new 5GMS AS is provisioned if needed.

- The 5GMS AF provides the list of suitable 5GMS AS and their information to MSH/EEC.

- The Application/MSH/EEC selects the best 5GMS AS instance based on the user class of service and the user’s location.

- The uplink streaming starts. The media is uplinked streamed to the 5GMS AS, the content is prepared according to the service class requirements and is delivered to distribution (CDN).

## A.X.2	Service relocation

In this use case, the relocation of 5GMS AS may occur due to various reasons:

- The relocation of the user.

- Audience diversity/geographical change may require increase transcoders.

- Change of service class which requires more computational resources

In any of the above cases, the service may need to be transferred to a new 5GMS AS. The relocation may be performed according to 6.4 using the ACR detection entity in Table A.X-1.

Table A.X-1: ACR detection entities for each relocation reason

| Relocation reason | ACR detection entity |
| --- | --- |
| User relocation | MSH/EEC |
| Audience diversity increase | 5GMS AS/EAS |
| Change of service class | The Application/MSH/EEC |