3GPP SA4#114-e meeting	S4-210798

May 18th – 28th 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR |  | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | S4-210798 ITT4RT: Presentation Overlay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | How to handle presentation type content in ITT4RT is on requrement of the work item and currently not sufficiently addresed in 26.114 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed a new section Y.6.4.4. to add functionality and message flows to handle signalling, detection and replacement of presentation content in the 360-degree content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to handle presentation type content in ITT4RT is not poperly addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.6.4.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== 1st CHANGE  =====

## Y.6.4.4	ITT4RT presentation overlay (screen share)

Y.6.4.4.0. General

One common situation in a meeting is to present additional material (e.g., slides, video, notes) on a display (screen or projector). When capturing such a display with a 360-degree camera, this can lead to significant quality degradations, based on the characteristics of the camera, display and lighting conditions. Simply most setups will not allow to capture both users and a display in high detail and ideal lighting, further the display refresh rate and camera capture rate are often mis-aligned. To mitigate this problem the ITT4RT client allows to replace the captured content with the original presentation material. We can consider the replacement of image data in the 360-degree video as a special case of overlays that should either be handled in the sending client of the 360-video or in the network (MRF/MCU) in the following way:

- Signal that content replacement is available

- Signal material as display content in 360-recording

- Identify position and orientation of content in 360-recording

- Replace content or signal overlay parameters

Y.6.4.4.1. Signal that content replacement is available

Currently the 360-degree video is indicated with the “a=3gpp_360video” attribute in the SDP negotiation (section 6.1). In order to indicate that content overlay replacement is available the SDP negotiation should add a new attribute “a=3gpp_360video_overlayreplacement:true”. This can either be done in the SDP offer by the sending 360-degree ITT4RT client or by the response from a central MRF/MCU entity.

Note: this step can be skipped if the replacement is fully handled by the 360-degree sending client (i.e., this client both is responsible for capturing the 360-degree content and the display of the presentation content)

Y.6.4.4.2. Signal material as display content in 360-recording

The availability of the presentation content should be signalled with the SDP parameter “a=content:slides”[81].

Note: this step can be skipped if the replacement is fully handled by the 360-degree sending client (i.e., this client both is responsible for capturing the 360-degree content and the display of the presentation content)

Y.6.4.4.3. Identify position and orientation of content in 360-recording

How the position and orientation is determined should be left as implementation detail that does not need further specification. The output of this analysis shall include the position of the content in the 360-degree video with the associated overlay characteristics to overlay/replace the image accordingly (see Y.6.4.3.2 Sphere-relative Overlay Configuration).

Note: Ideally while receiving both the 360-degree video and the presentation content the region should be identified automatically. However, a manual process could also be possible when handled directly by the sending UE.

Y.6.4.4.4. Replace content or signal overlay parameters

The solution is based on the definition of OMAF edition 1 that the remote users “viewing position is the centre of the unit sphere” [179] of the 360-degree image of the conference room. This means that all users view the 360-degree conference from the centre of the sphere, which is the capture position of the 360-degree camera.

Two options to replace content are possible, a) replace content directly in the 360-degree video (by injecting and re-encoding an adjusted version of the content given the identified overlay characteristics) and b) sending the video separately as overlay in the way specified in clause Y.6.4.3.2.

Replacing the content directly in the 360-degree video can be done either in the sending client of the 360-degree video or in the network (MRF/MCU).