Source:	Ericsson LM

Title:	Architecture mapping for AR conversational services and call flow

Agenda Item:	10.9 (FS_5GSTAR)

Document for:	Agreement

# Introduction

TR 26.998 describes several AR conversational services which are listed in Table 6.1. The mapping to 5G system architecture for AR conversational services needs to be defined. In the telco on 11th of May we discussed alternative options for supporting AR conversational use-cases.

In this contribution:

- we update the mapping of AR conversational services in permanent document to STAR UE

- we propose a call flow for AR two-party calls using immersive 3D media

Table 6.1 TR 26.998

| AR conferencing | 6.6 | 8. 360-degree conference meeting 9. XR Meeting 10. Convention / Poster Session 12. AR avatar multi-party calls 13. Front-facing camera video multi-party calls 19. AR Conferencing |
| --- | --- | --- |

# Proposed change

6.5	AR two-party calls

### 6.5.1	Introduction

AR Conversational services are end-to-end use-cases that include communication between two or more parties. There are different options for mapping to 5G system:

- The MTSI architecture (TS 26.114) supports audio and 2D video conversational services. Extending the MTSI architecture to support AR signaling and immersive media.

- Extending the 5GMS architecture (TS 26.501) to support AR conversational services by combining live uplink and live downlink.

- A new architecture based on browser implementations such as WebRTC.

Table 1 Comparison of different architecture options for supporting AR conversational services

| Component | MTSI/RTP (TS 26.114) | 5GMS/HTTP (26.501) | New architecture (WebRTC) |
| --- | --- | --- | --- |
| Protocol | SIP- and RTP-based. SDP signaling and formats for AR are missing and need to be defined. Encoding and decoding at MTSI client needs to be extended to support immersive media. | TCP- and HTTP-based streaming, using DASH/HLS and MPEG OMAF/CMAF technology. AR & immersive media content and signaling are assumed to work with HTTP-based streaming in the other use-case mappings. | Using WebRTC data channel and/or extending WebRTC audio/video for AR media such as immersive media communications. AR signaling aspects to be studied. |
| Connection establishment | Find and connect is solved through SIP and E.164 addressing in IMS. | Find-and-connect for the conversational, UE-to-UE, case is undefined. | WebRTC implementations offer dedicated APIs for connection establishment in various contexts such as social media platforms. Browser applications are widely available. |
| Performance | Technically possible, latency should in principle not be a problem to achieve, building on the existing QoS and policy framework in 5GC. | Low latency and QoS DASH support in 5GMS to be studied. | WebRTC is designed with low latency in mind but has no defined relation to QoS and policy framework in 5GC and use of that need to be studied. |
| Deployments | Cross-operator interconnect aspects are included. Edge processing functions to be studied. | Cross-operator interconnect aspects are currently ignored. Edge processing functions to be studied (e.g. EMSA). | Cross-operator interconnect aspects are currently not applicable since WebRTC is used OTT today, but will become relevant and need study, especially if used with QoS. Edge processing functions to be studied. |

### 6.5.2	Architecture mapping to AR conferencing use-case (STAR UE)

To describe the functional architecture for AR conversational use-cases (e.g. clause A5 in TR 26.998) and identify the content delivery protocols and performance indicators an end-to-end architecture is proposed. The end-to-end workflow for AR conferencing (one direction) is shown in Figure 1A. Camera is capturing the participant in an AR conferencing scenario. The camera is connected to a UE (e.g. laptop) via a data network (wired/wireless). Live camera feed, sensors and audio signals are provided to a UE/Edge node (or split) which processes, encodes, and transmits immersive media content to the 5G system for distribution. The immersive media processing function can include pre-processing of the captured 3D video, format conversion, and any other processing needed before compression. Immersive media content includes 3D representation, such as in form of meshes or point clouds, of participants in an AR conferencing scenario.  After processing and encoding, the compressed 3D video and audio streams are transmitted over the 5G system. A 5G STAR UE decodes, processes and renders the 3D video and audio stream.

The use-case be extended to bi-directional by adding a 3D camera on the receiver side and AR glasses on the sender side and applying a similar workflow.

Figure 1: Extensions to device architecture for conversational services (STAR UE)

### 6.5.2	Call flow

We consider an immersive AR two party call between Alice and Bob. The end-to-end call flow is described:

[STAR UE Alice - STAR UE Bob]:  Either one of the UEs can initiate an AR immersive call by starting an application on the phone or AR glasses.

[STAR UE Alice - STAR UE Bob]: Both UEs communicate with a signaling server to establish the AR call. During the session establishment, both parties agree on the format (e.g. point clouds, triangular/polygon meshes). The exact session type and configuration depends on the capabilities of STAR UE.

[STAR UE Alice]:  Alice is captured by a depth camera embedded within the STAR UE or an external camera which generates an immersive 3D media stream (audio and video).

[STAR UE Alice]: The immersive 3D media is encoded and transmitted in real-time to Bob over the 5G system. Additional pre-processing can be applied before encoding such as format conversion.

[STAR UE Bob]: The immersive 3D media is received on on Bob’s STAR UE. The immersive 3D media stream is decoded and rendered on AR glasses. Additional postprocessing can be applied before rendering such as format conversion, customization to match the stream to rendered environment e.g. filling holes.

[STAR UE Bob]: Bob is captured by a depth camera generating an immersive 3D media which is encoded and transmitted in real-time to Alice’s AR glasses.

[STAR UE Alice]: The immersive 3D media which is received, decoded and rendered on Alice’s AR glasses.

[STAR UE Alice - STAR UE Bob]: Both UEs terminate the service at the end of the call.

# Conclusion

It is proposed to include Section 2 in the TR.