| 3GPP TR 26.803 V1.2.1 (2021-05) |  |
| --- | --- |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G Media Streaming Extensions for Edge Processing (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview of Relevant Architectures	8

4.1	General	8

4.2	SA6 Edge Architecture	8

4.3	SA2 Edge Support	12

4.3.1	Overview	12

4.3.2	Process of Discovering EAS	14

4.3.2.1	General	14

4.3.2.2	DNS-based solutions for Multiple PDU Sessions	14

4.3.2.3	DNS-based solutions for Distributed Anchors	15

4.3.2.4	DNS-based solutions for Session Breakout	15

4.3.3.5	Summary of DNS based EAS Discovery	15

4.3.3.6	Discussion	16

4.3.4	Summary of EAS relocation solutions	16

4.4	 SA5 Edge Management	17

4.5	5GMS Architecture	18

5	Use Cases for Edge Media Processing	18

5.1	General	18

5.2	Downlink Streaming Use Cases	18

5.2.1	Caching Downlink Streaming Content	18

5.2.2	Split Rendering	20

5.2.3	User-generated live streaming	22

5.2.4	Augmented Video Streaming	26

5.2.5	Generalized Split and Cloud Rendering and Processing	28

5.2.6	Photo-realistic AR Rendering in Network	29

5.2.7	Media Services in the Edge	31

5.2.8	Partial delivery of 3D content (point cloud, mesh) for AR/MR device	32

5.3	Uplink Streaming Use Cases	33

5.3.1	Multi-Camera Uplink Stream Processing	33

5.3.2	Cloud/Split Rendering of Immersive Live Events	36

5.3.3	Pandemic Stadium	37

6	Potential 5GMS Architecture Extensions	39

6.1	General	39

6.2	EMSA Architecture	39

6.3	Generic Call Flows for Media Session Establishment	40

6.3.1	General	40

6.3.2	Client-driven edge discovery	41

6.3.3	AP-driven management of 5GMS edge processing	44

6.4	Identified Gaps in Architecture and Procedures	45

6.4.1	General	45

6.4.2	Justification of Identified Gaps	45

6.4.2.1	Gaps in client-driven edge discovery	45

6.4.2.2	Gaps in AP-driven management of 5GMS edge processing	46

7	Conclusions and Recommendations	46

Annex A: Deployment Scenarios	47

A.1	Deployment Scenario Option A	47

A.2	Deployment Scenario Option B	48

Annex B: Selected Use Case Mapping	49

B.1	Split Rendering	49

B.2	Edge Caching	51

Annex <X> (informative): Change history	53

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

In this Technical Report, we study the architectural enablers and extensions to the 5G Media Streaming Architecture for edge media processing. It covers aspects such as discovery, configuration, execution, and management of media processing in 5G to address the needs identified for example by the XR5G and FLUS items.

# 1	Scope

The present document is a study of use cases for multimedia processing in the edge and the potential 5G media streaming architecture extensions to enable them.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[4]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)".

[5]		3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[7]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

AF	Application Function

ASP	Application Service Provider

DNAI		Data Network Access Identifier

DNS	Domain Name Service

ECS	Edge Configuration Server

EEC	Edge Enabler Client

EES	Edge Enabler Server

PCF		Policy Control Function

PDU		Protocol Data Unit

PSA		PDU Session Anchor

QoS		Quality of Service

LADN		Local Area Data Network

LBO		Local Break Out

NEF		Network Exposure Function

SSC		Session and Service Continuity

SMF		Session Management Function

UPF		User Plane Function

URSP		UE Route Selection Policy

# 4	Overview of Relevant Architectures

## 4.1	General

In this clause, we provide an overview of ongoing SA activities on edge computing architectures.

## 4.2	SA6 Edge Architecture

SA6 has taken significant steps towards the definition of normative edge computing architecture for 5GC in TS 23.558  [3]. Starting from common scenarios, described in the Annex, a set of requirements is defined, and the following architecture is proposed:

Figure 4.2-1 SA6 Edge data network architecture

The architecture defines the key nodes and functions as well as the interfaces between them.

The identified functions with a brief description are given here:

- Edge Enabler Server (EES): provides supporting functions needed for Edge Application Servers and Edge Enabler Client.

- Edge Enabler Client (EEC): provides supporting functions needed for Application Client(s).

- Edge Configuration Server (ECS): provides supporting functions needed for the Edge Enabler Client to connect with an Edge Enabler Server.

- Edge Application Server (EAS): the application server resident in the Edge Data Network, performing the server functions. The Application Client connects to the Edge Application Server in order to avail the services of the application with the benefits of Edge Computing.

- Application Client (AC): application resident in the UE performing the client function. Details of the Application Client are out of scope of this specification.

A typical sequence of steps to use edge computing services is as follows:

1.	Service Provisioning

-	The EEC is provisioned with a list of EES instances, e.g. from the ECS.

2.	Registration

-	EES instances register with the ECS to publish their edge configuration capabilities.

-	The EEC registers with a selected EES for further EAS discovery and Edge Computing Service usage.

-	EAS instances register with EES instances to publish their edge capabilities.

3.	EAS discovery

-	The EEC queries the EES to discover specific EASs. Different types of filtering information contained in the EAS discovery filters can be used during this discovery phase in the EAS discovery request.

-	The EES identifies the appropriate EAS instance(s) according to the UE-specific service information and the UE location.

-	Via the EAS discovery response, the EEC receives the discovered EAS instance(s) with the corresponding EAS profile which may include additional information regarding matched capabilities, e.g. service permission levels, service area, KPIs.

-	The detailed information for key messages and elements of the EAS discovery procedures is shown as below in Tables 1, 2, 3, 4 and 5.

Table 4.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table Y. |

Table 4.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1 of TS 23.558 [3]. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1 of TS 23.558 [3]. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > EAS status | O | Required status of the EAS (e.g. enabled, disabled, etc.) |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Only one of the information elements shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

Table 4.2-3: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list | O | List of discovered EAS(s). Each element includes the information described below. |
| >> EAS profile | M | Profile of the EAS. Each element is described in Table XX. |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |

Table 4.2-4: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The topological service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. See possible formats in Table 8.2.7-1 of TS 23.558 [3]. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table YY |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE may also indicate whether the EAS supports ACT. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Table 4.2-5: Edge Application Server Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the Application Client supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the Application Client's service requests. |
| Availability | O | Advertised percentage of time the server is available for the Application Client's use. |
| Available Compute | O | The maximum compute resource available for the Application Client. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the Application Client. |
| Available Memory | O | The maximum memory resource available for the Application Client. |
| Available Storage | O | The maximum storage resource available for the Application Client. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the Application Client's use. |

4.	EAS relocation:

-	The detection entity role can be potentially performed by the Application Client (AC), the Edge Enabler Client (EEC), an Edge Enabler Server (EES) and/or an Edge Application Server (EAS).

-	A decision-making entity determines that application context relocation is required and instructs the execution entity to perform application context transfer.

-	An execution entity performs application context relocation as and when instructed by the decision-making entity.

-	After successful application context relocation, the EES is informed of the completion by the EAS and the EEC is informed of the completion by the EES.

## 4.3	SA2 Edge Support

### 4.3.1	Overview

Edge Computing has been identified as a key feature of 5G early on and has been specified as part of the 5G System architecture in clause 5.13 of [2]. It describes the selection of a close UPF for non-roaming or LBO-connected UEs. The specification lists different options to enable access to edge computing:

- User plane (re)selection: the 5G Core Network (re)selects UPF to route the user traffic to the local Data Network.

- Local Routing and Traffic Steering: the 5G Core Network selects the traffic to be routed to the applications in the local Data Network.

- Session and service continuity to enable UE and application mobility.

- An Application Function may influence UPF (re)selection and traffic routing via PCF or NEF.

- Network capability exposure: 5G Core Network and Application Function to provide information to each other via NEF or directly.

- QoS and Charging: PCF provides rules for QoS Control and Charging for the traffic routed to the local Data Network.

- Support of Local Area Data Network: 5G Core Network provides support to connect to the LADN in a certain area where the applications are deployed.

In addition, SA2 has started a new release 17 study item on enhancement of support for edge computing in 5GC [4] to address identified gaps to enable edge computing. The findings will be documented in TR 23.748 [4].

So far, the identified key issues are:

1.	Discovery of Edge Application Server:

The UE needs to find the optimal Edge Application Server (EAS) so that traffic is locally routed to the respective EAS. Furthermore, replacing the old EAS with new EAS when the old EAS is no more optimal is also an important aspect to address.  This key issue deals with such EAS discovery.

2.	Edge relocation:

As the UE moves across the 5G system, the UE location may change and require the network and the EAS to deal with the change. Furthermore, Edge Application Server (EAS) may become congested or may even become unavailable due to outage, which also need to be dealt by changing the EAS. This key issue deals with addressing such EAS relocations.

3.	Network Information Provisioning to Locate Applications with Low Latency:

There might be a need to deliver certain timely QoS information to the Edge Application Server (EAS) that is deployed locally. The usual way to do this is via the NEF, however it will not be possible to do this with low latency because the NEF is usually located in the central location to avoid frequent switching between NEFs. This key issue deals with being able to provision such QoS information to the local applications that run in the EAS with low latency.

4.	(Consecutive traffic steering in different N6-LAN)

Traffic steering might be needed consecutively first to Local Application Server and then to Central Application Server for further processing. Also for downlink, there might be a traffic first that moves to Central Application Server and then brought to the Local Application Server (to get it closer to the UE) and then finally to the UE.

5.	Activating the traffic routing towards local data network per AF request:

In order to activate the traffic routing towards Local (access to) Data Network, the SMF should be configured with the required DNAI. For ETSUN case, either SMF or I-SMF should be configured with the requested DNAI. For some of edge computing use case scenarios, the SMF or I-SMF of the PDU session may not be configured with the requested DNAI. This key issue deals with the mechanism to activate the traffic routing towards the Local Data Network in such cases.

Clause 6 of TR 23.748 [4] documents a set of solutions for the previously identified key issues.

The following architecture figures show 5GS and Edge Application Servers hosted in Edge Hosting Environment.

NOTE:	These figures show the relationship between the EAS and 5GC defined in TS 23.501 [2]. The application layer architecture for enabling edge computing is out of the scope of the SA2 study.

Figure 4.3.1-1: SA2 Architecture for Accessing Edge Application Server with UL CL/BP

Figure 4.3.1-2: SA2 Architecture for Accessing Edge Application Server without UL CL/BP

NOTE:	These figures show the relationship between the EAS and 5GC defined in TS 23.501 [2]. The application layer architecture for enabling edge computing is out of the scope of the SA2 study.

5GC supports at least the following three connectivity models to enable Edge Computing:

-	Distributed Anchor Point: the PDU Session anchor is moved far out in the network, to the local sites. It is the same for all the user PDU session traffic. Re-anchoring (SSC#2 and SSC#3) is used to optimize traffic routing for all applications when moving long distances.

-	Session Breakout: The PDU session has a PDU Session anchor in a central site and a PDU Session anchor in the local site. Only one of them provides the IP anchor point. The Edge Computing application traffic is selectively diverted to the local PDU Session anchor using UL Classifier or multihoming BP technology. Re-anchoring of the local PDU Session anchor is used to optimize traffic routing for locally diverted traffic as the user moves.

-	Multiple PDU sessions: Edge Computing applications use a specific PDU session with the PDU Session anchor in the local site. The rest of applications use a PDU Session with a central PDU Session anchor. The mapping between applications and PDU sessions is steered by the URSP rules. Re-anchoring (SSC#2 and SSC#3) is used to optimize traffic routing for Edge Computing applications as the user moves.

These three connectivity models are illustrated in the figure below:

Figure 4.3.1-3: SA2 5GC Connectivity Models for Edge Computing

### 4.3.2	Process of Discovering EAS

#### 4.3.2.1	General

DNS based solutions have been concluded as the final solution for discovering the Edge Application Server for the above three connectivity models.

#### 4.3.2.2	DNS-based solutions for Multiple PDU Sessions

This solution proposes to enhance the NEF service(s) to allow the AF to influence PCF decisions for URSP rules so that Domain Descriptors are used to steer the DNS and the Application traffic into specific PDU sessions with the PDU Session Anchor UPF that is servicing the EAS.

The following clarifications are therefore required in normative specification(s) on how the Domain Descriptors in URSP are used by the UE in clause 7.1.4 of TS 23.503 [5]:

-	When a UE attempts to resolve an FQDN, the PDU Session through which to send the DNS query message is controlled by URSP rules.

-	The UE should be able to use URSP procedures to set up the PDU session prior to sending a DNS query.-	The IP address of the DNS server for the PDU Session is configured on the UE by the SMF using one of the methods already specified.

The solution is not guaranteed to work when:

a)	The UE doesn't support URSP rules provisioned from the PCF.

b)	The DNS Server configuration at the UE has been overridden by the user.

c)	The Application Client deliberately circumvents the Operating System’s default DNS libraries, for example by using DNS-over-TCP (DoT), DNS-over-HTTPS (DoH) or any other over-the-top mechanism.

NOTE:	This can correspond to devices doing tethering, or to devices deployed for specific corporate purposes. Whether URSP rules work in the OS level is up to implementations. How the cooperation between the OS level and the modem works depends on the further studies.

These limitations and informative guidelines will be captured in a technical specification to cover scenarios where the Operating System, user or applications may override the operator-provided URSP or DNS settings. The guidelines should assume no restriction on the Operating System, user or application.

#### 4.3.2.3	DNS-based solutions for Distributed Anchors

The decision for anchoring the UE in the distributed anchor point scenario for Edge Computing will be described in a technical specification as:

a)	Using subscription policy information to set proper UE policy (e.g. URSP via usage of dedicated DNN); and/or

b)	Applying proper policies at session (SMF) level to apply PCC rules based on the Nnef_TrafficInfluence service and based on the user subscription.

#### 4.3.2.4	DNS-based solutions for Session Breakout

For Session Breakout connectivity mode, the LDNSR, a new stand-alone 5GC network function is proposed. It allows coordination of EAS Discovery using DNS and 5GC connectivity. The LDNSR facilitates selection of an EAS closer to the edge, and it allows Dynamic ULCL/BP/Local PSA insertion.

1.	The LDNSR is dynamically configured with address records of the EAS instances it is to handle so that it can respond appropriately when a local UE attempts to resolve one of these FQDNs.

2.	The SMF selects the LDNSR serving the PDU session and configures the UE to use the LDNSR as its DNS Server for that PDU Session.

3.	The LDNSR maintains a PDU session context during the lifetime of the PDU session and needs to be made aware of the release of the PDU Session.

4.	The LDNSR is able to inform SMF with the IP address of an EAS resolved by DNS, which that may trigger the SMF to perform local UPF insertion/relocation if needed.

Editor’s Note: The name of “LDNSR” is up to SA2’s final WID outcome.

#### 4.3.3.5	Summary of DNS based EAS Discovery

The DNS-based EAS Discovery solution depends on DNS message parsing. The Core Network can decide the location of the local PDU Session Anchor and can deliver the DNS query message to DNS with UE’s location indicated. Then the DNS can return the IP address of EAS. Finally, the UE connects to the Edge Application Server via the PDU Session.

When the UE moves, the 5GC (SMF) may change the user plane path according to the UE’s new location, i.e. TA granularity. Then the User Plane path change notification may be sent to the AF and AF decides whether to trigger the EAS relocation procedure or not.

NOTE:	Currently, the DNS-based EAS discovery solution is only useful with Operator-provided DNS Setting. If the DNS server is overwritten by a third party, or if the application or application library uses DNS over TCP (DoT) or DNS over HTTPS (DoH), the solution is still under development in SA2. The Operated-provided DNS can also be the recursive DNS.

Editor’s Note: Alternative solutions for the cases where enhanced/controlled DNS resolution won’t work will be up to SA2’s WID conclusion.

#### 4.3.3.6	Discussion

With the concluded solutions in SA2, the application client can directly discover and connect to the EAS before any application layer exchanges between the client and EAS. There will be less impact for SA4 aspects without frequent exchanges at the application layer.

However, without the exchanges between clients and application servers, which capability does this application need? Especially when the capability of EAS need to be determined based on the capability and requirements of clients. The SA2 solutions may need to be enhanced for discovering EAS with specific capabilities to avoid additional re-discovering the appropriate EAS.

### 4.3.4	Summary of EAS relocation solutions

Currently the concluded solutions in SA2 mainly focus on the following aspects:

1.	EAS Rediscovery

  - -	The DNS-based EAS rediscovery procedure is triggered by UE to find the new EAS. The triggers can be a new IP address assigned to the UE or the EAS relocation indicator within the NAS message from SMF. Then UE can establish the new connection to the new EAS.

  - -	The UE is informed via the application layer signaling, like HTTP REDIRECT, which is out of 3GPP SA2 scope.

2.	For less packet loss, buffering the uplink data in the target PSA-UPF is proposed when the EAS relocation begins.

3.	The User Plane latency can be taken into consideration by 5GC during the user plane path change/selection. The AF can give a UP latency request during the EAS relocation and 5GC can follow that requirement during the change/selection of UP path.

4.	The Coordination indication can be provided by the AF to allow connections to the source EAS and target EAS to coexist for a period of time.

5.	EAS IP replacement in the PSA UPF can be used to keep UE unaware of the EAS change. However, this conclusion assumes the TCP/TLS/QUIC context transfer between EAS can be handled by other solutions.

During the EAS relocation, how to transfer the EAS context including application layer context and transport layer context is out the scope of the SA2 study. Hence, the main concluded solutions seem like preparation work, e.g. EAS rediscovery triggers different considerations during the EAS relocation. How to define the context transfer is not defined yet.

Transfer of context between EAS instances is application- and deployment-specific, but a 3GPP-defined API to initiate context transfer would enable the 5GMS AF to interface with different virtual infrastructure management systems.

NOTE:	Certain applications may be sensitive to the speed of context transfer, so it would be useful for a 5GMS AF to be able to obtain an estimate in advance from the 5G System of how long a particular transfer might take.

## 4.4		SA5 Edge Management

SA5 has recently agreed a new study item [4] on the management aspects of edge computing. The focus of the study item will be on the following scenarios:

-	How 3GPP management solutions support the third-party service provider to deploy and manage the Edge Configuration Server.

-	How 3GPP management solutions support Edge Computing Service Provider(s) to deploy and manage Edge Enable Server(s).

-	How 3GPP management solutions support Edge Application Provider(s) to deploy and manage Edge Application Server(s).

As can be seen, SA5 has adopted the SA6 architecture and will study ways of enabling it through providing the necessary management functions.

The scope has been set to the following items:

  - 1. 	The lifecycle management of EAS, EES and ECS needed to support edge computing, by considering the various deployment scenarios.

  - 2.	Deployment and provisioning of 5GC network functions needed to support the edge computing.

  - 3.	Provisioning of EES, and ECS needed to support edge computing.

  - 4.	Performance assurance of EES, ECS and relevant 5GC functions needed to support edge computing.

  - 5.	Fault supervision of EES, ECS and relevant 5GC functions needed to support edge computing.

  - 6.	Mechanism(s) to enable and support EAS deployment on a particular edge data network.

  - 7.	Mechanism(s) to enable and support

    - -	ECSP to deploy and manage EES and ECS.

    - -	ASP to deploy and manage EAS.

  - 8.	Studying the need of providing management provisions to create and manage communication service(s) at a particular edge data network.

## 4.5	5GMS Architecture

SA4 has developed a 5G Media Streaming (5GMS) architecture that is designed to support media streaming sessions over 5G. The overall 5GMS architecture is depicted in the following figure:

Figure 4.5-1: 5G Media Streaming within the 5G System

The 5GMS architecture consists of three main functions: the 5GMS AF, the 5GMS AS and the 5GMS Client. These functions leverage the 5G System functionality to support and assist media streaming sessions. The 5GMS architecture supports both downlink and uplink streaming. It is also designed to support a wide range of streaming services, from video streaming to VR/MR/XR.

5GMS specifies a set of interfaces and APIs to allow the Application Provider and the UE to configure and request support for streaming sessions.

# 5	Use Cases for Edge Media Processing

## 5.1	General

Editor’s Note: only relevant use cases should be documented.

## 5.2	Downlink Streaming Use Cases

### 5.2.1	Caching Downlink Streaming Content

Description

A Mobile Network Operator that deploys a downlink streaming service or supports the delivery of media content from a third-party service wants to offer that content in the highest possible quality to all of its users. The MNO also notices that video streaming already accounts for a large part of the traffic on the backhaul network. For these reasons, the MNO wants to offload (part of the) content hosting from the CDN to caches near or within its network. Users of the service may access the content from the edge, allowing them to select higher quality renditions of the content (e.g., DASH representations) and play it back without interruptions. The MNO may improve the hit ratios of the cache by employing intelligent caching. Furthermore, to ensure that clients access the content from the optimal edge, the network operator may want to direct clients to this edge.

Categorization

Type: CDN Delivery: Download, Live Streaming, On Demand Streaming Device: Phone, tablet, HMD, TV

Preconditions

End user devices should be able to stream, decode, and display the video streams. Modern smartphones already have these capabilities.

Requirements in terms of Capabilities and QoS/QoE Considerations

The capabilities of edge nodes are similar to regular CDN nodes distributing video content, although at smaller scale. This means that edge nodes should have storage and HTTP serving capabilities, and UEs should have high-bandwidth connectivity to edge nodes. Higher video quality, less playback interruptions, and shorter loading times improve the QoE.

Feasibility and Industry Practices

CDNs are a well-known and well-established technology. Mobile CDN is receiving the attention of various third-party CDN operators due to the increase of mobile users and the increased prevalence of applications and services that target mobile users. A recent report "Mobile CDN Market" (https://www.transparencymarketresearch.com/mobile-cdn-market.html) lists different CDN operators operating a Mobile CDN, including: Cloudflare, Fastly, Akamai Technologies, Amazon Web Services and Ericsson. Cloudflare explains its reasons for supporting Mobile CDN: https://www.cloudflare.com/performance/accelerate-mobile-experiences/. This use case allows an MNO to offer services to third-party CDN providers or to improve the performance of its own Operator CDN (if the MNO offers CDN service itself) by enabling distributed CDN cache instances to run as Edge Application Servers (EAS). This use case relies on the network being able to select an edge cache that is closest to the UE client, as well as performing cache re-selection during mobility of the UE client. The architecture to support this is being worked on by SA2 in the Release 17 study item on enhancement of support for edge computing in 5GC under key issue 1 (‘Discovery of Edge Application Server’) and key issue 2 (‘Edge relocation’).

Nominal Cost Analysis

Using edge computing for video content caching is the next step in distributing video delivery. It will allow MNOs and streaming services to further scale up and serve more users, while reducing load on the backhaul network. As in a regular CDN node, a node at the edge can be used by many users at the same time and servers scale horizontally. MNOs can use existing facilities at PoPs or points further in the network with serving capabilities.

Benefits and Impact

The major benefit is expected for MNOs and service providers, who are able to serve more users with high quality video while significantly reducing the load on the backhaul network, thus improving the efficiency of the network infrastructure. End-users are expected to benefit as it will increase the access to content in a high video quality, also enabling demanding streaming applications including VR, and delivering those applications with shorter loading times and with fewer interruptions.

Potential Technical Requirements

1. It should be possible for edge caches to be operated either by the MNO or by a third-party service such as a 5GMSd Application Provider. 2. It should be possible for the network to steer clients to a certain edge or CDN. 3. It should be possible for (third-party) services to specify caching directives. 4. It should be possible for DASH clients to send hints (e.g., about anticipated upcoming requests) to the network enabling intelligent caching on the edge.

Potential Standardization Status and Needs

TBD

### 5.2.2	Split Rendering

Description

The system design for split rendering follows the discussion and requirements from TR26.928, clause 6.2.5. The architecture us shown in Figure 1. Figure 7. Split Rendering with Asynchronous Time Warping (ATW) Correction Raster-based split rendering refers to the case where the XR Server runs an XR engine to generate the XR Scene based on information coming from an XR device. The XR Server rasterizes the XR viewport and does XR pre-rendering. According to Figure Figure 1, the viewport is pre-dominantly rendered in the XR server, but the device is able to do latest pose correction, for example by asynchronuous time-warping (see clause 4.1) or other XR pose correction to address changes in the pose. - XR graphics workload is split into rendering workload on a powerful XR server (in the cloud or the edge) and pose correction (such as ATW) on the XR device - Low motion-to-photon latency is preserved via on device Asynchronous Time Warping (ATW) or other pose correction methods. The following call flow highlights the key steps: 1) An XR Device connects to the network and joins XR application a) Sends static device information and capabilities (supported decoders, viewport) 2) Based on this information, the XR server sets up encoders and formats 3) Loop a) XR Device collects XR pose (or a predicted XR pose) b) XR Pose is sent to XR Server c) The XR Server uses the pose to pre-render the XR viewport d) XR Viewport is encoded with 2D media encoders e) The compressed media is sent to XR device along with XR pose that it was rendered for f) The XR device decompresses video g) The XR device uses the XR pose provided with the video frame and the actual XR pose for an improved prediction using and to correct the local pose, e.g. using Asynchronus Time Warp. According to TR 26.928 [6], clause 4.2.2, the relevant processing and delay components are summarized as follows: - User interaction delay is defined as the time duration between the moment at which a user action is initiated and the time such an action is taken into account by the content creation engine. In the context of gaming, this is the time between the moment the user interacts with the game and the moment at which the game engine processes such a player response. - Age of content is defined as the time duration between the moment a content is created and the time it is presented to the user. In the context of gaming, this is the time between the creation of a video frame by the game engine and the time at which the frame is finally presented to the player. The round-trip interaction delay is therefore the sum of the Age of Content and the User Interaction Delay. If part of the rendering is done on an XR server and the service produces a frame buffer as rendering result of the state of the content, then for raster-based split rendering (as defined in clause 6.2.5) in cloud gaming applications, the following processes contribute to such a delay: - User Interaction Delay (Pose and other interactions) - - capture of user interaction in game client, - - delivery of user interaction to the game engine, i.e. to the server (aka network delay), - - processing of user interaction by the game engine/server, - Age of Content - - creation of one or several video buffers (e.g. one for each eye) by the game engine/server, - - encoding of the video buffers into a video stream frame, - - delivery of the video frame to the game client (a.k.a. network delay), - - decoding of the video frame by the game client, - - presentation of the video frame to the user (a.k.a. framerate delay). As Asynchronous Time Warp is applied the motion-to-photon latency requirements (of at most 20 ms) are met by XR device internal processing. What determines the network requirements for split rendering is time of pose-to-render-to-photon and the round-trip interaction delay. According to clause TR 26.928, clause 4.5, the permitted downlink latency is typically 50-60ms.

Categorization

Type: XR, Cloud Computing, GPU Delivery: Interactive, Split, Gaming Device: Phone, HMD, Glasses

Preconditions

On the device, a gaming application may be installed On the network, an XR Server is installed, that runs a gaming application as well GPU based rendering and an encoding.

Requirements in terms of Capabilities and QoS/QoE Considerations

Feasibility and Industry Practices

Steam Boundless XR: https://zerolight.com/de/news/press-releases/worlds-first-boundless-xr-over-5g-retail-experience

Cost Analysis

Potential Standardization Status and Needs

### 5.2.3	User-generated live streaming

Description

A social influencer starts a live captured media session similar to Facebook Live and publishes the content through 5G Media Uplink Streaming. The content is then distributed live to several or many viewers through 5G Media Downlink Streaming. The application provides several usage scenarios with various configuration options which may change during the live session: 1. The social influencer may: - a) be static, occasionally moving or highly mobile, for example in a vehicle, on skis or on a bicycle, - b) produce different quality of content, depending on lighting conditions, speed, as well as based on the quality of the camera and the available uplink bandwidth, - c) produce highly-valuable content that requires extra content protection, - d) want to capitalize on the stream by allowing ad-insertion in the content (targeted pre-roll and/or mid-roll). 2. The viewers may be quite diverse and changing because: - a) they may be dynamically joining or leaving the live stream, - b) their number might be just a few or they may quite many, in range of tens of thousands or more, for example for a popular influencer, - c) they may be geographically spread with different densities in various areas, and their densities may change during the session, - d) they may consume the service on different devices, for example on 4K TV sets, mobile phones, in-car receivers or tablets, with different operating systems, DRM capabilities as well as different codec hardware capabilities, - e) some of them may be mobile, i.e on a car or public transport, - they may react (smilies, comments, likes, audio dubs, images, avatars, and animations) to the content or previous reactions by the viewers who watched the content earlier. 3. The service requirements may be quite different. The content may need to be: - a) available for live and/or on-demand consumption, - b) only available for consumption after the end of the live uplink session, i.e. it is uploaded entirely before being made available to followers, - c) available with a required target latency with ranges in between capture and display of as low as 1 second up to several tens of seconds, - d) dubbed into the same or different language, - e) provided with automatic extraction and addition of captions/subtitles from the audio, - f) post-processed to improve the audio and/or visual quality, - g) processed by adding overlays and content tags and other augmented material, - h) indexed, including the addition of thumbnail navigation in real time, - i) provided to regional proxies with specific metadata such as black-out information, ad insertion opportunities, language settings or other service metadata, - j) profanity checked and appropriately altered before being distributed, - k) available for viewing for some time period (from a few minutes to forever) after the end of the live session. In a simple reference scenario of 5G Media Uplink and Downlink Streaming, the following aspects are supported: 1. An application manages the service. The server application may be run by external application providers, by MNO, or by a joint collaboration of application provider and MNO. 2. Uplink streaming is provided through 5G media uplink streaming to the application. 3. The application may perform one or more of the following processes: - a) It decodes the received content - b) It applied the various processes the content such as: - i) Upscaling - ii) Light correction - iii) Stabilization - iv) Dubbing - v) Captioning - vi) Overlaying and tagging - vii) Indexing - viii) Navigation improvements. - c) It encodes the uploaded content to 5GMS downlink streaming formats. - d) It packages the content and adds appropriate ad metadata. - e) It applies content protection and DRM. 4. As soon as the content becomes available, the server application uses downlink streaming for distribution. - a) It provisions a 5GMS downlink streaming service. - b) It ingests the content into a 5GMS streaming service. - c) It gets feedback from consumption reporting on what content is consumed and may change the encoded streaming formats. The following aspects may be critical for the service: 1. Can the service be provided throughout the session without any interruption considering the dynamic aspect of the service, for example when a highly mobile edge media streaming client makes a transition between two edge processing environments? 2. Can the desired end-to-end latency be met with the reference scenario and if not, what are possible ways to realize this? 3. Is the content generation flexible and fast enough to address different user population, yet highly utilized, i.e. the variations of the content is consumed by one or more viewers and is shared as much as possible by many viewers? 4. Is there a benefit to push certain processing closer to the influencer, certain viewers, or in-between (in terms of bandwidth, latency, and processing requirements)? NOTE: The meaning of flexible content generation needs to be explained.

Categorization

Type: 2D Delivery: 5GMS uplink streaming, 5GMS downlink streaming, live media streaming Device: Phone, HMD, Glasses

Preconditions

On the sending UE, 5GMS Uplink streaming is provided. On every receiving UE, a 5GMS downlink streaming client is provided. An Application Service Provider supports network interfaces for 5GMS Uplink Streaming (M1u and M2u) as well as 5GMS Downlink Streaming (M1d and M2d). Extended functionalities in uplink streaming and downlink streaming may be supported such that the Application Service Provider can delegate certain tasks to the 5G Media Streaming System.

Requirements in terms of Capabilities and QoS/QoE Considerations

Potential new required capabilities of 5G Media Streaming System: 1. Decoding of content received through uplink streaming. 2. Processing of the content with functionalities such as: - a) Upscaling - b) Light correction - c) Stabilization - d) Dubbing - e) captioning - etc. 3. Transcoding of the uploaded and processed content to 5GMS downlink streaming formats. 4. Packaging/Encrypting the content and adding appropriate ad metadata. Relevant KPIs: 1. Subjective/objective quality of the encoded streams in terms of bitrates/quality. 2. Latency for the end-to-end service. 3. Latency of each processing step. 4. Seamless integration of ads with the main content. 5. Scalability and handling of different number of devices. 6. Quality of the delivered service in terms of streaming metrics. 7. Computational efficiency in terms of: - a) Encoding and Decoding workflow - b) Encryption and D - c) Manifest generation - d) Target-based advertising 8. Seamless session and service continuity of media streaming when the user’s high level of mobility causes a transition to a new edge processing environment.

Feasibility and Industry Practices

Today, the above processing may be achieved by running cloud processing on the application server for all the functionalities mentioned above. Alternatively, third-party cloud-based transcoding services exist. The number of streams, support of multiple codecs, the amount of delay, the number of representations in the encoding ladder varies depending on the service. Codec support evolves with the availability of new codecs. Services like Facebook Live encode the content on demand and on the fly. Services like YouTube Live provide channels for professional content providers. They can insert advertisement clips, but the ad insertion occurs using two video elements. Services like TikTok allow the user to add a lip-sync to a song, to add his/her reactions on a small window over the video, and to make short movies using filter and speed change of the video. The user also can capture a video next to a pre-recorded video.

Nominal Cost Analysis

The cost of service increases linearly with the number of ingest streams. The cost of service increases less than linearly with the number of download streaming clients as the encoding and caching requirement will be common with the large number of viewers. Cost analysis needs to take into account if there a benefit or necessity to push certain processing closer to the users/edge (in terms of bandwidth, latency and processing requirements)?

Benefits and Impact

Potential Benefits and Impacts when running services within 5GMS network: Potential Benefits and Impacts when running services on the edge:

Potential Technical Requirements

Potential Requirements: 1. Establish workflows as defined in the use case with the addition of service parameters. 2. Distribute the tasks across 5G System and 5GMS components. 3. Seamless transition of application/service media streaming flows between different edge media processing environments.

Potential Standardization Status and Needs

### 5.2.4	Augmented Video Streaming

Description

Augmented video streaming will help to understand the video and enhance the interest of the video. In live streaming scenes, it can analyze the video content with AI technology, refine the video information, understand the video content and enhance the video quality, such as improving the resolution and color. It may also superimpose AR effect to tag the identified video information interestingly, such as the AR effect of the pass path in a football game or the AR effect on human face or background during streaming, etc. Processing at the edge can reduce the time required for video enhancement processing and network transmission. 1. Augmented video streaming with AI. Due to limited shooting conditions or the need for new video features, augmented video streaming with AI can provide a service that edge nodes is used to enhance the captured video to improve the user experience. a) An encoded video streaming is compressed and uploaded to an edge node, and the video resolution, color gamut and other parameters should be specified in configuration. b) The edge node server decodes the video streaming, then with the installed AI capabilities, sets up the enhancement functions, such as resolution enhancement, color enhancement, signal-to-noise ratio enhancement, video feature recognition, subtitle, etc., for video editing. c) Augmented Video is re-encoded and distributed to users. 2. Augmented video streaming with AR. It can provide AR experience to its users. a) An encoded video streaming is compressed and uploaded to an edge node. b) The edge node server decodes the video streaming, identifies and understands the video features with AI capability, and loads the corresponding AR special effects on the understanding results. The AR special effects are overlapped with the video streaming. c) The edge node distributes it to users. 3. Augmented video streaming with interaction. AI augmented content may consider the user’s preferences. AR augmented content will output according to the user's FOV. Therefore, the workflow of the edge node can involve interactions. According to the user's preference, the edge node server selects the corresponding augmented video streaming to distribute, or processes the video according to the user's FOV. When there is a high requirement for QoE of enhanced video, the terminal device cannot meet this requirement and cloud or edge servers are needed for processing. And in the case of local video streaming transmission, the video streaming can be processed directly at the edge. Compared with uploading to the cloud-based processing, it can reduce roundtrip time delay (RTT) of network and avoid the waste of resources on the core network. For AI video enhancement, users can choose the function of the video enhancement. If the AI video enhancement processing functions are deployed on the cloud, downlink transmission needs to involve multi-video streams with different enhancement effects for users to choose from. The pressure of the downlink will be multiplied. Therefore, it is better for edge nodes to support different video enhancement processing. For AR effects enhancement, users need to use AR devices to watch enhanced AR video and have higher requirement on delay, like MTP (Motion To Photons) latency requirement. It is necessary for split rendering of edge processing to ensure this.

Categorization

Type: XR, AI, Cloud Computing Delivery: Interaction Device: Phone, HMD, TV

Preconditions

On the device side, a video application or a browser supporting XR player with interaction is needed. On the network side, a video server is installed for video data structuring process and its key techniques like deep learning, big data technology and cloud storage. Edge nodes need to store some AR effects in advance.

Requirements in terms of Capabilities and QoS/QoE Considerations

Requirements: - 1. Real-time adaptation of encoding and transcoding - 2. Overlay graphics. - 3. CDN and content caching - 4. 6DOF interaction KPIs: - 5. MTP - 6. Operation response time - 7. The number of users

Feasibility and Industry Practices

Intel edge computing solutions At the 2018 China Mobile Global Partner Conference, China Mobile and Intel jointly released a set of video processing unit and intelligent video edge computing solution. https://www.intel.sg/content/www/xa/en/cloud-computing/visual-cloud.html - Media Analytics - AR solutions

Cost Analysis

Potential Standardization Status and Needs

### 5.2.5	Generalized Split and Cloud Rendering and Processing

| Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| See several use cases in TR 26.928 should be added to the already agreed split rendering use case by reference to TR 26.928 [6]. 3; Streaming of Immersive 6DoF; VR; 3DoF+, 6DoF; Streaming Interactive Split; HMD with a controller. 4; Emotional Streaming; 2D, AR and VR; 2D, 3DoF+, 6DoF; Streaming Interactive, Split; Phone and HMD. 5; Untethered Immersive Online Gaming; VR; 6DoF; Streaming, Interactive, Split; HMD with a Gaming controller. 6; Immersive Game Spectator Mode; VR; 6DoF; Streaming, Split; 2D screen or HMD with a controller. 21; Immersive 6DoF Streaming with Social Interaction; VR and Social VR; 3DoF+, 6DoF; Streaming Interactive Conversational Split; XR5G-V3 XR5G-V4. 22; 5G Online Gaming Party; VR; 6DoF; Streaming, Interactive, Split, D2D; XR5G-V3 XR5G-V4. 23; Spatial Shared Data; AR; 6DoF; Streaming Interactive Conversational Split; XR5G-AX. |
| Categorization |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Preconditions |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Requirements in terms of Capabilities and QoS/QoE Considerations |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Feasibility and Industry Practices |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Nominal Cost Analysis |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Benefits and Impact |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Potential Technical Requirements |  |  |  |  |  |  |
| See several use cases in TR 26.928 [6]. |  |  |  |  |  |  |
| Potential Standardization Status and Needs |  |  |  |  |  |  |
| Edge/Cloud processing and rendering is a promising technology to support online gaming in power- and resource constrained devices. Relevant aspects for generalized cloud/split rendering include: 1. A generalized XR cloud and split rendering application framework based on a scene description. 2. Support for 3D formats in split and cloud rendering approaches. 3. Formats and protocols for XR Pose information delivery and possibly other metadata in the uplink at sufficiently high frequency. 4. Content Delivery protocols that support generalized split/cloud rendering. 5. Distributions of processing resources across different resources in the 5G system network, in the application provider domain (cloud) and the XR device. 6. Supporting the establishment of Processing Workflows across distributed resources and managing those. 7. 5QIs and other 5GS/Radio capabilities that support generalized split/cloud rendering by coordination with other groups. 8. Edge computing discovery and capability discovery based on work in SA2 and SA6 (see clause 4.3.6). |  |  |  |  |  |  |

### 5.2.6	Photo-realistic AR Rendering in Network

Description

Bob wants to position a complex 3D object with significant amount of data (materials, roughness, texture, transparency) and place it into a real-world environment using an AR device. For this purpose, he uses a local rendering engine, but he is very unsatisfied as natural lights are not well handled and the object looks unrealistic. A cloud service offers a PBR-based cloud rendering, and he gives it a try. He is really happy with the rendering, he gets sparkling effects and starts to move around the object and wants to put it into different angles and positions. However, moving, rotating, interacting the object makes him annoyed again as the latency of the interaction is only executed after several hundred of milliseconds. However, he gets the offering from his 5G Operator that he can get cloud-based rendering in the edge, promising latencies that makes the 3D objects present and interactive. The 5G Operator permits to ingest popular 3D objects into the network and provides an improved rendering for their consumers. Parts of the objects may be static, others may be timed. In certain cases, even several objects may be rendered at the same time. Example: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/pbr-materials

Categorization

Type: AR, 3D Delivery: Split rendering, edge rendering Device: Phone, AR devices

Preconditions

On the UE, a 5G modem is available, an AR rendering and a depth camera is available. In the network, advanced rendering functionalities are available that are accessible with the 5GS and 5G Edge enablers and also a data storage to host complex 3D objects. An Application Service Provider supports network interfaces for ingesting popular 3D objects into the network.

Requirements in terms of Capabilities and QoS/QoE Considerations

Potential new required capabilities of 5G Media Streaming System: - 1. Fast uplink streaming of natural environments including protocols and formats to included depth and light conditions. - 2. Discovery and establishment of photo-realistic rendering functionalities that can work with uplink camera information. - 3. Fast Downlink streaming to support interaction with objects including new light conditions, etc. - 4. Formats and protocols that allow post-processing to the latest pose Relevant KPIs: - 5. Quality of rendered scene. - 6. Immersiveness and presence of the scene. - 7. Latency of each processing step. For many details, please refer to TR 26.928 [6].

Feasibility and Industry Practices

Nominal Cost Analysis

Benefits and Impact

Potential Benefits and Impacts when running services within 5GMS network: Potential Benefits and Impacts when running services on the edge:

Potential Technical Requirements

Potential Standardization Status and Needs

### 5.2.7	Media Services in the Edge

Description

In the media production and distribution, the use of cloud services get more and more popular. Examples for standardized approaches are for example documented here: - https://www.fims.tv/, - https://tech.ebu.ch/groups/mcma - https://www.smpte.org/microservices-media-and-entertainment-key-benefits-and-challenges These applications include Artificial Intelligence (AI) enabled services, such as: speech-to-text, automatic subtitling, translation, text-to-speech, face recognition and identification, emotion detection, speech-to-text evaluation, FFMPEG media transformation. Is there a benefit to provide some applications on the edge rather than the cloud?

Categorization

Type: 2D Delivery: Cloud and edge processing Device: phone, TV, tablet

Preconditions

Potential new required capabilities of 5G Media Streaming System: - - Ingest - - Distribution Relevant KPIs:

Requirements in terms of Capabilities and QoS/QoE Considerations

Potential new required capabilities of 5G Media Streaming System: Relevant KPIs:

Feasibility and Industry Practices

See above links

Nominal Cost Analysis

Benefits and Impact

Potential Benefits and Impacts when running services within 5GMS network: Potential Benefits and Impacts when running services on the edge:

Potential Technical Requirements

Potential Standardization Status and Needs

### 5.2.8	Partial delivery of 3D content (point cloud, mesh) for AR/MR device

Use Case Name

Partial delivery of 3D content (point cloud and mesh) for AR/MR device

Description

Service provider provides various types of contents to be overlaid over real environment for augmented reality experience. Certain 3D content is huge as the earth, which is called digital twin of past, current and future of the world. The content is live and continuously updated even though it is past. Therefore, delivering one entire content doesn’t make any sense nor is it possible. In this case, a partial delivery of 3D content serves user’s demand. For example, the user wants to travel overseas. The user can select places to stay and place to visit, then a digital twin of the hotel room with a view and sightseeing points will be partially delivered to the user. More practically, only the viewport can be delivered then rendered upon user’s position and view direction. Another example is as follows. Certain 3D content has hundreds of millions of vertices and faces to provide the finest detail possible. For online catalogues of cars, selecting options to try a look and feel of each individual order is always enjoyable. User may hear engine sound and touch dashboard through the playback of such content. Since the level of details for rendering of the vertexes depends on a distance from user’s viewpoint, proper level of density and the only vertexes related with the view can be delivered. The 3D content is encapsulated for the purpose, and MEC does the required post processing to generate partial view then encoding the content.

Categorization

Type: AR, MR, VR, XR, 2D, 3D, Cloud Computing Delivery: Downlink, Interactive, Live streaming, On Demand Streaming, Conversational Device: Phone, HMD, Glasses, Laptop

Preconditions

On the content provider side, delivery or live streaming of 3D content are available. On the UE side, consumption of 3D content and reporting of relevant viewport information are available. On the network side, an MEC server is installed. Performing partial extraction, encapsulation and encoding of the 3D content are available.

Requirements in terms of Capabilities and QoS/QoE Considerations

Potential requirements: 1. Real-time partial extraction and control level of details of 3D content 2. Point cloud and mesh encoding/decoding 3. UE viewport information report Relevant KPIs: 4. The number of voxels and faces per second for partial extraction 5. The number of voxels and faces per second for encoding

Feasibility and Industry Practices

Cost Analysis

Similar with split rendering since position and direction of user’s viewport would require different part of the 3D content.

Potential Standardization Status and Needs

1. How to exchange UE viewport information in 3D space. 2. How to define and signal delivered part out of the total 3D content space. 3. How to measure MEC performance index of the partial extraction, and quality of level of details.

## 5.3	Uplink Streaming Use Cases

### 5.3.1	Multi-Camera Uplink Stream Processing

Description

Some streaming services need multi-camera based scene capture and post-processing. For example, two fisheye lenses or multiple wide-angle lenses used in VR 360 video streaming service can derive 360° uplink video stream by using stitching algorithms implemented in the camera or servers located near the camera. As an extension, the edge node can optimize the multi-camera uplink streaming depending on the viewport of the predominant users or the device capabilities that are consuming the content or the number of users. For instance, it can save uplink bandwidth previously occupied by non-highlighted regions. Another example is free angle live streaming, in which an arc-shaped row of time-synchronized cameras are used to collect the images of an object from different perspectives. Through the composition algorithm of adjacent servers, the uplink live stream can be generated for downlink users to freely choose the viewing angle. Another one is telepresence using AR/MR device. Multiple cameras are deployed to collect several streams, based on which the 3D mesh or 3D point cloud generation algorithms are followed. These algorithms are expected to run on edge using its computing power to meet real-time requirements and keep synchronized. The following key steps are highlighted for both of these use cases: 1. A camera group is set up in correct position and connects to the network, and calibration configurations which describe the relative position relationship are sent to the edge. Each camera’s exposure values are also sent. 2. Edge nodes set up multiple decoders and algorithms to deal with the uplink streams and output the processed result. Mainly there are two reasons supporting the edge node necessity in this use case: 1. VR 360 stitching, free angle video composition, and 3D mesh/point cloud producing algorithms need strong computing CPUs and GPUs rather than those integrated on mobile platforms. Put these servers on edge node instead of capture worksite can reduce space need and transportation costs, more importantly, give mobility to capture device thus clear motion limitations during application. 2. Multiple cameras bring multiple video streams to uplink network. Taking VR 360 stitching for example, an 8K VR camera rig (KANDAO Obsidian) mostly adopted in China Mobile’s service uses 6 cameras to be stitched and each of them are 4K resolution. That means nearly 6 × 25Mbps bandwidth is the minimum requirement. Cloud-based architecture feels difficult to provide stable connection to support this requirement compared to an edge node, and even if it can be guaranteed, longer network latency makes it difficult for streams to be synchronized under given time threshold, which are very important to all following steps.

Categorization

Type: VR 360, free angle viewing, Delivery: Uplink, Live Streaming Device: Phone, HMD

Preconditions

On devices, 5G connection ability should be installed, and a built-in or external time clock may be used to keep multiple streams to be synchronized. On the network side, multiple decoders and at least one encoder are needed to deal with media streams input and output.

Requirements in terms of Capabilities and QoS/QoE Considerations

Requirements: - 1. 5G connection ability on each camera. - 2. Real-time multiple decoding and encoding instrument on edge. - 3. High CPU/GPU computing ability need on edge server to process multiple streams. - 4. Real-time Stitching Algorithms for multiple streams and Image processing. Relevant KPIs: - 5. Steady uplink bandwidth more than N × k ×(single stream bit rate). NOTE: N is the number of streams, and k is the parameter of bitrate to network bandwidth, usually can be set to 1.5 or 2. - 6. Scalability of the solution with the increase of cameras and/or sessions.

Feasibility and Industry Practices

For VR 360 streaming service, China Mobile has tested cloud-based stitching using 5G SA uplink with professional VR camera system. And there are also some mature business solutions existing, for example cloud stitching product provided by StreamboxVR. Here is the link of introduction and demo: https://www.streamboxvr.com/cloud_stitching/ Using a Ricoh Theta S 360 camera, together with Avenir Micro or Drone, pre-stitched streams can be sent to Streambox Cloud, and users can directly get a VR 360 live streaming service. For free angle live streaming service, China Mobile has made a live streaming application used in “The 2nd Youth Games of the People’s Republic of China”, together with partners including ZTE and Qualcomm. It uses 5GS and Edge computing to realize free angle viewing of wrestling matches. The link of this demo is: https://www.iqiyi.com/v_19rtgqhknc.html Users can choose their view angle freely at any time during streaming service. For telepresence service using AR/MR device, there are also many implementations. https://m.youtube.com/watch?v=4oJJN2eH6U0 For stitching streams reflecting highlight regions as follows:

Cost Analysis

This use case mainly provides uplink solutions, generated media streams are served to subsequent operators, thus the cost for downlink stage and user side does not belong to its costs. For the service itself, it needs to pay for multiple 5G modem-integrated cameras, edge computing hardware and software.

Potential Standardization Status and Needs

1. Functionality on the edge node. 2. Data transfer between UE and edge node. NOTE: Other processing may be added.

### 5.3.2	Cloud/Split Rendering of Immersive Live Events

Description

In Use Case 3 in TR 26.928 [6], Annex A.4, streaming and consumption of a recorded highlight of basketball match is considered, being technology agnostic in a sense that rendering may be done in the device only or may be done completely or at least partially in the network. In this use case, the aforementioned scenario is extended to streaming of immersive live events and rendering is done at least partially in the network. The immersive live event may be a basketball match that is produced by highly-professional multi-camera arrays, possibly several 100s of them and the information is provided to a cloud-environment for pre-processing that data. The immersive 6DoF live media content is offered and shared with a remote audience for streaming access can place the remote audiences within the context of a scene and give them immersive experiences: 1. Controlling the orientation of the scene and view direction, i.e. a 6DoF pose. For example, when a remote participant with an XR display (e.g., HMD, AR glass) rotates his head, the XR display can extract the 6DoF pose and provides it to a remote rendering. 2. The content is at least partially pre-rendered in the network according to the latest pose provided from the user in order to minimize the traffic being sent on the downlink as well as to adjust the rendering to the device capabilities. In order to control the immersive latency requirements properly, rendering in the network needs to be timely close to the latest user pose – justifying the usage of an edge node. 3. The edge nodes process the latest 6DoF pose, and generate a pre-rendered version of the live scene for the user’s predicted pose. This pre-rendered viewport is encoded, sent and decoded at the remote device. The XR display can finally render the scene that he is currently watching based on the received viewport, possibly taking into account local pose corrections to achieve a full immersive experience. 4. In addition, the content may further be augmented with overlays, graphics or other data. The data may either be provided for all users or may be user-targeted. For example, the director overlays some value-added information (e.g., the real-time game scores, the sponsor's advertisements) on top of the scene and then distributes the rendered scene to the remote audiences. Especially, if the director can dynamically plant the wonderful slow slots or 3D animated models in a certain region of the scene (e.g., the lower right), a remote viewer can use his finger to zoom or rotate them to look.

Categorization

Type: VR, AR, Edge Computing, GPU Delivery: Downlink, Live Streaming, Split Rendering Device: HMD, AR glass, Smartphone

Preconditions

On the device: - Application is installed that permits to consume the scene. - The application uses existing HW capabilities on the device, including A/V decoders, rendering functionalities as well as sensors. Inside-out Tracking is available. On the network: - Cloud computation capabilities are available to produce the content - Cloud/edge computation capabilities are available to properly pre-render and encode the content

Requirements in terms of Capabilities and QoS/QoE Considerations

QoS requirements: - 1. Bit rates and latencies in the uplink are sufficient to provide an up-to-date pose to the cloud computation. - 2. Bit rates and Latencies that are sufficient to render the viewport within the immersive limits. - 3. Sufficient bandwidth to deliver multiple media streams, QoE requirements: - 1. Fast reaction to manual controller information. - 2. Reaction to head and limited body movement within immersive limits. - 3. Seamless experiences when moving across positions. - 4. Providing sufficient AV experience to enable presence. https://xinreality.com/wiki/Presence

Feasibility and Industry Practices

Cost Analysis

Potential Standardization Status and Needs

### 5.3.3	Pandemic Stadium

Description

In an extension to the preceding Use Case “Cloud/Split Rendering of Immersive Live Events”, the stadium is empty because of a pandemic, but the remote users want to interact with the live event, and even more, the teams and competitors in the stadium get aggregated feedback based on the reactions from the remote users in real time. Due to XR technologies, a remote audience can interact with both the main characters of live events and the other remote audiences to get more immersive experiences as followed: 1. Interacting with the other remote audiences who are geographically distributed. For example, when a remote audience with his friends who are in the virtual environment are watching the same concert, they are able to have voice conversation with each other. They can even cheer the main singer to the echo which can be fed back into the stadium. A network processing server can aggregate the voices from the remote audiences and reflect them to the stadium. 2. Interacting with the main characters of live events (e.g., the signer in the concert) remotely. For example, if a remote audience with a mobile device (e.g., a tablet or smartphone) virtually watching the concert where there is a 3D body model of the singer on the top of the scene, he can apply a selected set of 2D/3D AR effects to this 3D body model. He is also able to select his reconstructed 3D model which can reflect his body movements, and overlay it to the scene. It looks like he is with the main singer together. Furthermore, a cloud processing entity can distribute the rendered scene including the two 3D models to the other remote audiences. 3. In the cases documented above, latencies are critical in order to provide a “presence” feeling for both the audience and the actors/players. The use cases are the extensions to those discussed in clause 2.1, the media rendering procedures are similar to these ones described in clause 2.1.

Categorization

Type: AR, VR, MR, Edge Computing, GPU, Social Interaction Delivery: Downlink, Live Streaming, Interactive, Split rendering Device: HMD, Glasses, smartphones

Preconditions

On the device: 1. Application is installed that permits to consume the event and participate in the event. 2. The application uses existing hardware capabilities on the device, including A/V encoders and decoders, rendering functionalities as well as sensors. On the network: 3. Cloud computation capabilities are available to aggregate the user input. 4. Cloud/edge computation capabilities are available to render. At the live event: 5. Recording and playout functionalities to sense the remote audience.

Requirements in terms of Capabilities and QoS/QoE Considerations

QoS: - 1. Bit rates and latencies in the uplink are sufficient to provide user interaction to the cloud computation. - 2. Bit rates and latencies that are sufficient to render the viewport within the immersive limits. - 3. Sufficient bandwidth to deliver multiple media streams. - 4. Sufficiently low latencies end-to-end to provide a sense of presence. QoE: - 5. Providing sufficient AV experience to enable presence. https://xinreality.com/wiki/Presence - 6. Latencies low enough to provide a sense of crowd-reaction remote. - 7. Synchronization of user voice communication with the corresponding actions. - More details are necessary.

Feasibility and Industry Practices

Cost Analysis

Potential Standardization Status and Needs

# 6	Potential 5GMS Architecture Extensions

## 6.1	General

Editor’s Note: This clause will document the identified gaps. Identified gaps may go beyond the scope of the SA4 work, in which case the responsible groups will be contacted.

Editor’s Note: The potential extensions are mainly based on the identified relevant use cases in section 5.

## 6.2	EMSA Architecture

The EMSA architecture is an integration of the 5GMS architecture, the SA6 Edge architecture and the SA5 management architecture. The EMSA architecture is depicted in the following figure:

Figure 6.2-1: Reference edge-enabled 5GMS media architecture

The above architecture demonstrates how the edge-enabled 5GMS media architecture leverages the edge functionality and architectural elements specified in TS 23.558 [3]. In this illustration, the EEC, EES, and EAS are shown as parts of the MSH, 5GMS AF, and 5GMS AS, respectively.

This architecture implies the following:

1.	A Media Session Handler (MSH) that is edge-enabled is required to implement the EDGE-5 API.

2.	A 5GMS AF that is edge-enabled is required to support EES functionality including:

  - -	EDGE-3 API towards the EAS function of 5GMS AS instances.

  - -	EDGE-6 API for registering with an ECS function.

  - -	EDGE-9 API for media session relocation.

  - -	EDGE-1 API for supporting registration and provisioning of EEC functions, and discovery by them of EAS instances.

3.	A 5GMS AF that is edge-enabled may perform compute resource allocation using the MnS-C interface.

4.	A 5GMS AS that is edge-enabled is required to support EAS functionality including EDGE-3 API.

In general, in order to deploy edge support for the 5GMS application, each 5GMS function has to become edge-enabled and a union of each 5GMS function with its corresponding edge function (MSH + EEC, AF + EES, AS + EAS) would satisfy the requirements. In this regard, various deployment scenarios including the standalone existence of the edge functions can be considered.

Editor’s Note: Examples of the different deployment scenarios will be shown in Annex A.

This edge-enabled 5G media streaming architecture supports both client-driven as well as AP-driven management of the edge processing session.

In the client-driven approach, the 5GMS-Aware Application is aware of the support of edge processing in the network and takes steps, such as using the EDGE-5 APIs, to discover and locate an application server in the edge DN.

In Application Provider (AP)-driven approach, the 5GMS Application Provider provisions 5GMS features as normal within the scope of an M1 Provisioning Session, but additionally requests the 5GMS System to accommodate the provisioned features using edge computational resources, where appropriate. The 5GSMS AF acts on behalf of the 5GMS Application Provider to allocate processing resources based on the application’s needs. In order for this transparent resolution to work, SA2 is expected to define a solution to allow the AF to influence the DNS resolution for the domain name(s) related to the media distribution by the Application Provider as described in clause 4.3.2.

## 6.3	Generic Call Flows for Media Session Establishment

### 6.3.1	General

Two generic call flow for edge-enabled 5G media streaming session establishment are demonstrated below.

### 6.3.2	Client-driven edge discovery

Figure 8 outlines a detailed call flow for client-driven session establishment.

Figure 6.3.2-1: Client-driven session establishment

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g. to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. The steps are:

1.	Spawn ECS: In this step, a new ECS is instantiated to manage new or increased demand for edge processing.

2.	Spawn 5GMS AF: In this step, a new 5GMS AF that is edge-enabled is instantiated to handle new or increased demand for media sessions with edge processing.

3.	EES Configuration: The EES is configured for a specific Edge Data Network.

4.	EES Registration with ECS: The EES registers with the ECS that is in authority over the target EDN.

The 5GMS Application Provider Provisioning phase is performed prior to the establishment of any related media streaming sessions by the 5GMS Application Provider. Subsequent updates to the provisioning session are possible.

5.	Create Provisioning Session: In this step, the 5GMS Application Provider creates a new provisioning session.

6.	Provision 5GMS features: In this step, the 5GMS Application Provider may create different configurations such as Content Hosting, Reporting, Edge Processing, etc.

During the UE Edge Computing Discovery phase, the UE discovers an EAS instance offering 5GMS AS functionality.

7.	Application Initialization: The user launches the 5GMS-Aware Application. The application performs any required initialization steps.

8.	Locate EAS/5GMS AS: The Application Client requests the location of one or more suitable EAS instances offering the "5GMS AS" capability that are able to serve the application.

9.	Locate local EES: The EEC queries the ECS for a suitable EES.10. Register with EES: The EEC registers with the selected EES.

10.	Register with EES: The EEC registers with the selected EES.

11.	Request list of "5GMS AS" EAS instances: The EEC contacts the EES to query for one or more EAS instances offering the "5GMS AS" capability that can serve the session, using EAS discovery filters (see Table 8.5.4.2-2 in TS 23.558 [3]) provided by the Application Client, e.g. “5GMS AS” for EAS type, appropriate values for service feature(s), and other EAS characteristics.

The optional subflow is for provisioning an additional 5GMS AS instance if a suitable EAS instance offering the "5GMS AS" capability cannot be located. The steps are:

12.	Check resource template: The 5GMS AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.

13.	Instantiate new EAS/5MGS AS: The 5GMS AF requests the MnS to instantiate a new "5GMS AS" EAS instance with the specified requirements and considering parameters provided in the query by the EEC.

14.	Spawn 5GMS AS instance: The MnS creates a new instance of the EAS offering "5GMS AS" capability with the requested placement and resources.

15.	EAS configuration: The newly instantiated "5GMS AS" EAS instance is configured.

16.	Register EAS with EES: The newly instantiated EAS instance registers itself with the triggering EES.

17.	Configure provisioned features: This may include configuring and launching the server-side application in the 5GMS AS.

Completion of UE Edge Computing Discovery phase:

18.	List of suitable “5GMS AS” EAS instances: The EES/5GMS AF responds to the EEC with a list of “5GMS AS” EAS instances and their characteristics in an EAS discovery response (see Table 8.5.3.3-1 in TS 23.558 [3]).

19.	Select preferred “5GMS AS” EAS instance: The AC and/or EC select(s) a “5GMS AS” EAS instance from the provided list, based on the AC’s desired criteria.

After successful discovery of a “5GMS AS” EAS instance, the actual streaming session may start in the 5GMS Session phase:

20.	Start session: The 5GMS-Aware Application invokes the Media Streamer with appropriate streaming access parameters (e.g. a Media Player Entry such as a DASH MPD URL).

21.	Session starting event: The application informs the Media Session Handler about the start of a new 5GMS session.

22.	Retrieve service access information: The Media Session Handler retrieves Service Access Information from the 5GMS AF appropriate to the 5GMS session.

23.	Media transfer: The 5GMS-Aware Application connects to the selected EAS "5GMS AS" and the streaming starts.

24.	Method calls and notifications: Supporting information about the 5GMS session is passed from the Media Stream Handler to the Media Session Handler.

25.	Reporting, network assistance, and dynamic policy: The Media Session Handler exchanges supporting information about the 5GMS session with the 5GMS AF.

26.	End session: the 5GMS-Aware Application informs the Media Session Handler that the 5GMS session has ended.

27.	Session ending event: The Media Streamer informs the Media Session Handler about the end of the 5GMS session.

28.	Final reporting: The Media Session Handler performs any final reporting to the 5GMS AF.

### 6.3.3	AP-driven management of 5GMS edge processing

Figure 9 outlines a detailed call flow for the AP-driven management of edge processing. In the previous sequence, the optional provisioning of an additional 5GMS AS instance occurs in response to an explicit call from the Application Client to an on-board EEC, whereas in this sequence it occurs, if needed, as part of and in response to Application Provider provisioning. It may also be repeated at any time during the lifetime of the 5GMS Provisioning Session in response to satisfy demand levels (not illustrated for brevity).

Figure 6.3.3-1: AP-driven management of 5GMS edge processing

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g. to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. Steps 1–4 are identical to those described in clause 6.3.2 above.

The 5GMS Application Provider Provisioning phase is performed prior to the establishment of any related media streaming sessions by the 5GMS Application Provider. Subsequent updates to the provisioning session are possible. Steps 5–6 are identical to those described in clause 6.3.2 above.

The optional sub-flow to provision an additional 5GMS AS instance may be repeated multiple times on need basis to add new capacity or to increase existing capacity for edge processing. The edge processing capacity is tailored for the specific 5GMS Application Provider based on the information in the Provisioning Session. Steps 7–12 are identical to steps 11–16 described in clause 6.3.2 above.

After successful discovery, the actual streaming session may start in the 5GMS Session phase. Steps 13–21 are identical to steps 19–27 described in clause 6.3.2 above.

NOTE:	In this call flow, the Application Client (AC) and EEC are not used to discover the 5GMS AS location. Instead, a Media Player Entry may be provided to the Media Session Handler by the 5GMS AF in the Service Access Information at M5 (step 15), or otherwise the location of the 5GMS AS is provided directly to the 5GMS-Aware Application via (out of scope) interface M8.

## 6.4	Identified Gaps in Architecture and Procedures

### 6.4.1	General

Based on the recommended EMSA architecture in clause 6.2 and the high-level call flows defined in clause 6.3, we can identify the following gaps in the existing 5GMS architecture:

Gap 1	The procedures and services offered by the 5GMS AF and the 5GMS Client need to be defined.

Gap 2	Extensions to the M1 and M5 interfaces are required to provide additional procedures for provisioning and edge processing request.

Gap 3	Definition of the M3 interface between the 5GMS AF and 5GMS AS to manage edge processing.

Gap 4	Procedures to support session mobility and context transfer, triggered by the UE or by the 5GMS AF. In particular, the application and media-specific aspects of the Application Context Relocation would need to be specified.

Gap 5	Procedures in the 5GMS client and interactions with the application in the UE to discover and select an appropriate EAS/5GMS AS for the media processing needs of the application are only partially supported.

### 6.4.2	Justification of Identified Gaps

#### 6.4.2.1	Gaps in client-driven edge discovery

The following provides a reference to the gaps and the corresponding procedure step in the call flow at clause 6.3.2:

1.	Gap 2: In step 12, “The 5GMS AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.”.

How the EES embedded in the 5GMS AF gains access to the edge processing resource template needs to be investigated, and whether M1 supports providing this template during step 6 “Provision 5GMS features”. Otherwise, the API that the 5GMS AF uses to access this template needs to be identified and possibly defined.

2.	Gap 5: In steps 18 and 19, if more than one EAS is expected to be provided in the response, then one of the actors in the system needs to select the best one.

This process is not defined. For example, if the 5GMS AF selects the best EAS based on the template provided by the Application Provider during the “Provision 5GMS features” step, and based on EAS discovery filters provided by EEC, then this process must be indicated and required by “5GMS AF/EEC”.

#### 6.4.2.2	Gaps in AP-driven management of 5GMS edge processing

The following provides a reference to the gaps and the corresponding procedure step in the call flow at clause 6.3.3:

1.	Gap 2: In step 6, “Provision 5GMS features”, the desired characteristics of the EAS, such as geographical service area, service continuity support, and service features (as indicated in the EAS discovery filter supplied by the Application in the client-driven call flow) must be signalled to the 5GMS AF. It is not clear whether an API and/or resource for such signalling is supported by M1 in TS 26.512 [7].

# 7	Conclusions and Recommendations

Edge media processing is an enabler for a variety of immersive media streaming services, such AR/MR and cloud gaming, which require stringent QoS guarantees to operate properly. The 5GMS architecture has been developed to support media streaming services by leveraging 5G System functionality to optimize the streaming experience. Extensions to the 5GMS architecture are necessary to leverage 5G edge computing capabilities and integrate them into media streaming workflows.

The present document provides an overview of the different edge-related activities that are taking place in 3GPP. It also collects together a set of media streaming use cases that rely on or benefit from media processing at the edge. Based on these, a recommended architecture for edge media processing is derived and documented. The architecture is an integration of the 5G Media Streaming architecture with the architectures and procedures for establishment, control, and management of edge computing sessions that have been developed elsewhere in 3GPP.

It is recommended that normative work be initiated with the following objectives:

1.	Extend the 5GMS Architecture to support edge media processing according to the recommended architecture in clause 6.2.

2.	Enhance the procedures and services that are offered by the 5GMS AF and the 5GMS Client to enable establishment and management of media streaming session with edge processing.

3.	Extend the M1 interface to support the provisioning of edge media processing.

4.	Extend the M5 interface to support discovery and request of edge media processing resources.

5.	Define the M3 interface between the 5GMS AF and 5GMS AS to manage edge processing resources and sessions.

6.	Specify media and application context relocation information based on the Application Context Relocation procedures for session continuity in edge media processing.

Annex A:
Deployment Scenarios

## A.1	Deployment Scenario Option A

In one possible deployment option, the 5GMS functions and the SA6 MEC functions are co-located together and the 5GMS functions would be special instantiations of the corresponding SA6 MEC functions.

Figure 11: Potential deployment architecture option A

5GMS entities in the media function layer runs over the top of the SA6 MEC entities in the general function layer. Some parameter exchanges between these layers may be needed to trigger or receive the outcome of Edge-related procedures, i.e. the service provisioning, registration, EAS discovery, Service Continuity. For instance, during the EAS relocation procedure, the EES may help to find the target EAS instance and also to provide the selected target AS instance to the media function layer.

## A.2	Deployment Scenario Option B

Figure 12: Potential deployment architecture option B

The 5GMS functions and SA6 MEC functions are co-located and the SA6 MEC functions are realized inside the 5GMS functions. This means that the 5GMS functions also realize the SA6 MEC functionalities to enable the edge-based 5GMS service, including the necessary edge server management functionalities, like registration, service provisioning, EAS discovery, etc.

Annex B:
Selected Use Case Mapping

# B.1	Split Rendering

The split rendering use case covers scenarios where heavy graphics rendering is performed at the edge with low latency. The device receives a pre-rendered representation of the viewport and may run some pose correction (e.g. Asynchronous Time Warp) to adjust the view to the current user’s viewport.

The use case realization can be roughly described by the following walkthrough:

1.	An XR Device connects to the network and connects to an XR application,

a.	Sends static device information and capabilities (supported decoders, viewport),

2.	Based on this information, the XR server sets up encoders and formats,

3.	Loop:

a.	XR Device collects XR pose (or a predicted XR pose).

b.	XR Pose is sent to XR Server.

c.	The XR Server uses the pose to pre-render the XR viewport.

d.	XR Viewport is encoded with 2D media encoders.

e.	The compressed media is sent to XR device along with XR pose that it was rendered for.

f.	The XR device decompresses video.

g.	The XR device uses the XR pose provided with the video frame and the actual XR pose for an improved prediction using and to correct the local pose, e.g. using Asynchronous Time Warp.

Steps 1 and 2 relate to the session setup. Step 3 is about the operation of the service. Due to the nature of this use case, it is believed that the UE-management session establishment and control is more appropriate. The application is fully aware that the bulk of the rendering is to be performed on the edge with low latency and as such will request the necessary resources and the establishment of the edge session.

A break down of steps 1 and 2 is given to provide call flows for the session setup. In this call flow, it is assumed that the Application Provider provisions the 5GMSd AF with the processing configuration and resource needs prior to session setup. The EEC in the Media Session Handler discovers the EAS according to the client-driven discovery call flow documented in clause 6.3.2. The 5GMSd AF is responsible for allocating the actual resources using the MnS function, when an actual session is started.

The client-driven edge discovery call flow is used as a basis. The provided call flow is a condensed version of that call flow. Other variations of this walkthrough may be possible.

Figure 13: Call Flow for Split Rendering

The steps are provided here in detail:

1.	The provisioning step allows the Application Provider to configure required resources for its application sessions.

a.	The Application Provider sends a request to the 5GMS AF to create a new Provisioning Session.

b.	The 5GMS AF creates QoS and Compute resource templates. It may use services offered by the MnS-C and PCF.

2.	The 5GMS-Aware Application is started on the UE, and it connects to the Application Provider to create a new application session.

3.	The 5GMS-Aware Application informs the Media Session Handler about the starting session.

4.	The Media Session Handler creates a new session with the 5GMS AF. The Media Session Handler may provide EEC functionality to the 5GMS-Aware Application. The Media Session Handler provides information to the 5GMS AF about the required processing capabilities, application information, QoS requirements, etc.

5.	The 5GMS AF verifies that the requested QoS and compute resources are aligned with the resource templates provided in the Provisioning Session. Through its EES functionality, it starts by checking the EAS instances that registered with it and capable of running the service. If one or more suitable EAS instances are not found, the 5GMS AF/EES may use the MnS-C interface to allocate a new EAS instance for the application.

6.	The 5GMS AF confirms resource availability to the MSH and shares a list of potential EAS/5GMS AS to the Media Session Handler through its EDGE-1 interface.

7.	The Media Session Handler may provide the list of suitable EAS/5GMS AS instances to the Application. The Application picks one EAS/5GMS AS instance and establishes a connection to it. Alternatively, the application may use DNS resolution to discover the assigned EAS.

8.	Once the 5GMS-Aware Application is connected to the discovered EAS, it starts exchanging rendered viewport and pose information with the EAS/5GMS AS.

NOTE:	The Media Session Handler may throughout the lifetime of the session update the processing and network QoS requirements based on the application’s request and changing needs.

# B.2	Edge Caching

The following call flow summarises the procedures for provisioning and using an Operator CDN that automatically scales according to client load. This follows the generic call flow in clause 6.3.3 that is driven by the 5GMSd Application Provider. Consequently, the UE components are unaware that the Content Hosting feature of the 5GMSd AS is deployed at the network edge, and neither the AC nor the EEC play any role in the sequence.

Figure 14: Call flow for edge-enabled Content Hosting

The steps are as follows:

1.	The 5GMSd Application Provider provisions an edge-enabled Content Hosting feature in the 5GMSd AF.

a.	The 5GMSd Application provisions the Content Hosting feature by invoking M1d calls on the 5GMSd AF. As part of this, it marks the Provisioning Session as eligible for edge deployment.

b.	The 5GMSd AF creates QoS templates and Compute resources templates suitable for an edge-deployed 5GMSd AS with the Content Hosting feature.

c.	The 5GMSd AF may at this point decide to spawn one or more 5GMSd AS instances at various different edge locations to support the anticipated demand, based on the contents of the QoS template and Compute Resource template.

d.	Any newly spawned 5GMSd AS instances configure their respective EAS instances.

e.	Any newly spawned EAS instance registers itself with its EES.

2.	The UE uses 5GMS downlink streaming session.

a.	The 5GMSd-Aware Application may acquire Service Access Information from the 5GMSd Application Provider via M8d (out of scope).

b.	The 5GMSd-Aware Application invokes the Media Stream Handler using an M7d API call to inform it that a downlink streaming session is starting.

c.	The Media Stream Handler invokes the Media Session Handler using an M6d API call to inform it that a downlink streaming session is starting.

d.	The Media Session Handler requests Service Access Information from the 5GMSd AF.

The 5GMSd AF and the EES collaborate to determine the most appropriate entry point address to include in the Service Access Information for the requesting Media Session Handler, based for example on its IP address. Alternatively, the entry point may be generic domain name that resolves to the most appropriate EAS instance providing the “5GMSd AS” capability and the “content hosting” feature.

f.	The Media Stream Handler commences downlink media streaming via M4d from the 5GMSd AS edge instance indicated in the Service Access Information.

Content is acquired from the 5GMSd Application Provider via M2d as normal.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 12th February 2021 | 112-e | S4-210313 |  |  |  | Documentation of relocation solutions |  |
| 12th February 2021 | 112-e | S4-210314 |  |  |  | Updates to EAS discovery |  |
| 12th February 2021 | 112-e | S4-210320 |  |  |  | EMSA architecture |  |
| 4th March | MBS Adhoc | S4aI201139 |  |  |  | EMSA call flow updates from S4aI201140 | 0.5.2 |
| 4th March | MBS Adhoc | S4aI201144 |  |  |  | Online updates | 0.6.0 |
| 13th April 2021 | 113-e | S4-210633 |  |  |  | Updates to the SA6 procedure descriptions | 1.2.0 |
| 13th April 2021 | 113-e | S4-210631 |  |  |  | Updates to UE-driven call flow | 1.2.0 |
| 13th April 2021 | 113-e | S4-210625 |  |  |  | Editorial changes to the TR | 1.2.0 |
| 13th April 2021 | 113-e | S4-210626 |  |  |  | Gaps and conclusions | 1.2.0 |
| 13th April 2021 | 113-e | S4-210627 |  |  |  | Split rendering use case walkthrough | 1.2.0 |