3GPP TSG SA WG4 #114e	S4-210869

# E-meeting, 18th – 28th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | 0009 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial positioning of the chroma samples for VR Video Profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Tencent, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For for BT.2020 and BT.2100, the relative spatial positioning of the chroma samples is different than the default and for BT.709. Signaling this difference is recommended in the HEVC specification, but not on this specificatin. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Carry forward the recommendation from the HEVC specification for 3GPP media profiles using BT.2020 and BT.2100 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambigous specification leading to interop problems. Potential visual quality problems in the presentation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5.5, 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

#### 5.1.5.5	Colour information and Transfer Characteristics

A Bitstream conforming to the 3GPP VR Main H.265/HEVC Operation Point shall use either Recommendation ITU-R BT.709 [3] colorimetry or Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance for standard dynamic range (SDR).

Specifically, in the VUI, the colour parameter information shall be present, i.e.:

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	If BT.709 [3] is used, it shall be signalled by setting colour_primaries to the value 1, transfer_characteristics to the value 1 and matrix_coeffs to the value 1.

-	If BT.2020 [4] and SDR is used,

-	it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set, the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

A Receiver conforming to the 3GPP VR Main H.265/HEVC Operation Point shall be capable of decoding and rendering according to any of the two above configurations.

=====  CHANGE  =====

#### 5.1.6.5	Colour information and Transfer Characteristics

A Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall use either Recommendation ITU-R BT.709 [3] colorimetry or Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance for standard dynamic range (SDR). For High Dynamic Range (HDR), BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are used.

Specifically, in the VUI, the colour parameter information shall be present, i.e.:

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	If BT.709 [3] is used, it shall be signalled by setting colour_primaries to the value 1, transfer_characteristics to the value 1 and matrix_coeffs to the value 1.

-	If BT.2020 [4] and SDR is used,

-	it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set	the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

-	If BT.2020 [4] and ITU-R BT.2100 [11] are used in HDR,

-	it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 16 and matrix_coeffs to the value 9,

-	the chroma_loc_info_present_flag should be equal to 1, and if set, 	the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

A Receiver conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall be capable of decoding and rendering according to any of the three above configurations.

SEI messages for HDR metadata signalling may be used. The requirements and recommendations for Bitstreams and Receivers as documented in TS 26.116 [12], clause 4.5.5.7 also apply for the 3GPP VR Flexible H.265/HEVC Operation Point.