3GPP TSG|WG-S4 Meeting #115-e 	S4-211049

18th – 27th Aug 2021

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Extensions to the FLUS for supporting media processing on network

## Acronym: FLUS_NET

## Source(s): Tencent

## Unique identifier:

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a:

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

Media processing on the network is an important functionality for 5G applications. While TS 26.238 provides hints on the use of media processing on the network, the current specification is incomplete in this regard.

ISO/IEC 23090-8 Network-Based Media Processing (NBMP) provides a framework for instantiating, running, and monitoring media processes and workflows on Cloud and network. While NBMP specification is mentioned in TS 26.238 has an informative annex on NBMP, the deployment scenarios, the workflows, and the deployment scenarios are not clear.

Recently the FS_FLUS_NBMP study on the deployment of NBMP with FLUS was conducted. The study explores various deployment scenarios and their required call flows. This study identifies two extensions needed for TS 26.238 to enable those deployment scenarios.

## 4	Objective

The work item has the following objectives:

- Extending the Sink Resource properties to provide the capability of signaling the URL address of the Workflow Manager.

- Extending the Sink Configuration properties to provide the capability of the workflow creation and update request as well as the corresponding responses along with the status code of those responses.

## 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 26.238 | Extensions to the FLUS to provide capabilities to support NBMP deployment with FLUS | TSG#94 |  |

## 6	Work item Rapporteur(s)

Iraj Sodagar (irajs@live.com), Tencent

## 7	Work item leadership

SA4

## 8	Aspects that involve other WGs

None.

## 9	Supporting Individual Members

Supporting IM name

Tencent