3GPP TSG-SA4#115e Meeting 	S4- 211157

18th August – 27th August 2021

Source: 	Tencent

Title: 	Draft CR updates on Multiple grouping of 360 video

Document for	Agreement

Agenda item: 	12.5

# Introduction:

In SA4#114, Multiple grouping of 360 videos (S4-210932) was accepted to be included in the PD. This document proposes moving the text for the draft CR. The below updates are proposed to be added in section Y.6.8.1 of the draft CR.

First Change

Y.6.8.1

When an ITT4RT-Tx client sends a 360 video media stream to the MRF, it may include an attribute "a= other_overlays", which indicates if the MRF is allowed to group the media streams from that ITT4RT-Tx client with media streams from other ITT4RT-Tx clients. If all ITT4RT-Tx clients allow other_overlays, the MRF will group all itt4rt_group from all conference rooms into a single itt4rt_group from the MRF to the ITT4RT-Rx client. If one or more ITT4RT-Tx clients do not allow other_overlays, it would be grouped in a separate itt4rt_group group.

NOTE: If multiple itt4rt_group are created, an ITT4RT-Rx client would need to re-negotiate the session to switch to media streams from other itt4rt_group. However, doing so may add further burden on the signaling nodes.

End of Changes

# Proposal

The proposal is to move the agreed changes to the draft CR.