Source:	Samsung Electronics Co., Ltd., Tencent, Qualcomm Incorporated, Nokia Corporation

Title:	[FS_5GSTAR] Procedures and call flows

Agenda Item:	11.10

Document for:	Discussion and Agreement

- Introduction

This contribution contains the aligned outcome of the offline discussions from S4-211121, S4-211122, S4-211124 and S4-211125.

- Changes to clause 4.3

Add the following editor’s note under figure 4.3-1.

Editor’s Note:

- The figure needs to be updated to add the following sessions:

  - Add scene/immersive server

  - Create a scene session between Scene Manager and the Scene Server

  - Create media sessions for each object in loop 9

  - Possible others

- Changes to clause 6.2

Replace clause 6.2.4 as below.

### 6.2.4	Typical Procedures and call flows

#### 6.2.4.1	STAR-based media streaming

Figure 6.2.4.1-1 illustrates the procedure diagram for 5G immersive media downlink streaming using a STAR-based UE when all essential AR/MR functions in a UE are available without an assist by an edge.

Figure 6.2.4.1-1: STAR-based procedure for 5G downlink streaming

Prerequisites and assumptions:

-	The AR/MR Scene Manager includes immersive media rendering and scene graph handling functionalities.

-	The Media Client includes immersive content delivery and immersive media decoding functionalities.

-	The AR/MR Application in the STAR UE is run by the user.

-	The STAR UE initialises AR registration (starts analysing the surroundings where a user/UE is located), it namely:

a.	captures its surroundings via camera(s)

b.	analyses where the device is located

c.	registers the device into the analysed surroundings.

-	AR/MR Application and AR/MR Application Provider have exchanged some information, such as device capability or content configuration, for content rendering. The exchange procedures for device capability and content configuration are FFS.

-	AR/MR Application Provider has established a Provisioning Session and its detailed configurations has been exchanged.

-	AR/MR Application Provider has completed to set up ingesting immersive contents.

Procedures:

- The scene content is ingested by the 5GMSd AS.

- Service Announcement is triggered by AR/MR Application. Service Access Information including Media Client entry or a reference to the Service Access Information is provided through the M8d interface.

- Desired media content is selected.

- Optionally, the Service Access information is acquired or updated.

- The AR/MR Application initializes the scene manager with the entry point (full scene description) URL.

- The Media Client establishes the transport session for receiving the entry point (scene description).

- The Media Client requests and receives the full scene description.

- The entry point (scene description) is processed.

- The AR/MR Scene Manager requests the creation of a new AR/MR session from the AR Runtime.

- The AR Runtime creates a new AR/MR session.

- The Media Client and/or AR/MR Scene Manager notifies the necessary QoS information required to the Media Session Handler.

- The Media Session Handler shares the information with the 5GMSd AF, in some cases including desired QoS information.  Based on existing provisioning by the AR/MR Application Provider, the 5GMSd AF may request QoS modifications to the PDU sessions.

Media delivery session for each media strea/object.  For static AR objects, a simple URL may be provided in the entry point information for downloading the AR object media data.:

- For the required media content, the Media Client establishes the transport session(s) to acquire delivery manifest(s) information.

- The Media Client requests and receives the delivery manifest(s) from the 5GMSd AS.

- The Media Client processes the delivery manifest(s).  It determines for example the number of needed transport sessions for media acquisition.  The Media Client should be able to use the delivery manifest(s) information to initialize the media pipelines for each media stream.

- The AR/MR Scene Manager and Media Client configures the rendering and delivery media pipelines.

- The Media Client establishes the transport session(s) to acquire the media content.

- The latest pose information is acquired by the AR/MR Scene Manager and shared to the Media Client.

- The Media Client requests the immersive media data according to the delivery manifest processed, possibly taking into account pose information (e.g., viewport dependent streaming).

- The Media Client receives the immersive media data and triggers the media rendering pipeline(s), including the registration of AR content into the real world accordingly.

- The Media Client decodes and processes the media data. For encrypted media data, the Media Client may also perform decryption.

- The Media Client passes the media data to the AR/MR Scene Manager.

- The AR/MR Scene Manager renders the media, and passes the rendered media to the AR Runtime, which performs further processing such as registration of the AR content into the real world, and pose correction.

#### 6.2.4.2	EDGAR-based media streaming

Figure 6.2.4.2-1 illustrates the procedure diagram for 5G immersive media downlink streaming using an EDGAR-based UE.

Figure 6.2.4.2-1: EDGAR-based procedure for 5G downlink streaming

Prerequisites and assumptions:

-	The AR/MR Scene Manager includes immersive media rendering and scene graph handling functionalities.

-	The Media Client includes immersive content delivery and immersive media decoding functionalities.

-	The AR/MR Application in the EDGAR UE is run by the user.

-	The EDGAR UE initialises AR registration (starts analysing the surroundings where a user/UE is located), it namely:

a.	captures its surroundings via camera(s)

b.	analyses where the device is located

c.	registers the device into the analysed surroundings.

-	AR/MR Application and AR/MR Application Provider have exchanged some information, such as device capability or content configuration, for content rendering. The exchange procedures for device capability and content configuration are FFS.

-	AR/MR Application Provider has established a Provisioning Session and its detailed configurations has been exchanged.

-	AR/MR Application Provider has completed to set up ingesting immersive contents.

Procedures:

Steps 1~8: Identical to steps 1~8 from the STAR UE case (figure 6.2.4.1-1).

- Based on the processed scene description and the device capabilities, the 5GMSd AS/EAS is selected, and edge processes are instantiated using the processes defined in EDGE:

  - The AR/MR Scene Manager sends the scene description and the device capabilities to 5GMS AS. The 5GMS AS derives the EAS KPIs and if needed selects a new AS/EAS (through AF) based on the new KPI.  Then the edge processes are started, and a new entry proint URL is provided to the AR/MR Scene Manager.

  - The AR/MR Scene Manager derives the EAS KPIs from the scene description and device capabilities, requests the AF to provide the list of suitable EAS. Then the AR/MR Scene Manager selects the AS/EAS and requests to start the edge processes in the AS. The edge processes are started, and a new entry point URL is provided to the AR/MR Scene Manager.

- The AR/MR Scene Manager requests the simplified scene description. The edge processes derive the simplified scene description from the full scene description and provide it to AR/MR Scene Manager.

- The simplified entry point (simplified scene description) is processed.

Steps 12~19:	 Identical to steps 9~16 from the STAR UE case (figure 6.2.4.1-1).

- The Media Client establishes the transport session(s) to acquire the media content.

- The 5GMSd AS initiates and starts a media session. This media session forms a stateful session loop specific to the UE, containing steps 22~25:

Stateful media session loop (steps 22~25):

- The latest pose information is acquired by the AR/MR Scene Manager and shared to the Media Client.

- The Media Client sends the latest pose information to the 5GMSd AS.

- The 5GMSd AS performs pre-rendering of the media based on the latest received pose information. Pre-rendering may typically consist of decoding and rendering immersive media, and encoding the rendered (2D) media.

- The pre-rendered media is sent by the 5GMSd AS to the Media Client.

- The Media Client decodes and processes the media data. For encrypted media data, the Media Client may also perform decryption.

- The Media Client passes the media data to the AR/MR Scene Manager.

- The AR/MR Scene Manager renders the media, and passes the rendered media to the AR Runtime, which performs further processing such as registration of the AR content into the real world, composition, and pose correction.

- Changes to clause 6.3

Add the contents below into clause 6.3.4.

### 6.3.4	Typical Procedures and call flows

#### 6.3.4.1	STAR-based interactive immersive service

Figure 6.3.4.1-1 illustrates the procedure diagram for interactive immersive services using a STAR-based UE when all essential AR/MR functions in a UE are available without an assist by an edge.

Figure 6.3.4.1-1: STAR-based procedure for interactive immersive service

Editor’s Note

- Stateless or stateful?:

  - Hybrid: “scene update” depending on interactions

  - Establish scene session with application

  - Depending on interactions, you can get scene updates

  - Within the scene session, you have media delivery sessions

  - Show immersive scene generator in interactive immersive server

- STAR vs EDGAR:

  - The bigger loop interaction with the scene is common to both STAR and EDGAR (using interation info + pose info)

  - For EDGAR there is also a “smaller” loop for user based pre-rendering which requires pose information

- Further work:

  - Clean-up of figure, and add procedure descriptions to STAR case

  - Matching figure and descriptions for EDGAR case

- Changes to clause 6.4

Add the contents below into clause 6.4.4.

### 6.4.4	Typical Procedures and call flows

#### 6.4.4.1	STAR-based cognitive immersive service

Figure 6.4.4.1-1 illustrates the procedure diagram for cognitive immersive services using a STAR-based UE when all essential AR/MR functions in a UE are available without an assist by an edge.

Figure 6.4.4.1-1: STAR-based procedure for cognitive immersive service

Editor’s Note

- Similar to interactive case:

  - Bigger loop now includes cognitive data (UE sensor&media data)

  - Interactive processing changes to cognitive processing (it may not necessarily result in scene updates but also things like semantic perception results e.g. labels)

- Further work:

  - Match figure to interactive case and add descriptions

  - Possibly add EDGAR case

- Proposal

We propose to update clauses 4.3, 6.2, 6.3 and 6.4 of TR 26.998 according to the changes specified in this document.