3GPP TSG-SA4 Meeting #115-e	S4-211081

# Online,  18 – 28 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev |  | Current version: | 0.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on RTC and OTT QoS support for 5GSTAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSTAR |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This provides background information on WebRTC and some QoS considerations. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 4.5.6	WebRTC

The Web Real-Time Communication (WebRTC) is an API and set of protocols that enable real-time communication. The WebRTC protocols enable any two WebRTC agents to negotiate and setup a bidirectional and secure real-time communication channel. The WebRTC API exposes a JavaScript-based API to enable the development of applications that make use of the user’s existing multimedia capabilities to establish real-time communication sessions. However, access to the WebRTC set of protocols is possible through other programming languages.

The WebRTC protocols are developed and maintained by the rtcweb group in IETF. The WebRTC API is developed by W3C.

The WebRTC API can be decomposed into three layers:

- API for web developers that consists mainly of the MediaStream, RTCPeerConnection, and RTCDataChannel objects.

- API for browser and user agent implementers and providers

- Overridable API for audio/video capture and rendering and for network input/output, which the browser implementers can hook their own implementations to.

The main WebRTC stack components are the voice engine, the video engine, and the transport component.

The transport component ensures a secure transport channel for both parties of the call to communicate. It relies on an RTP protocol stack that runs over DTLS and leverages the SRTP profile.

The following diagram depicts the WebRTC protocol stack:

WebRTC delegates the signaling exchange to the application. The signaling protocol and format may be chosen by the application freely. However, the offer and answer are generated in the SDP format. The ICE candidates may be provided as strings or in JSON format.

WebRTC needs negotiation for the following purposes:

- Negotiation of the media streams and formats: this relies on the SDP offer/answer mechanism to generate and validate media streams and parameters. The negotiation procedure is depicted in the diagram below.

- Negotiation of the transport parameters: this relies on ICE to identify and test ICE candidates. Whenever a higher priority ICE candidate is validated, the connection will switch to it.

The following call flow shows an example of the ICE negotiation process:

Due to the separation of the negotiation of the transport parameters from the media parameters, appropriate QoS negotiation should consider consecutive and asynchronous changes to the connection parameters. In case a relay server, such as a TURN server, is deployed, the QoS negotiation should be extended to appropriately cover the outbound streams as well.