3GPP TSG-S4 Meeting #116-e	S4-211389

# Online, 10th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR |  | rev |  | Current version: | 0.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Potential solution for network slicing in 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_EXT |  |  |  |  |  | Date: |  |  | 4th November 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.12.6	Candidate Solutions

The application provider may use the provisioning interface to define a set of network slices that can be used for the media streaming sessions that it offers. It may associate each operation point (e.g. 4K HDR, HD, SD) with a dedicated network slice. The access to each network slice is restricted to the users that have a valid subscription to that service level. The list or groups of users that are to be authorized to use a certain slice is provided during the provisioning step and can be updated subsequently.

The following diagram shows a call flow for a potential solution using network slicing:

The Steps are described as follows:

- The provisioning step is performed by the Application Provider to define the configuration for all upcoming media streaming sessions

  - The AP requests the definition of one or more network slices that correspond to the operation points that it offers for the service

  - The 5GMS AF requests the creation or update of network slices based on the provided provisioning information

  - The MSH in the client retrieves the service access information from the 5GMS AF

  - The 5GMS AF responds with the service access information that contains a mapping between the operation point and the S-NSSAI that should be used by the session

- The application is started and retrieves the entry point for the media streaming session from the application provider

- The AP provides the application with the entry point, which contains information about the allowed operation points, e.g. in the DASH service descriptor

- The application informs the MSH about the start of a new media streaming session and indicates the selected operation point

- The MSH informs the 5GMS AF about the starting media streaming session and the desired operation point

- The 5GMS AF requests the application of the corresponding QoS profile to the session

- The MSH may request a modification to the PDU session to use the allowed network slice as indicated by the S-NSSAI.

- The AMF may request admission control to use the selected slice from the NSACF

- If allowed, the PDU session is established/updated to use the selected S-NSSAI

- The media streaming may start

Streaming session shall use the eMBB SST slice/service type. The Slice Differntiator shall be mapped to an operation point of the service that is indicated through the externalReference as defined in 7.9.3.1 of TS26.512.