Agenda item: 	8.7

Source: 	Tencent Cloud

Title: 	Application context relocation by reference

Document for	Discussion and Agreement

# Introduction

At one of the recent SA4’s MBS ad-hoc calls, the SA6 liaison response was shared that confirmed the following two concepts:

- Each server/or server application that connects to an application client, is an EAS. Therefore, even if a container consists of multiple applications, each is considered a separate EAS.

- The relocation of an application context can only occur through EES, i.e. the location of the storage is transferred from S-EAS to S-EES and then to T-EES and finally to T-EAS (EELManagedACR procedure).

# Discussion

# Currently supported application context transfer

TS23.558 in its EELManagedACR provides two methods for transferring the application context from S-EAS to T-EAS (TR23.558 clause 8.8.2.5):

“10.	The Application Context is transferred from S-EAS to the T-EAS at implementation specific time. In the case of EELManagedACR, the S-EES accesses the Application Context from the address as per step 1 and the S-EES and T-EES engage in the ACT from S-EAS to the T-EAS (obtained as per step 5) in a secure way. Further the T-EAS accesses the Application Context made available by the T-EES. If S-EAS performs the ACT directly with T-EAS, the specification of such process is out of scope of the present document.

NOTE 1:	The Application Context is encrypted and protected by the application layer. The S-EES and the T-EES engage in the packet level transport of the Application Context and they have no visibility to the content of the Application Context.”

One option is that while the S-EAS Application Context is accessible by S-EAS from its storage location, it seems that in EELManagedACR the transfer of the Application context is made by actual transfer of data in the following data flow:

S-EASS-EES T-EEST-EAS

The other alternative is provided is the direct transfer of Application Context from S-EAS to T-EAS but that is not part of the standard.

# Proposal: Transfer by reference

One additional alternative would be to use the EELManagedACR to exchange the connection information needed for the transfer between S-EAS and T-EAS through S-EES and T-EES but leave the actual transfer to S-EAS and T-EAS and outside of scope of the standardization. The value of this solution is that it provides a mechanism to signal the connection between two application servers without taking the burden of data transfer.

In this case, one of the following information can be provided:

- From S-EAS to T-EAS:

  - Source address of the Application Context storage

  - The unique Application Context ID

  - The supported protocols

  - Security information

  - Expiration time

- From T-EAS to A-EAS:

  - Destination address of the Application Context storage

  - The unique Application Context ID

  - The supported protocols

  - Security information

  - Expiration time

Optionally, information c, d, and e, and any additional information can be stored in a scheme, and a scheme identifier is used to signal the scheme:

- From S-EAS to T-EAS:

  - Source address of the Application Context storage

  - The unique Application Context ID

  - Scheme-identifier

  - Scheme

- From T-EAS to A-EAS:

  - Destination address of the Application Context storage

  - The unique Application Context ID

  - Scheme-identifier

  - Scheme

# Proposal

We propose the following:

- Include section 2 in the dCR for further discussion.

- Send a liaison response to SA6, request considering the approach proposed in section 2.