3GPP TSG-SA4 Meeting #116-e	S4-211494

# Online, 10th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.452 | CR | 0005 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to EVS Alternative Fixed-Point Source Code |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS, Nokia Corporation, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_codec-SA4 |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correct various issues as listed below: - Regression in AVQ decoding which was introduced with the previous CR and which could cause glitches in the decoder audio output: - Correct wrong condition in encoder-preprocessing to make sure LP analysis at 16kHz if ACELP@16k core was selected and buffers are updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct issues listed above, as described here: - re8_decode_base_index(): Correct AVQ sign handling. This affects the bit-exactness of the testsequences at various operation points. - pre_proc(): Correct erroneous condition to trigger LP analysis at 16kHz if ACELP@16k core was selected and buffers are updated. This doesn’t affect the bit-exactness of the testsequences. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Severe issues for the deployed codec, incl. glitches in the audio signal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Electronic attachment to 26.452, specifically: c-code/EVS_dec.exe c-code/EVS_cod.exe c-code/lib_com/disclaimer.c c-code/lib_dec/lead_deindexing_fx.c c-code/lib_enc/pre_proc_fx.c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.442 CR0048, 26.443 CR0045 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 26.444 CR0045 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start code change 1

diff -rwNBu 26452-g30/c-code/lib_com/disclaimer.c 26452-g40/c-code/lib_com/disclaimer.c

--- 26452-g30/c-code/lib_com/disclaimer.c	2021-08-06 09:40:46.000000000 +0200

+++ 26452-g40/c-code/lib_com/disclaimer.c	2021-11-02 16:21:58.775546000 +0100

@@ -12,8 +12,8 @@

{

fprintf(fPtr, "\n===========================================================================\n");

-    fprintf(fPtr,   " EVS Codec 3GPP TS26.452 August 12, 2021. \n");

-    fprintf(fPtr,   " Version 16.3.0\n");

+    fprintf(fPtr,   " EVS Codec 3GPP TS26.452 November 04, 2021. \n");

+    fprintf(fPtr,   " Version 16.4.0\n");

fprintf(fPtr,   "===========================================================================\n\n\n");

return 0;

}

diff -rwNBu 26452-g30/c-code/lib_dec/lead_deindexing_fx.c 26452-g40/c-code/lib_dec/lead_deindexing_fx.c

--- 26452-g30/c-code/lib_dec/lead_deindexing_fx.c	2021-08-06 09:40:47.000000000 +0200

+++ 26452-g40/c-code/lib_dec/lead_deindexing_fx.c	2021-11-02 16:21:58.407574000 +0100

@@ -189,7 +189,18 @@

m1 = sub(k1, 1);

m2 = 8;

move16();

-        IF(k1 == 7)

+        l = 1;

+        FOR ( i = 0; i < 8; i++ )

+        {

+            if ( x[i] == 0 )

+            {

+                l = 0;

+                move16();

+            }

+        }

+        test();

+        /* if the signs are constrained and all components are non-zero  */

+        IF ( ( k1 == 7 ) && ( l > 0 ) )

{

m2 = 7;

move16();

diff -rwNBu 26452-g30/c-code/lib_enc/pre_proc_fx.c 26452-g40/c-code/lib_enc/pre_proc_fx.c

--- 26452-g30/c-code/lib_enc/pre_proc_fx.c	2021-08-06 09:40:49.000000000 +0200

+++ 26452-g40/c-code/lib_enc/pre_proc_fx.c	2021-11-02 16:21:59.150526000 +0100

@@ -1069,7 +1069,7 @@

test();

test();

test();

-    IF( ( ((st->tcxonly == 0) || !(NE_32(st->core_brate_fx,FRAME_NO_DATA)||NE_32(st->core_brate_fx,SID_2k40)))&&EQ_16(st->L_frame_fx,L_FRAME16k)&&EQ_16(st->codec_mode,MODE2))||

+    IF( ( ((st->tcxonly == 0) || !(NE_32(st->core_brate_fx,FRAME_NO_DATA) && NE_32(st->core_brate_fx,SID_2k40)))&&EQ_16(st->L_frame_fx,L_FRAME16k)&&EQ_16(st->codec_mode,MODE2))||

( EQ_16(st->L_frame_fx,L_FRAME16k)  && EQ_16(st->codec_mode,MODE1)  ) )

{

/* update signal buffers */

End code change 1