| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 1.3.5 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VTM configuration files for S4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing VTM configuration files for Scenario 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of VTM configuration for S4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of VTM results for S4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

##### 8.2.3.5.1	Overview

Table 8.2.3.5.1-1 provides an overview of the H.266/VVC test tuples provided for this scenario. For provided bitstreams, reference H.266/VVC software implementation VTM13.0 and corresponding encoder configuration have been used. Source code and utilized configuration files are available here: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/releases/VTM-13.0

The details are also provided here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/VTM/streams.csv.

Table 8.2.3.5.1-1 Test Tuple generation with H.266/VVC for messaging and social sharing scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A01-VTM | 8.2.3.5.2.2 | S4-R01 | VTM13.0 | S4-VTM-01 | QP=[22, 27, 32, 37] | S4-A01-VTM-<QP> |
| S4-A02-VTM | 8.2.3.5.2.2 | S4-R02 | VTM13.0 | S4-VTM-01 | QP=[22, 27, 32, 37] | S4-A02-VTM-<QP> |
| S4-A03-VTM | 8.2.3.5.2.2 | S4-R03 | VTM13.0 | S4-VTM-01 | QP=[22, 27, 32, 37] | S4-A03-VTM-<QP> |
| S4-A04-VTM | 8.2.3.5.2.2 | S4-R04 | VTM13.0 | S4-VTM-01 | QP=[22, 27, 32, 37] | S4-A04-VTM-<QP> |
| S5-A05-VTM | 8.2.3.5.2.2 | S4-R01 | VTM13.0 | S4-VTM-02 | QP=[22, 27, 32, 37] | S4-A05-VTM-<QP> |
| S5-A06-VTM | 8.2.3.5.2.2 | S4-R02 | VTM13.0 | S4-VTM-02 | QP=[22, 27, 32, 37] | S4-A06-VTM-<QP> |
| S5-A07-VTM | 8.2.3.5.2.2 | S4-R03 | VTM13.0 | S4-VTM-02 | QP=[22, 27, 32, 37] | S4-A07-VTM-<QP> |
| S5-A08-VTM | 8.2.3.5.2.2 | S4-R04 | VTM13.0 | S4-VTM-02 | QP=[22, 27, 32, 37] | S4-A08-VTM-<QP> |

##### 8.2.3.5.2	Test Model and Configurations

###### 8.2.3.5.2.1	Common Parameters

To generate the anchor bitstreams, the following is applied:

- VTM Main 10 in low delay B configuration is used with screen content tools enabled. In addition, to enable screen content tools, the following settings are set to 1:  IBC, HashME, BDPCM.

- InternalBitDepth is 10 # codec operating bit-depth where all sequences (including 8 bit sequences) are coded with an internal bitdepth of 10 in accordance with [44] and metrics are calculated in 10 bits.

- Each source sequence is encoded with the following parameters: QP: [22, 27, 32, 37]

###### 8.2.3.5.2.2	S5-VTM-01: Main 10 Profile with no fixed Intra

Each source sequence is encoded with the following configurations:

-	The common parameters defined in clause 8.2.3.5.2.1.

-	IntraPeriod with no fix interval

-	GOPSize is equal to 8. Each B picture refers to up to 4 preceding pictures in display order within the GOP

The detailed settings are defined in the attached configuration file s4-vtm-01.cfg.

###### 8.2.3.5.2.3	S5-VTM-02: Main 10 Profile with fixed Intra every second

Each source sequence is encoded with the following configurations:

-	The common parameters defined in clause 8.2.3.5.2.1.

-	IntraPeriod such that 1 second is achieved

-	DecodingRefreshType: (2) IDR

-	IntraQPOffset and QPoffsets are set equal to 0

-	Each B picture refers to immediately preceding pictures in display order.

The detailed settings are defined in the attached configuration file s4-vtm-02.cfg.

===== END OF CHANGE 1 =====