3GPP TSG- S4 Meeting #116e	S4-211517

# Electronic Meeting, 10th November – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | <CR#> | Rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Annex B: Nmb8 User Plane ingest examples |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_EXT |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The intention of this informative annex is to start illustrating different ingest options. The terminology of properties is based on xMB and may get aligned to MBUSA Terminology at a later stage. Note: the document is not intended to drive naming of features or features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex B (informative):
Nmb8 User Plane ingest examples

## B.1	General

This annex provides an overview of the different Nmb8 User Plane protocol stacks for the defined distribution methods. (The Nmb2 Control Plane is used to select the distribution method.)

## B.2	Object Distribution Method

This clause illustrates the various Nmb8 options for Object Distribution Method.

### B.2.1	Object Distribution Method ingest with Pull

The AF provides the file URLs to the MBSTF via MBSF and the MBSTF fetches the files using HTTP. The MBSTF handles all MBS-related complexity, e.g. converting the HTTP message payload into an IP multicast suitable protocol, adding AL-FEC, etc. The AF delegates the delivery of MBS of Service Announcement Metadata to the MBS Client (i.e. IP multicast protocol details, etc) to the MBSF.

Figure B.2.1-1 illustrates a setup, where the MBSTF pulls files from a File Server. Nmb2 is used to provide the file URLs to the MBSTF.

Figure B.2.1-1: Object Distribution Method using Pull Mode (HTTP GET)

The following Session Properties allow the configuration of this Nmb8 mode:

-	Session type is set by the AF to Objects.

-	Ingest mode (property specific to the Session type) is set by the AF to Pull.

-	The Object manifest (property specific to the Session type, with the carousel delivery ancillary information) is updated by the AF with object URLs to be fetched and transmitted by the MBSTF and then send. The MBSF updates the Service Announcement according to the File List Object manifest information.

### B.2.2	Object ingest with Push

The AF pushes the objects into the MBSTF using HTTP. The MBSTF handles all MBS-related complexity, e.g. converting the HTTP message payload into an IP multicast suitable protocol, adding AL-FEC, etc. The AF delegates the delivery of MBS of Service Announcement metadata to the MBS Client (i.e. DASH MPD, IP multicast protocol details, etc.) to the MBSTF via MBSF.

Figure B.2.2-1 illustrates a setup in which an AF pushes objects into the MBSTF using HTTP PUT.

Figure B.2.2-1: Object Distribution Method using Push Mode (HTTP PUT)

The following Session Properties allow the configuration of this Nmb8 mode:

-	Session type is set by the AF to Objects.

-	Ingest mode (property specific to the Session type) is set by the AF to Push.

-	The MBSTF provides the Push URL (Session Type specific property) to the AF.

-	Display base URL contains the base URL for the objects. The MBSF replaces the Push URL part of the URL with the value of the Display base URL for inclusion in the FLUTE FDT instance and (in some cases) in the Service Announcement.

## B.3	Packet Distribution Method

This clause illustrates the various Nmb8 options for Packets Distribution Method.

### B.3.1	RTP Streaming mode

### The AF pushes the packets into the MBSTF using RTP/UDP. The MBSTF handles all MBS-related complexity, e.g. converting the RTP payload into an IP multicast suitable protocol, adding AL-FEC, etc.

Figure B.3.1-1 illustrates a setup of Packets Distribution Method using RTP Streaming Mode. The Nmb2 is used to provide the necessary information (for example RTSP URL or SDP) to the MBSTF via MBSF.

Figure B.3.1- 1: Packet Distribution Method using RTP Streaming Mode

The following Session Properties allow the configuration of this Nmb8 mode:

-	Session type is set by the AF to Packet distribution method.

-	Delivery mode configuration for User Plane (property specific to Session type) is set by the AF to RTP Streaming.

-	RTSP URL (Session Type specific property) is set by the AF and points to the RTSP server hosting the content. The MBSTF establishes the RTSP session to control the RTP flows.

-    or Session Description Parameters for User Plane (property specific to Session type) is set by the AF and contains the UDP flow mapping descriptions. The MBSTF provides the User Plane IP addresses (Session Type specific property) to the AF or joins the Multicast Address (Session Type specific property) to receive the RTP streaming.

### B.3.2	Proxy mode

The AF pushes the packets into the MBSTF using UDP. The MBSTF handles all MBS-related complexity, e.g. converting the UDP payload into an IP multicast suitable protocol.

Figure B.3.2-1 illustrates a setup of Packets Distribution Method using Proxy Mode. The Nmb2 is used to provide the necessary information to the MBSTF via MBSF.

Figure B.3.2-1: Packets Distribution Method using Proxy mode

The following Session Properties allow the configuration of this Nmb8 mode:

-	Session type is set by the AF to Packet distribution method.

-	Delivery mode configuration for User Plane (property specific to Session type) is set by the AF to Proxy.

-	Session Description Parameters for User Plane (property specific to Session type) is set by the AF and contains the UDP flow mapping descriptions. The MBSTF provides the User Plane IP addresses (Session Type specific property) to AF or joins the Multicast Address (Session Type specific property) to receive the UDP streaming.

### B.3.3 Forward only mode

### The AF pushes the packets encapsulated with IP Multicast into the MBSTF using UDP. MBSTF just forwards the packets to the MBS bearer.

Figure B.3.3-1 illustrates a setup of Packet distribution method with Forward only Mode. Nmb2 is used to provide the necessary information to the MBSTF via MBSF.

Figure B.3.3-1: Packets Delivery Method using Forward only Mode

The following Session Properties allow the configuration of this Nmb8 mode:

-	Session Type is set by the AF to Object distribution method.

-	Delivery mode configuration for User Plane (property specific to Session type) is set by the AF to Forward only.

-	Session Description Parameters for User Plane (property specific to Session type) is set by the AF and contains the UDP flow mapping descriptions. The MBSTF provides the User Plane IP addresses (Session Type specific property) to AF or joins the Multicast Address (Session Type specific property) to receive the UDP streaming.

**** Last Change ****