3GPP TSG SA WG4#117e	S4-220020

E-meeting, 14th – 23rd February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | draft | rev |  | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] 5GMS via eMBMS - Hybrid Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 07/02/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See work item |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 5GMS via eMBMS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item objectives not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document assumes that the dCR in S4-220018 is agreed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.10.5	Procedures for Hybrid Services: 5GMS content delivery via 5G System and eMBMS

## 5.10.5.1 	General

Hybrid services refer to the case for which a basic service is available on eMBMS and at the same time on unicast. The service on unicast may be richer and extended and may provide additional user experiences. For the hybrid use cases, it is expected that the content is statically provisioned on different delivery networks.

The call flow in Figure 5.10.5-1 extends the call flow defined in clause 5.6.1 to address generic cases for the hybrid use cases. Specific additional use cases are presented in the remainder of clause 5.10.5.

Figure 5.10.5-1: High Level Procedure for DASH content via Hybrid Delivery

Steps:

1:	The 5GMS Aware Application triggers the Service Provisioning and permits hybrid distribution of the media content

2:	Specifically the 5GMSd AF provisions for MBMS delivery, the MBMS delivery session is setup.and the BMSC informs the 5GMS AF on ingest resources.

1b): The 5GMS AF provider modifies the manifest to provide the different resources as available either on a DN or on MBMS hosted 5GMS AS. The manifest and the ingest resources are provided to the 5GMS Application Service Provider. The manifest may also be updated by the 5GMS Application Service Provider.

3: The media content is announced to the 5GMSd-aware application and the application request the entry points for the service.

4: Ingest starts from the 5GMSd Application Providers and the 5GMS AS may forward directly content that is delivered through to the BMSC.

5: The BMSC starts one or several MBMS Delivery Sessions.

6: The media content is selected by the application

7: The application initiates the media streaming session through Media session handler.

8: The media session handler initiates the MBMS streaming services

9: The media session handler through the information from the MBMS client informs the application that the service is ready.

10:	The application starts media playback

11:	The MPD/manifest is requested by the Media Player. The manifest may be on the local 5GMS AS (populated by the 5MBS delivery) or on the 5GMS AS on the DN or even on both.

12: The Media Player processes the MPD and identifies that resources are available on different 5GMS AS

13: The Media Player continuously together with the application selects the content and different content options.

14: The Media Player continuously checks with the Media Session handler and possibly forwarded to the MBMS client if the MBMS User Service data is available, how to use the different content. This depends on the hybrid scenario. Different policies may be considered.

15:	The Media Player requests initialization information either from local or remote 5GMS AS. The Media Player repeats this step for each required initialization segment.

16:	The Media Player receives the initialization information.

17:	The Media Player requests media segments according to the MPD either from local or remote 5GMS AS.

18:	The Media Player receives media segments and puts the information into the appropriate media rendering pipeline.

19:	Previous steps are repeated according to the MPD information.

#### 5.10.5.2 Interactive Service

In a specific hybrid scenario, an interactive service may be provided while the main resources are delivered via MBMS exclusively. In this case, the following instantations apply:

-	in step 2, the MPD only includes resources pointing to the local DN.

-	the steps 11 as well as 15-19 are all terminated in the local DN.

## 5.10.5.3 Session Continuity

In a specific hybrid scenario, the service is made available on both networks, on 5GMS AS and on MBMS. Only a single Representation of each Adaptation Set may be provided on broadcast. In this case, the following instantations apply:

-	in step 2, one Representation is of each Adaptation Set is provided on MBMS.

-	as long as the streaming service is accessible over MBMS, the Media Player selects the media content in the steps 11 as well as 15-19 from the local DN, the resources from the 5GMS AS are unavailable.

-	if the streaming service gets unavailable from MBMS, the Media Player switches to accessing the media content in the steps 11 as well as 15-19 from the 5GMS AS.

-	once the streaming service gets available again from MBMS, the Media Player switches back to accessing the media content in the steps 11 as well as 15-19 from the local DN.

## 5.10.5.4	Time-shifted viewing

In a specific hybrid scenario, the service is made available on both networks, on 5GMS AS and on MBMS. Only a single Representation of each Adaptation Set may be provided on broadcast. The content is available in timeshifted manner on the 5GMS AS. In this case, the following instantations apply:

-	in step 2, one Representation is of each Adaptation Set is provided on MBMS.

-	if the streaming service is accessible over MBMS and the user watches the live edge, the Media Player selects the media content in the steps 11 as well as 15-19 from the local DN, the resources from the 5GMS AS are unavailable.

-	if the user switches to time-shift mode or streaming service gets unavailable from MBMS, the Media Player switches to accessing the media content in the steps 11 as well as 15-19 from the 5GMS AS.

-	once the streaming service gets available again from MBMS and the user returns to the live edge, the Media Player switches back to accessing the media content in the steps 11 as well as 15-19 from the local DN.

## 5.10.5.5	Content or component replacement

In a specific hybrid scenario, the service is made available on both networks, on 5GMS AS and on MBMS. However, only a single Representation of selected Adaptation Sets may be provided on broadcast. Some Adaptation Sets are only available on unicast. In another case, two or more content alternatives for a period of time may exist, but only one alternative is provided over MBMS.

In this case, the following instantations apply:

-	in step 2, the MPD is generated to define the different content alternatives.

-	if the streaming service is accessible over MBMS and the user watches content available on broadcast, the Media Player selects the media content in the steps 11 as well as 15-19 from the local DN, the resources from the 5GMS AS are unavailable.

### -	if the user switches to content or content components, the Media Player switches to accessing the media content in the steps 11 as well as 15-19 from the 5GMS AS. If only a component is replaced, the Media Player accesses content from MBMS and 5GMS AS at the same time.