3GPP TSG-WG SA2 Meeting #149E e-meeting 	S4-220114

Elbonia, February 14th – 23rd, 2022

Source:	Huawei, HiSilicon

Title:	Discussion on the Nmb10 service design

Document for:	Discussion and agreement

Agenda Item:	8.7

Work Item / Release:	5MBUSA/ Rel-17

# 1. Introduction

In 5MBUSA, the combination of MBSF and MBSTF offers multicast and broadcast functionalities similar to the BM-SC in MBMS. It is agreed that the Nmb10 shall follow the xMB-C/MB2-C interface design which are different interfaces defined in different specs in MBMS. This paper intends to propose the Nmb10 service design based on some previous work in S4al221285 and S4-211520.

# 2. Discussion

In MBMS, the xMB-C/MB2-C interfaces are mainly used to create/update/delete the MBMS Service/Session. However, there are different aspects between BM-SC and the content provider/GC server for the xMB and MB2 logic. From the  xMB-C perspective, the Nmb10 interface need to support the creation/deletion/update of the MBS User Service and MBS Session (including MBS Ingest Session and MBS Distribution Session). From the MB2 aspect, the Nmb10 interface need to support the TMGI allocation/de-allocation and the MBS Session management (including MBS Ingest Session and MBS Distribution Session). So functionally, the Nmb10 interface can be divided into three aspects, MBS User Service management, TMGI management and MBS Session management.

Besides, the Nmbsmf_TMGI and Nmbsmf_MBSSession services have already been defined in SA2 and CT3. As shown in clause 7.1.1.2 “MBS Session Creation without PCC” of TS 26.247, the AF/NEF can directly invoke the above two services for support of GC services and transparent services (also known as services with Packet Distribution Method). It’s better to reuse these well-defined services for MBSF service design in Nmb10.

Besides, from the AF perspective, the exposed service APIs should be compatible and consistent no matter whether there is a combination of MBSF/MBSTF deployed. Additionally, it is much easier for other working groups in 3GPP or other organizations to refer and cite, e.g. to refer the GC service support from the SA6 perspective.

Based on the functionalities provided by Nmb10 interfaces, the service design can be splitted into the following parts independently. The MBS Session may not depend on the MBS Service Provisioning when the GC services is using the MBS system.

- TMGI allocation/de-allocation, which can mostly follow SA2 and CT3 design for support of GC services.

- MBS Service provisioning for MBS User Service Establishment, which mostly follows xMB interface design.

- MBS Session Provisioning for MBS user plane functionality configuration (e.g. ingestion and distribution), which follows both xMB and MB2 interface design.

  - MBS Ingest Session Provisioning

  - MBS Distribution Session Provisioning

Therefore, the Nmbsf services can be defined in the following aspects:

- Nmbsf_TMGI_Allocation/Deallocation

- Nmbsf_MBSService_Create/Delete/Modify

- Nmbsf_MBSSession_Create/Delete/Modify

# 3. Proposal

## X.1 Nmbsf services

### X.1.1	General

The following table illustrates the MBSF Services for MBS.

Table X.1.1-1: NF services provided by MBSF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nmbsf_TMGI | Allocate | Request/Response | NEF, AF |
|  | Deallocate | Request/Response | NEF, AF |
| Nmbsf_MBSService | StatusSubscribe | Subscribe/Notify | NEF, AF |
|  | StatusUnsubscribe |  | NEF, AF |
|  | StatusNotify |  | NEF, AF |
|  | Create | Request/Response | NEF, AF |
|  | Update | Request/Response | NEF, AF |
|  | Delete | Request/Response | NEF, AF |
|  | Get | Request/Response | NEF, AF |
| Nmbsf_MBSSession | Create | Request/Response | NEF, AF |
|  | Update | Request/Response | NEF, AF |
|  | Delete | Request/Response | NEF, AF |
|  | StatusSubscribe | Subscribe/Notify | NEF, AF |
|  | StatusUnsubscribe |  | NEF, AF |
|  | StatusNotify |  | NEF, AF |

### X.1.2	Nmbsf_TMGI service

#### X.1.2.1	General

Service description: NF Service Consumer can use this service to request the allocation of TMGIs and release allocated TMGIs.

#### X.1.2.2	Nmbsf_TMGI_Allocate service operation

Service operation name: Nmbsf_TMGI_Allocate

Description: This service is used by the NF Service Consumer to request the allocation of TMGI(s) or request to refresh previously allocated TMGI(s).

Inputs, Required: Number of TMGIs.

Inputs, Optional:  TMGI(s) (i.e. the TMGI(s) to be extended the expiry time).

Outputs, Required: TMGI(s), Expiry Time of the TMGI(s).

Outputs, Optional: None.

#### X.1.2.3	Nmbsf_TMGI_Deallocate service operation

Service operation name: Nmbsf_TMGI_Deallocate

Description: NF Service Consumer can use this service to request the release of TMGI(s).

Inputs, Required: TMGI(s).

Inputs, Optional: None.

Outputs, Required: Success or not.

Outputs, Optional: None.

### X.1.3	Nmbsf_MBSService service

#### X.1.3.1	General

Service description: The Nmbsf_MBSService service is invoked by the AF/NEF to manage the MBS User Service in the MBS system.

#### X.1.3.2	Nmbsf_MBSService_Create service operation

Service operation name: Nmbsf_MBSService_Create

Description: Create a new MBS User Service in the MBS system.

Input, Required: User Service Identifier.

Input, Optional: External service identifier, service class, service announcement mode, service language, target service areas, service names, service descriptions

Output, Required: Result Indication.

Output, Optional: cause, service properties, Service Announcement fetch URL.

#### X.1.3.3	Nmbsf_MBSService_Update service operation

Service operation name: Nmbsf_MBSService_Update

Description: Update the established MBS User Service.

Input, Required: User Service Identifier.

Input, Optional: Service properties.

Output, Required: Result Indication.

Output, Optional: Cause.

#### X.1.3.4	Nmbsf_MBSService_Delete service operation

Service operation name: Nmbsf_MBSSession_Delete

Description: Terminate the MBS User Service in the MBS system.

Input, Required: User Service Identifier.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

#### X.1.3.5	Nmbsf_MBSService_StatusNotify service operation

Service operation name: Nmbsf_MBSSession_StatusNotify

Description: This service operation is used by the MBSF to notify its consumers about the status change of the MBS User Service.

Input, Required: User Service Identifier, Event ID.

Input, Optional: Event information.

Output, Required: Result Indication.

#### X.1.3.6	Nmbsf_MBSService_StatusSubscribe service operation

Service operation name: Nmbsf_MBSSession_StatusSubscribe

Description: This service operation is used by the NF service consumer (e.g. NEF, AF) to subscribe notification about events related to the status of the MBS User Service.

Input, Required: MBS Session ID, event ID(s), Notification Target Address.

Output, Required: When the subscription is accepted: Subscription Correlation ID.

#### X.1.3.7	Nmbsf_MBSService_StatusUnsubscribe service operation

Service operation name: Nmbsf_MBSService_StatusUnsubscribe

Description: This service operation is used by the NF service consumer (e.g. NEF, AF) to unsubscribe to notification about events related to the status of the MBS User Service.

Input, Required: Subscription Correlation ID.

Output, Required: Result Indication.

### X.1.4	Nmbsf_MBSSession service

#### X.1.4.1 Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new MBS session during MBS session creation. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID.

Input, Optional: User Service ID, Date Ingest Session ID, Distribution Session ID, MBS Session type, MBS service area, Start date-time, End date-time, service description, QoS information, Input Transport Address Request, MBS (Distribution) Session state (active/inactive), Maximum bit rate, Maximum delay, distribution method, distribution operating method, object acquisition identifiers. For subscription to notifications event ID(s), Notification Target Address.

Output, Required: Result Indication.

Output, Optional: TMGI, Expiry Time of the TMGI, Cause, MB-UPF tunnel info, MBSTF tunnel endpoint address, MBSTF traffic flow information.

#### X.1.4.2	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSSession_Update

Description: Update the established MBS session.

Input, Required: MBS Session ID.

Input, Optional: User Service ID, Date Ingest Session ID, Distribution Session ID, MBS Session type, MBS service area, Start date-time, End date-time, service description, QoS information, Input Transport Address Request, MBS (Distribution) Session state (active/inactive), Maximum bit rate, Maximum delay, distribution method, distribution operating method, object acquisition identifiers. For subscription to notifications event ID(s), Notification Target Address.

Output, Required: Result Indication.

Output, Optional: Cause.

#### X.1.4.3	Nmbsmf_MBSSession_Delete service operation

Service operation name: Nmbsmf_MBSSession_Delete

Description: Release the MBS session. The MBS session is deleted and the subscription (if any) to notifications about events related to the status of the MBS session is terminated.

Input, Required: MBS Session ID.

Input, Optional: User Service ID, Date Ingest Session ID, Distribution Session ID.

Output, Required: Result Indication.

Output, Optional: Cause.

#### X.1.4.4	Nmbsmf_MBSSession_StatusNotify service operation

Service operation name: Nmbsmf_MBSSession_StatusNotify

Description: This service operation is used by the MBSF to notify its consumers about the status change of the MBS session.

Input, Required: MBS Session ID, Event ID.

Input, Optional: Event information.

Output, Required: Result Indication.

#### X.1.4.5	Nmbsmf_MBSSession_StatusSubscribe service operation

Service operation name: Nmbsmf_MBSSession_StatusSubscribe

Description: This service operation is used by the NF service consumer (e.g. NEF, AF) to subscribe notification about events related to the status of the MBS session.

Input, Required: MBS Session ID, event ID(s), Notification Target Address.

Output, Required: When the subscription is accepted: Subscription Correlation ID.

#### X.1.4.6	Nmbsmf_MBSSession_StatusUnsubscribe service operation

Service operation name: Nmbsmf_MBSSession_StatusUnsubscribe

Description: This service operation, which is applicable for broadcast MBS session, is used by the NF service consumer (e.g. NEF, AF) to unsubscribe to notification about events related to the status of the MBS session.

Input, Required: Subscription Correlation ID.

Output, Required: Result Indication.

# 3. Conclusion and proposal(s)

It’s proposed to agree the section 3 as a pCR for the network function services in 5MBUSA.