3GPP TSG-WG SA4 Meeting #117E e-meeting  	S4-220118

Elbonia, February 14th – 23rd, 2022

# Title:	[DRAFT] LS Reply on SA4 requirements for QoE

# Response to:	LS (R2-211665)

# Release:	Rel-17

# Work Item:	NR_QoE-Core

Source:	SA4

To:	RAN2

Cc:	RAN3

Contact Person:

Name:	Qi Pan

Tel. Number:

E-mail Address:	panqi8@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA4 thanks RAN2 about the LS on SA4 requirements for QoE and provides the answers as following.

“The QoE configuration shall only be checked by the client when each session starts, and thus all logging and reporting criterias for an ongoing session shall be unaffected by any QoE configuration changes received during that session. This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts. Thus changes to the QoE configuration will only affect sessions started after these configuration changes have been received.”

Question 1: What is the meaning and motivation for the above-mentioned requirement in clause 10.1 of TS 26.247?

Answer: It makes sense to collect QoE measurements for a full session. If a QoE session is stopped and re-started, it means a new QoE session is started.

Question 2: Does the requirement copied above mean that the QoE client should continue QoE measurements and reporting for ongoing sessions even when the UE moves out of the QoE area scope, or does it mean the geographical filtering change does not affect ongoing session filtering i.e. the old geographical filtering instead of the new geographical filtering is applicable for ongoing sessions?

Answer: When the geographical filtering changes, the ongoing sessions will not be impacted as indicated in the clause 10.1 of TS 26.247. In RAN3 LS R3-212976, RAN3 has agreed that the network is responsible for keeping track of whether the UE is inside or outside the area and configures / releases configuration accordingly. The UE can just follow the network orders (configuration/release).

Question 3: Is there a difference from QoE session continuity point of view between release caused by the UE moving out of area and release caused by some other reasons?

Answer: No. The UE will anyway release the QoE session for both cases.

Questions 4: RAN2 has noticed the LocationFilter in the Quality Reporting Scheme for DASH in chapter 10.5 in 26.247. Can this LocationFilter be used to fulfil the requirement in clause 10.1, if the requirement is confirmed?

Answer: In TS 26.247, the network can only choose one filtering mechanism based on the following statement like “Note that if geographical filtering is handled on the network side (i.e. QoE reporting is turned on/off  by the network depending on the UE location), no LocationFilter should be specified in the QoE Configuration, as this would mean two consecutive filterings.”

Question 5: Is the application aware of the UEs current cell?

Answer: The application can get aware of the UE’s current cell via the AT Command +CREG. If the network is responsible for keeping track of UE mobility as agreed in RAN3, there is no need for APP to configure the location filtering as stated above.

Question 6: What is the difference between the geographical area handled in RAN and the geographical area handled in the application by means of LocationFilter?

Answer: And if we consider cell list as filtering, both mechanisms will have the same effects.

Question 7: Are both location filter and RAN based filtering needed?

Answer: Both are needed, and TS 26.247 has defined that the network can only choose one

Question 8: Is LocationFilter available only for DASH or also for MTSI and other service types?

Answer: It’s available for DASH, VR and MTSI services.

2. Actions:

To RAN2 group.

ACTION: 	SA4 asks RAN2 group to take the above reply into account and provide feedback if any.

3. Date of Next TSG SA WG4 Meetings:

SA4#118-e	6 – 14 April  2022		E-Meeting

SA4#119-e	11 – 20 May 2022		E-Meeting