3GPP TSG SA WG4#117-e meeting	S4-220157

# 14th – 23rd February 2022														Revision of S4aQ210176

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | <CR#> | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for HaNTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HaNTE |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UEs featuring non-traditional earpieces pose challenges for handset mode acoustic testing. For example, a UE may exclusively use a vibrating display to produce sound when operating in handset mode, offering no clearly identifiable centre of an earpiece to position the headset for testing. Additionally, such UE could have its acoustic response affected by the choice of handset positioner mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirement for receive frequency response at maximum volume setting with specific mask |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handsets Featuring Non-Traditional Earpieces (HaNTE) handsets with poor audio performance cannot be properly identified with existing requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 6.4.2, 7.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 26.131 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.4.2	Handset and headset UE receiving

#### 5.4.2.1	Nominal volume (handset and headset UE)

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction shall be within a mask, which can be drawn with straight lines between the breaking points in table 2 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 2: Handset and headset receiving sensitivity/frequency mask for 8N application force

| Frequency (Hz) | Upper limit (dB) 8 ± 2 N | Lower limit (dB) 8 ± 2 N |
| --- | --- | --- |
| 100 | 6 |  |
| 300 | 6 | -6 |
| 3 400 | 6 | -6 |
| 4 000 | 6 |  |
| NOTE 1: All sensitivity values are expressed in dB on an arbitrary scale NOTE 2: The basis for the target frequency responses in send and receive is the orthotelephonic reference response measured between 2 subjects 1 m apart under free-field conditions and assumes an ideal receive characteristic. Under these conditions the overall frequency response shows a rising slope. The present document no longer uses the ERP as the reference point for receive but the diffuse-field. With the concept of diffuse-field based receive measurements a rising slope for the overall frequency response is achieved by a flat target frequency response in send and a flat diffuse-field based receive frequency response. |  |  |

Figure 2: Handset and headset receiving sensitivity/frequency mask for 8N application force

Compliance shall be checked by the relevant test described in TS 26.132.

#### 5.4.2.2	Maximum volume (handset UE [featuring non-traditional earpieces])

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction shall be within a mask, which can be drawn with straight lines between the breaking points in table 2M on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 2M: Handset receiving sensitivity/frequency mask for 8N application force

| Frequency (Hz) | Upper limit (dB) 8 ± 2 N | Lower limit (dB) 8 ± 2 N |
| --- | --- | --- |
| 100 | 6 |  |
| 300 | 6 | -6 |
| 3 400 | 9 | -6 |
| 4 000 | 9 |  |
| NOTE 1: All sensitivity values are expressed in dB on an arbitrary scale NOTE 2: The basis for the target frequency responses in send and receive is the orthotelephonic reference response measured between 2 subjects 1 m apart under free-field conditions and assumes an ideal receive characteristic. Under these conditions the overall frequency response shows a rising slope. The present document no longer uses the ERP as the reference point for receive but the diffuse-field. With the concept of diffuse-field based receive measurements a rising slope for the overall frequency response is achieved by a flat target frequency response in send and a flat diffuse-field based receive frequency response. |  |  |

Figure 2M: Handset receiving sensitivity/frequency mask for 8N application force

Compliance shall be checked by the relevant test described in TS 26.132.

### 6.4.2	Handset and headset UE receiving

#### 6.4.2.1	Nominal volume (handset and headset UE)

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction, shall be within a mask, which can be drawn with straight lines between the breaking points in table 10 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 10: Handset and headset receiving sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) 8 ± 2 N | Lower limit (dB) 8 ± 2 N |
| --- | --- | --- |
| 100 | 6 |  |
| 200 | 6 | -10 |
| 300 | 6 | -6 |
| 1 000 | 6 | -6 |
| 2 000 | 8 | -6 |
| 5 000 | 8 | -6 |
| 6 300 | 8 | -12 |
| 8 000 | 8 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 10: Handset and headset receiving sensitivity/frequency mask

Compliance shall be checked by the relevant test described in TS 26.132.

#### 6.4.2.2	Maximum volume (handset UE [featuring non-traditional earpieces])

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction, shall be within a mask, which can be drawn with straight lines between the breaking points in table 10M on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 10M: Handset receiving sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) 8 ± 2 N | Lower limit (dB) 8 ± 2 N |
| --- | --- | --- |
| 100 | 6 |  |
| 200 | 6 | -10 |
| 300 | 6 | -6 |
| 1 000 | 6 | -6 |
| 2 000 | 11 | -6 |
| 5 000 | 11 | -6 |
| 6 300 | 11 | -12 |
| 8 000 | 11 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 10M: Handset receiving sensitivity/frequency mask

Compliance shall be checked by the relevant test described in TS 26.132.

#### 7.4.2.1	Handset UE receiving

#### 7.4.2.1.1	Nominal volume

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction, shall be within a mask, which can be drawn with straight lines between the breaking points in table 19 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 19: Handset receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 5 |  |
| 200 | 5 | -8 |
| 250 | 5 | -5 |
| 5000 | 5 | -5 |
| 12500 | 5 | -11 |
| 16000 | 5 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

It is desired as a performance objective that the receiving sensitivity/frequency response be within the mask which can be drawn with straight lines between the breaking points in table 20.

Table 20: Handset receiving sensitivity/frequency performance objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 4 |  |
| 200 | 4 | -4 |
| 250 | 4 | -4 |
| 5000 | 4 | -4 |
| 12500 | 4 | -7 |
| 16000 | 4 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 16: Handset receiving sensitivity/frequency masks. The frequency response of the EVS codec operating as specified in TS 26.132 (super-wideband 24,4kbit/s, using the specified P.501 speech test signal), is plotted for reference, normalized to 0dB at 1kHz.

A UE operating in super-wideband mode shall pass the super-wideband requirements (i.e. when measured according to in 1/3rd octaves) as specified in Table 19, and shall also pass the wideband sensitivity/frequency characteristics requirements in the wideband range using the wideband measurement (i.e. measured in 1/12th octaves) as specified in Table 10.h

Compliance shall be checked by the relevant test described in TS 26.132.

##### 7.4.2.1.2	Maximum volume ([handset UE featuring non-traditional earpieces])

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction, shall be within a mask, which can be drawn with straight lines between the breaking points in table 19M on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 19M: Handset receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 5 |  |
| 200 | 5 | -8 |
| 250 | 5 | -5 |
| 5000 | 8 | -5 |
| 12500 | 8 | -11 |
| 16000 | 8 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

It is desired as a performance objective that the receiving sensitivity/frequency response be within the mask which can be drawn with straight lines between the breaking points in table 20M.

Table 20M: Handset receiving sensitivity/frequency performance objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 4 |  |
| 200 | 4 | -4 |
| 250 | 4 | -4 |
| 5000 | 7 | -4 |
| 12500 | 7 | -7 |
| 16000 | 7 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 16M: Handset receiving sensitivity/frequency masks. The frequency response of the EVS codec operating as specified in TS 26.132 (super-wideband 24,4kbit/s, using the specified P.501 speech test signal), is plotted for reference, normalized to 0dB at 1kHz.

A UE operating in super-wideband mode shall pass the super-wideband requirements (i.e. when measured according to in 1/3rd octaves) as specified in Table 19M, and shall also pass the wideband sensitivity/frequency characteristics requirements in the wideband range using the wideband measurement (i.e. measured in 1/12th octaves) as specified in Table 10M.

Compliance shall be checked by the relevant test described in TS 26.132.

- End of changes