Source:	Ericsson LM

Title:	IVAS-7a - Additional processing tools

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

At SA4#113-e, the source continued the discussion on methods for level normalization, specifically for stereo and multi-channel audio [1]. Appropriate level adjustment methods are necessary for setting the nominal operating level and to avoid biases in listening tests due to level differences.

In the discussion, methods used in some past standardization efforts, e.g., G.711.1/G.722 stereo extension [2], ITU-T P.700 [3] and ITU-R BS.1770 [4] were brought up. A few companies reported positive experiences with using BS.1770 for level normalization, and the question was raised if there are available implementations that may be used in the IVAS standardization effort.

The source has since then worked on an example implementation of BS.1770, which is currently under review for inclusion in Open ITU STL [5].

Further, as announced at SA4#116-e, implementations of spatial distortion reference units (SDRU and ESDRU) are also intended to be submitted to STL. The source has now contributed an example implementation of the ESDRU to Open ITU STL, also under review.

# Proposal

It is proposed that the new tools made available in STL are incorporated in the draft IVAS-7a (Processing plan for selection phase) [6] as suggested below. Please note that the proposal is based on some further proposed amendments, all collected in [7], and that numbering is not accurate and would need to be adjusted to current version and further updates based on the agreements in SA4.

In order to describe the pre-processing steps prior to level normalization for stereo, some additional stereo processing stages including interleaving/de-interleaving and high-pass filtering are proposed. It is noted that the cut-off frequency for such high-pass filter may potentially be reduced compared to the pre-processing that was used for EVS.

# References

- 3GPP Tdoc S4-210569: Common tools for level normalization, Ericsson LM

- TD 178 (WP 3/16): Processing test plan for the ITU-T Joint G.722/G.711.1 SWB/Stereo extension Optimisation/Characterization phase, V0.5, March 2011

- Recommendation ITU-T P.770: Calculation of loudness for speech communication (06/2019)

- Recommendation ITU-R BS.1770-4: Algorithms to measure audio programme loudness and true-peak audio level (10/2015)

- Open ITU STL: https://github.com/openitu/STL

- Pdoc IVAS-7a: Processing plan for selection phase, v.0.2.0

- S4-220166: Proposed updates for IVAS-7a, Ericsson LM

## Processing for stereo inputs

### Pre-processing

#### Step 1: De-interleaving of 48 kHz stereo input file

Figure 1: Pre-processing for stereo input

#### Step 2: Windowing and high-pass filtering per 48 kHz left/right channel file

Figure 2: Windowing and high-pass filtering (48 kHz)

#### Step 3: Interleaving of high-pass filtered left/right channel files

Figure 3: Interleaving of high-pass filtered channels (48 kHz)

#### Step 4: Level adjustment, resampling, concatenation and low/high level scaling

Note: Resampling from 48 kHz to corresponding sampling rate for WB, SWB processing.

Figure 4: Level adjustment (48 kHz)

### Reference conditions

Editor’s note: Binaural presentation of stereo is not covered in the figures below and is for further discussion.

Figure 5: Processing of Reference conditions.

### CuT conditions (non-JBM)

Figure 6: CuT processing for non-JBM conditions. The EID is bypassed for error-free conditions.

### CuT conditions (JBM)

Figure 7: CuT processing for JBM conditions.

# Processing Modules

This section specifies the operation of pre- and post-processing modules being utilized by the processing stages.

## Pre- and post-processing operations

(…)

### Level adjustment

(…)

#### BS.1770 Level adjustment

To normalize the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N –lev L -conf xxxx Input48 Output48

where N is the number of channels, L the target level in LKFS (default: -26), and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

#### BS.1770 Level measuring

To measure the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N -conf xxxx Input48

where N is the number of channels, and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

(…)

### Interleave audio files

#### Interleave stereo files

To interleave stereo files, use:

stereoop.exe -interleave inputL inputR output

### De-interleave audio files

#### De-interleave stereo files

To de-interleave stereo files, use:

stereoop.exe -split input outputL outputR

(…)

## Processing

This section describes the main processing modules utilized by the processing stages.

(…)

### ESDRU reference conditions

To generate an ESDRU reference at distortion level alpha for a file Input48, use:

esdru.exe -e_step 0.5 -seed 1 -sf fs alpha Input Output

where fs is the sample frequency, default 48 kHz, where alpha=[0.0,…,1.0] and alpha=0.0 corresponds to the highest distortion level.