3GPP TSG SA WG4#117e		S4-220066

E-meeting, 14th – 23rd February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.347 | CR | draft | rev |  | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [TEI-17+TRAPI] Extensions to MBMS-URLs for ROM Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17+TRAPI |  |  |  |  |  | Date: |  |  | 07/02/2022 |
| Category: | C |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Different scenarios exist for which other services, applications or web sites use a URL to point to the service, or to a specific resource of a service. There are cases for which finding a service should be possible by - Using a URL that points to the service or an object delivered on a service - No unicast is available - Service acquisition is quick This particularly applies for ROM services. Examples are - Pointers from Emergency Alert services to Emergency Broadcast media services - Fast redirection from unicast to broadcast - Reference in service guides such as DVB-I The MBMS URL is a good choice for thos, but is underspecified |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define an MBMS URL that points to ROM services using the TMGI - Add additional parameters to the URL in order to support quickly finding the service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service announcements can not be found quickly and will delay the access the to the ROM service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.2.4 (new), Annex F (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service; Stage 1".

[3]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[6]	3GPP TR 26.852: "Multimedia Broadcast/Multicast Service (MBMS); Extensions and profiling".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[9]	Object Management Group: "Interface Definition Language™ (IDL™) 4.0".

[10]	IETF RFC 3066: "Tags for the Identification of Languages".

[11]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[12]	3GPP TS 29.116: "Representational state transfer over xMB reference point between content provider and BM-SC".

[13]	IETF RFC 7595: "Guidelines and Registration Procedures for URI Schemes".

[14]	IETF RFC 7230: " Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[15]	IETF RFC 7553, "The Uniform Resource Identifier (URI) DNS Resource Record"

[16]	IETF RFC 6335, "Internet Assigned Numbers Authority (IANA) Procedures for the Management of the Service Name and Transport Protocol Port Number Registry"

[17]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[18]	3GPP TS 36.211, "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[19]	3GPP TS 36.104, "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[20]	3GPP TS 24.116, "Stage 3 aspects of system architecture enhancements for TV services".

=====  CHANGE  =====

### 8.2.4	MBMS-URL for ROM Services

For Receive-only Mode (ROM) services, DNS resolution or preprovioning is not always possible. Hence, an extended URL from is needed in order to find service announcements and services. For this purpose, a specific extension and instantiation of the generic URL introduced in clause 8.2.2 is provided in this clause. In particular, the following domain name is explicitly reserved for Receive-only Mode services:

mbms://<tmgi>.tmgi.3gpp.org

where <tmgi> is the ASCII hexadecimal representation of the TMGI of the ROM service, encoded in up to twelve characters.

Leading zeros in the hexadecimal value may be omitted from the <tmgi> string. For example, the two MBMS-URLs mbms://000000901056.tmgi.3gpp.org and mbms://901056.tmgi.3gpp.org encode an identical TMGI value.

If such a URL is provided, then the ROM service shall have the following properties:

-	Based on the requirements in TS 24.116 [20], clause 6.3.3:

-	The first digit of the TMGI value is always zero, indicating the service is a ROM service as defined in TS 26.346 [5].

-	If the service is a Service Announcement service:

-	The first five digits of the TMGI value are always zero, as specified by TS 26.346 [5].

-	The service shall include a required capability '23’, i.e. the Service Announcement the MBMS User Service Discovery/Announcement Profile 1b as documented in clause L.3 of TS 26 346 [5] shall be used.

-	If the service is a User Service, then the first digit of the TMGI value is zero and at least one of the second to fifth digits is non-zero, as specified by TS 26.346 [5].

-	The User Service Description should include the r16:ROMSvcRfParams child element and signal EARFCN for frequency, subcarrier spacing and bandwidth.

For such a URL, additional mid-part &name=value pairs are defined according to Table 8.2.4-1.

Table 8.2.4-1: Name–value pairs in ROM MBMS-URL

| Parameter name | Value and semantics | Optionality |
| --- | --- | --- |
| serviceArea | The list of service areas that are also present in the userServiceDescription. | Optional If omitted, the serviceArea value is unknown. |
| frequency | The Frequency value also present in the userServiceDescription, coded as EARFCN, as defined in 3GPP TS 36.101 [17]. | Optional If omitted, the frequency value is unknown. |
| subcarrierSpacing | The subcarrierSpacing value also present in the userServiceDescription, coded as subcarrier spacing (∆f) values per 3GPP TS 36.211 [18]. | Optional If omitted, the subcarrierSpacing value is unknown. |
| bandwidth | The bandwidth value, restricted to be one of the specified channel bandwidth values in 3GPP TS 36.104 [19]. | Optional If omitted, the bandwidth value is unknown. |
| serviceId | The serviceId in the Service Announcement channel that points to the referenced User Service. | Optional If omitted, the MBMS-URL points to an SA service. |

If the URL points to a Service Announcement channel, then information from the URL may be used by an MBMS-URL Handler to generate a User Service Description Bundle including a userServiceDescription and an associated SDP fragment. A bundle template is provided in Annex F. This SA file may be used to initiate the MBMS Client using the addSA() method defined in clause 6.2.3.22.

A few examples are provided in the following:

EXAMPLE 1:	This URL identifies a Service Announcement channel in a specific area and with receiver frequency parameters.

mbms://901056.tmgi.3gpp.org&serviceArea=40201&frequency=68616&subCarrierSpacing=1.25&bandwidth=8

EXAMPLE 2:	This URL identifies a Service Announcement channel in a different area and with receiver frequency parameters.

mbms://901056.tmgi.3gpp.org&serviceArea=65535,65536&frequency=68616&subCarrierSpacing=1.25&bandwidth=6

EXAMPLE 3:	This URL identifies a specific service identified by a serviceId in a specific area and with receiver frequency parameters.

mbms://901056.tmgi.3gpp.org&serviceArea=40201&frequency=68616&subCarrierSpacing=1.25&bandwidth=8&serviceId="television-service"

=====  CHANGE  =====

Annex F (informative):
User Service Description templates for use with ROM Service Announcement MBMS URLs

# F.1	Introduction

Assuming an MBMS-URL for ROM services as defined in clause 8.2.4 is provided, then the MBMS Client may create a Service Announcement file using the parameters from the URL and the SDP Template in clause F.2 and the User Service Description Template in clause F.3 by replacing the bold semantics.

# F.2	SDP template

v=0 o=3gpp 2890844526 2890842807 IN IP4 <see TS 26.346, clause C.17> t=0 0 a=mbms-mode:broadcast <tmgi in decimal> 1 a=FEC-declaration:0 encoding-id=1 a=source-filter: incl IN IP4 * <see TS 26.346, clause C.17> a=flute-tsi:0 m=application 12345 FLUTE/UDP 0 c=IN IP4 <see TS 26.346, clause C.17>/<see TS 26.346, clause C.19> a=FEC:0

# F.3	User Service Description template

<?xml version="1.0" encoding="UTF-8"?> <bundleDescription xsi:schemaLocation="urn:3GPP:metadata:2005:MBMS:userServiceDescription USD-schema-main.xsd" xmlns="urn:3GPP:metadata:2005:MBMS:userServiceDescription" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:r7="urn:3GPP:metadata:2007:MBMS:userServiceDescription" xmlns:r8="urn:3GPP:metadata:2008:MBMS:userServiceDescription" xmlns:r9="urn:3GPP:metadata:2009:MBMS:userServiceDescription" xmlns:r12="urn:3GPP:metadata:2013:MBMS:userServiceDescription" xmlns:r14="urn:3GPP:metadata:2017:MBMS:userServiceDescription" xmlns:r15="urn:3GPP:metadata:2019:MBMS:userServiceDescription" xmlns:r16="urn:3GPP:metadata:2020:MBMS:userServiceDescription" xmlns:sv="urn:3gpp:metadata:2009:MBMS:schemaVersion"> <userServiceDescription serviceId="rom-sa-bootstrap" r14:romService="true"> <deliveryMethod sessionDescriptionURI="http://www.example.com/3gpp/mbms/session1.sdp"> <sv:delimiter>0</sv:delimiter> <sv:delimiter>0</sv:delimiter> </deliveryMethod> <sv:delimiter>0</sv:delimiter> <r16:ROMSvcRfParams> <16:Frequency r16:subCarrierSpacing="value from URL subCarrierSpacing" r16:bandwidth="value from URL bandwidth">value of URL frequency</r16:Frequency> </r16:ROMSvcRfParams> <sv:delimiter>0</sv:delimiter> </userServiceDescription> <sv:schemaVersion>3</sv:schemaVersion> </bundleDescription>