FIRST CHANGE

# 1	Scope

The present document specifies the set of APIs and associated resource structures and data models pertaining to the collection and reporting of UE-related data (synonymously “UE data”) by the Data Collection Application Function (AF) in the 5G System for subsequent event exposure services offered to network consumer entities, as defined in 3GPP TS 23.501 [2], TS 23.502 [3], TS 23.288 [4], TS 29.517 [5], TS 29.510 [6] and TS 26.531 [7] .

NEXT CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[6]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[7]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[8]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[9]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[10]	"CORS (Cross-Origin Resource Sharing)" protocol as defined in the ‘Fetch’ standard of WHATWG: https://fetch.spec.whatwg.org/#cors-protocol.

[11]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[12]	Void.

[13]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[14]	3GPP TS 26.512: “5G Media Streaming (5GMS); Protocols”.

[15]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[16]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[17]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

NEXT CHANGE

### 4.2.3	Data collection and reporting provisioning

#### 4.2.3.1	General

An Application Service Provider, via its Provisioning AF, shall use the Ndcaf_DataReportingProvisioning service offered by the Data Collection AF to provide data collection and reporting provisioning information, as defined in clause 4.2 of TS 26.531 [7], to the Data Collection AF via reference point R1 in the form of Data Reporting Configuration resources. A given Data Reporting Configuration contains instructions and other information to be followed/used by the DataCollection AF in the exposure of UE event data to consumer entities, for a specified application. In addition, for that application, the Data Reporting Configuration contains information to be followed/used by data collection client(s) of the Data Collection AF in the collection, processing and reporting of UE data to the Data Collection AF for the associated application service and Event ID(s).

NOTE:	Exposure of UE data obtained from data collection clients by the Data Collection AF to the Application Service Provider or to other consumer entities is part of event subscription, management and publication functionality described in clause 4.2.8.

The provisioning process begins with the Provisioning AF using the procedures defined in clause 4.2.3.2 to create a Data Reporting Provisioning Session resource as an umbrella for subsequent Data Reporting Configuration resources.

The process then proceeds with the Provisioning AF using the procedures defined in clause 4.2.3.3 to provide the Data Collection AF with one or more Data Reporting Configuration resources. The event exposure related provisioning information contained in a given Data Reporting Configuration pertains to one application, identified by its External Application Identifier, and one type of exposed event, uniquely identified in the 5G System by its Event ID, as defined in clause 4.15.1 of TS 23.502 [3]. The UE data collection and reporting related information in that Data Reporting Configuration pertains to parameters and rules for use by data collection client(s) in the collection and reporting of UE data of the associated application to the Data Collection AF.

The Ndcaf_DataReportingProvisioning service and baseline provisioning information contained in the Data Reporting Configuration resource are further described in clauses 4.4 and 4.6.2, respectively, of TS 26.531 [7].

#### 4.2.3.2	Data Reporting Provisioning Session procedures

##### 4.2.3.2.1	General

Prior to provisioning of data collection and reporting, the Provisioning AF shall create a new Data Reporting Provisioning Session. The following CRUD operations are used to manage Data Reporting Provisioning Session resources. Additional details, including definition of the Data Reporting Provisioning Sessions API, are provided under clause 6.2.

##### 4.2.3.2.2	Create Data Reporting Provisioning Session

This procedure shall be used by the Provisioning AF to create a new Data Reporting Provisioning Session. The HTTP POST method shall be used for this purpose.

Editor’s Note: Describe key attributes of the Data Reporting Provisioning Session resource here, especially the access controls that realise the data exposure restrictions affecting all Data Reporting Configuration children of the Provisioning Session.

Upon successful creation, the Data Collection AF shall respond with a 201 (Created) response message that includes the resource identifier of the newly created Data Reporting Provisioning Session in the body of the reply and the URL of the resource, including its resource identifier, shall be returned as part of the HTTP Location header field.

##### 4.2.3.2.3	Retrieve Data Reporting Provisioning Session properties

This procedure is used by the Provisioning AF to obtain the properties of an existing Data Reporting Provisioning Session from the Data Collection AF. The HTTP GET method shall be used for this purpose.

##### 4.2.3.2.4	Update Data Reporting Provisioning Session properties

This procedure is used by the Provisioning AF to modify an existing Data Reporting Provisioning Session in the Data Collection AF. The HTTP PUT method shall be used for this purpose.

##### 4.2.3.2.5	Destroy Data Reporting Provisioning Session

This procedure is used by the Provisioning AF to destroy a Data Reporting Provisioning Session. The Data Collection AF shall use the HTTP DELETE method for this purpose.

As a side-effect of destroying a Data Reporting Provisioning Session, the Data Collection AF shall release any associated resources, purge any cached data, and delete all UE data reporting configurations associated with this Data Reporting Provisioning Session.

#### 4.2.3.3	Data Reporting Configuration procedures

##### 4.2.3.3.1	General

Upon the successful creation of a Data Reporting Provisioning Session, the Provisioning AF shall use the procedures defined in this clause to configure UE data collection and reporting functionality specific to an application in the Data Collection AF. This clause defines the basic procedures. Additional details, including definition of the Data Reporting Conifiguration Sessions API are provided under clause 6.3.

A given instance of a Data Reporting Configuration resource is identified by the dataReportingConfigurationType and dataReportingConfigurationId properties of the DataReportingConfiguration resource, The properties of that resource, as described in the following sub-clauses, pertain to UE data collection and reporting by different data collection clients to the Data Collection AF.

##### 4.2.3.3.2	Data Reporting Configuration types

The type of a Data Reporting Configuration resource is identified by the dataCollectionClientType property of the ProvisioningSession resource as specified in clause 6.2.3.

NOTE:	The dataCollectionClientType property corresponds to the Data collection client type parameter in table 4.6.2-1 of TS 26.531 [7].

Editor’s Note: Define a common enumeration DataCollectionClientType in clause 5.4.

##### 4.2.3.3.3	Create Data Reporting Configuration

This procedure is used by the Provisioning AF to create a Data Reporting Configuration resource within the scope of a particular Provisioning Session. The HTTP POST method shall be used for this purpose and the request message body may include a DataReportingConfiguration resource, as specified under clause 6.3.

Editor’s Note: Describe key attributes of the Data Reporting Configuration resource here.

Upon success, the Data Collection AF shall respond with a 201 (Created) response message and the resource URL for the newly-created Data Reporting Configuration resource shall be returned in the Location header field.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

This procedure may be performed multiple times to provision different Data Reporting Configurations in the scope of a particular Provisioning Session.

##### 4.2.3.3.4	Retrieve Data Reporting Configuration

This procedure is used by the Provisioning AF to obtain the properties of an existing Data Reporting Configuration resource from the Data Collection AF. The HTTP GET method shall be used for this purpose.

If successful, the Data Collection AF shall respond with a 200 (OK) and the requested DataReportingConfiguration resource shall be returned in the body of the HTTP response message.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

##### 4.2.3.3.5	Update Data Reporting Configuration

The update operation is invoked by the Provisioning AF to initially upload the representation of a Data Reporting Configuration resource created using the procedure in clause 4.2.3.3.3 or, in the case of an existing Data Reporting Configuration resource, to entirely replace or modify certain properties of that resource. All available properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the Data Collection AF shall respond with a 200 (OK) reflecting the successful update operation.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

##### 4.2.3.3.6	Destroy Data Reporting Configuration

This operation is used by the Provisioning AF to destroy a Data Reporting Configuration resource and to terminate the related UE data reporting procedure. The HTTP DELETE method shall be used for this purpose.

As a result, the Data Collection AF should release any associated resources, deliver any pending data to subscribed recipients, as appropriate to the Event ID in question, and delete any corresponding configurations.

If the procedure is successful, the Data Collection AF shall respond with a 200 (OK) response message.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

NEXT CHANGE

# 6	Ndcaf_DataReportingProvisioning service

## 6.1	General

This clause specifies the APIs associated with the Ndcaf_DataReportingProvisioning service for use by the Provisioning AF of an Application Service Provider to provision data collection and reporting in the Data Collection AF.

## 6.2	Data Reporting Provisioning Sessions API

### 6.2.1	Overview

This clause specifies the Data Reporting Provisioning Sessions API used by an Application Service Provider’s Provisioning AF to provision a data collection and reporting configuration in a Data Collection AF, as described in clause 4.2.3.2.

### 6.2.2	Resources

#### 6.2.2.1	Resource structure

Figure 6.2.2-1 depicts the URL base path for the Data Reporting Provisioning Sessions API that is accessible through the following URL base path:

{apiRoot}/v1/ndcaf_data-reporting-provisioning/sessions/sessionId

Figure 6.2.2.1-1: URL base path model of Data Reporting Provisioning Session related resources

Table 6.2.2-1 provides an overview of the resources and applicable HTTP methods of the Ndcaf_DataReportingProvisioningSessions API.

Table 6.2.2-1: Operations supported by the Data Reporting Provisioning Sessions API

| Service name | Operation name | Resource name | Resource path suffix | HTTP method | Description |
| --- | --- | --- | --- | --- | --- |
| Ndcaf_Data‌Reporting‌Provisioning | Create‌Provisioning‌Session | Data Reporting Provisioning Sessions | /sessions | POST | Used to create a new Data Reporting Provisioning Session resource at the Data Collection AF. If the operation succeeds, the URL of the created Data Reporting Provisioning Session resource shall be returned in the Location header of the response. |
|  | Retrieve‌Provisioning‌Session | Data Reporting Provisioning Session | /sessions/ {sessionId} | GET | Retrieves a Data Reporting Provisioning Session resource from the Data Collection AF. |
|  | Update‌Provisioning‌Session |  |  | PUT | Modifies an existing Data Reporting Provisioning Session resource. |
|  | Destroy‌Provisioning‌ Session |  |  | DELETE | Destroys a Data Reporting Provisioning Session resource at the Data Collection AF. |

#### 6.2.2.2	Data Reporting Provisioning Sessions collection resource

##### 6.2.2.2.1	Description

The Data Reporting Provisioning Sessions collection resource represents the set of all Data Reporting Provisioning Sessions at a given Data Collection AF (service) instance. This collection resource allows the Provisioning AF to create a new Data Reporting Provisioning Session resource at, and to receive configuration details for that session from, the Data Collection AF.

##### 6.2.2.2.2	Resource definition

Resource URL: {apiRoot}/v1/ndcaf_data-reporting-provisioning/sessions

The Data Reporting Provisioning Sessions collection resource shall support the resource URL variables as defined in table 6.2.2.2.2-1.

Table 6.2.2.2.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Fully-Qualified Doman Name of the Data Collection AF and path prefix. |

##### 6.2.2.2.3	Resource Standard Methods

###### 6.2.2.2.3.1	Ndcaf_DataReportingProvisioning_CreateSession operation using POST method

This method shall support the URL query parameters as specified in table 6.2.2.2.3.1-1.

Table 6.2.2.2.3.1-1: URL query parameters supported by the POST method on this resource

| Parameter | Data type | Cardinality | Description |
| --- | --- | --- | --- |

This method shall support the request data structure and header as specified in tables 6.2.2.2.3.1-2 and 6.2.2.2.3.1-3, and the response data structures and response codes and associated headers as specified in tables 6.2.2.2.3.1-4 and 6.2.2.2.3.1-5.

Table 6.2.2.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Cardinality | Description |
| --- | --- | --- |
| DataReporting‌Provisioning‌Session | 1 | Data supplied by the Provisioning AF to enable creation of a new Data Reporting Provisioning Session at the Data Collection AF. |

Table 6.2.2.2.3.1-3: Headers supported for POST requests on this resource

| HTTP request header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Authorization | string | 1 | For authentication of the Provisioning AF (see NOTE). |
| Origin | string | 0..1 | Identifies the origin of the requester. |
| NOTE: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the access token associated with “client credentials grant” as defined in RFC 6749 [X]). |  |  |  |

Table 6.2.2.2.3.1-4: Data structures supported by the POST response body on this resource

| Data type | Cardinality | Response codes | Description |
| --- | --- | --- | --- |
| DataReportingProvisioningSession | 1 | 201 Created | The creation of a Data Reporting Provisioning Session resource is confirmed and the current state of the resource is provided by the Data Collection AF in the response message body. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] also apply. |  |  |  |

Table 6.2.2.2.3.1-5: Headers supported by the 201 (Created) response code on this resource

| HTTP response header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Location | string | 1 | The URL of the newly created resource at the Data Collection AF, according to the structure specified in clause 6.2.2.1.{ |
| Access-Control-Allow-Origin | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. |
| Access-Control-Allow-Methods | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: POST, PUT, DELETE |
| Access-Control-Expose-Headers | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: Location |

NOTE:	Standard HTTP redirection using a 3xx response code with the Location header as well as Alt-Svc are allowed.

#### 6.2.2.3	Data Reporting Provisioning Session resource

##### 6.2.2.3.1	Description

The Data Reporting Provisioning Session resource represents a single provisioning session within the collection of Data Reporting Provisioning Sessions at a given Data Collection AF.

##### 6.2.2.3.2	Resource definition

Resource URL: {apiRoot}/v1/ndcaf_data-reporting-provisioning/sessions/sessionionId

This resource shall support the resource URI variables defined in table 6.2.2.3.2-1.

Table 6.2.2.3.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.2.2.2 |
| sessionId | string | Identifier of an individual session instance of the Data Reporting Provisioning Sessions collection. |

##### 6.2.2.3.3	Resource standard methods

###### 6.2.2.3.3.1	Ndcaf_DataReportingProvisioning_RetrieveSession operation using GET method

Editor’s Note: To be added.

###### 6.2.2.3.3.2	Ndcaf_DataReportingProvisioning_UpdateSession operation using PUT method

This method shall support the URL query parameters specified in table 6.2.2.3.3.2-1.

Table 6.2.2.3.3.2-1: URL query parameters supported by the PUT method on this resource

| Name | Data type | Cardinality | Description |
| --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.2.2.3.3.2-2 and the response data structures and response codes specified in table 6.2.2.3.3.2-4.

Table 6.2.2.3.3.2-2: Data structures supported by the PUT request body on this resource

| Data type | Cardinality | Description |
| --- | --- | --- |
| DataReporting‌Provisioning‌Session | 1 | Parameters to replace previously provided or updated configuration data by the Provisioning AF for a Data Reporting Provisioning Session resource. |

Table 6.2.2.3.3.2-3: Headers supported for PUT requests on this resource

| HTTP request header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Authorization | string | 1 | For authentication of the Provisioning AF (see NOTE). |
| Origin | string | 0..1 | Indication of the origin of the requester. |
| NOTE: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the access token associated with “client credentials grant” as defined in RFC 6749 [X]). |  |  |  |

Table 6.2.2.3.3.2-4: Data structures supported by the PUT response body on this resource

| Data type | Cardinality | Response codes | Description |
| --- | --- | --- | --- |
| DataReporting‌Provisioning‌Session | 1 | 200 OK | The Data Reporting Provisioning Session resource as successfully modified by configuration data provided by the Provisioning AF |
| ProblemDetails | 0..1 | 307 Temporary Redirect | Temporary redirection imposed by the Data Collection AF for a Data Reporting Provisioning Session modification request. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance (see NOTE 1). Applicable if the feature "ES3XX" (Extended Support of HTTP 307/308 redirection as defined in TS 29.502 [11]) is supported. |
| ProblemDetails | 0..1 | 308 Permanent Redirect | Permanent redirection imposed by the DC-AF for a Data Reporting Provisioning Session modification. The response shall include a Location header field containing an alternative URL of the resource located in another DC-AF (service) instance (see NOTE 1). |
| ProblemDetails | 0..1 | 404 Not Found | Indication that the Data Reporting Provisioning Session resource does not exist (see NOTE 2). |
| NOTE 1: Applicable if the feature "ES3XX" (Extended Support of HTTP 307/308 redirection as defined in TS 29.502 [11]) is supported. NOTE 2: Failure cases are described in sub-clause 6.2.4. NOTE 3: In addition to the above response codes, mandatory HTTP error status codes for the PUT method as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] also apply. |  |  |  |

Table 6.2.2.3.3.2-5: Headers supported by the 200 (OK) response code on this resource

| HTTP response header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. |
| Access-Control-Allow-Methods | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: Location. |

Table 6.2.2.3.3.2-6: Headers supported by the 307 (Temporary Redirect) and 308 (Permanent Redirect) response codes on this resource

| HTTP response header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Location | string | 1 | An alternative URL of the resource located in another Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. |
| Access-Control-Allow-Methods | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: POST, PUT, DELETE |
| Access-Control-Expose-Headers | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: Location |

###### 6.2.2.3.3.1	Ndcaf_DataReportingProvisioning_DestroySession operation using DELETE method

This method shall support the URL query parameters specified in table 6.2.2.3.3.1-1.

Table 6.2.2.3.3.1-1: URL query parameters supported by the DELETE method on this resource

| Name | Data type | Cardinality | Description |
| --- | --- | --- | --- |

This method shall support the request data structures as specified in table 6.2.2.3.3.1-2 and the response data structures and response codes as specified in table 6.2.2.3.3.1-4.

Table 7.2.2.3.3.1-2: Data structures supported by the DELETE request body on this resource

| Data type | Cardinality | Description |
| --- | --- | --- |

Table 6.2.2.3.3.1-3: Headers supported for DELETE requests on this resource

| HTTP request header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Authorization | string | 1 | For authentication of the Provisioning AF (see NOTE). |
| Origin | string | 0..1 | Identifies the origin of the requester. (NOTE 2) |
| NOTE: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the access token associated with “client credentials grant” as defined in RFC 6749 [X]). |  |  |  |

Table 6.2.2.3.3.1-4: Data structures supported by the DELETE response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Data Reporting Provisioning Session resource matching the SessionId was destroyed at the Data Collection AF. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection during Data Reporting Session destruction. The response shall include a Location header field containing an alternative URL of the resource located in another DC-AF (service) instance (see NOTE1). |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection during Data Reporting Session destruction. The response shall include a Location header field containing an alternative URL of the resource located in another DC-AF (service) instance (see NOTE 1) |
| ProblemDetails | O | 0..1 | 404 Not Found | The Data Reporting Session resource does not exist (see NOTE 2). |
| NOTE 1: Applicable if the feature "ES3XX" (Extended Support of HTTP 307/308 redirection as defined in TS 29.502 [11]) is supported. NOTE 2: Failure cases are described in sub-clause 6.2.4. NOTE 3: In addition to the above response codes, mandatory HTTP error status codes for the PUT method as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] will also apply. |  |  |  |  |

Table 6.2.2.3.3.1-5: Headers supported by the 204 response code on this resource

| HTTP response header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. |
| Access-Control-Allow-Methods | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | 0..1 | Part of CORS [10]. Present if the request included the Origin header. Valid values: Location. |

Table 6.2.2.3.3.1-6 Headers supported by the 307 and 308 response codes on this resource

| HTTP response header | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| Location | string | 1 | An alternative URL of the resource located in another DC-AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | 0..1 | Part of CORS [10].Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location. |

### 6.2.3	Data model

#### 6.2.3.1	General

Table 6.2.3-1 specifies the data type used for the Ndcaf_DataReportingProvisioningSessions API.

Table 6.2.3-1: Data type associated with Ndcaf_DataReportingProvisioningSession API

| Data type | Clause defined | Description |
| --- | --- | --- |
| DataReporting‌Provisioning‌Session | 6.2.3.2.1 | Provisioning data on data collection and reporting for the Data Collection AF. |

Table 6.2.3-2 specifies data types re-used from other specifications by the Ndcaf_DataReportingProvisioningSessions API, including a reference to the respective specifications.

Table 6.2.3-2: Externally defined data types used by Ndcaf_DataReportingProvisioningSession API

| Data type | Comments | Reference |
| --- | --- | --- |
| ApplicationId | Identifier of the application for UE data collection and reporting by data collection clients, and which is associated with the event exposed by the Data Collection AF. | 3GPP TS 29.571 [13] |

#### 6.2.3.2	Structured data types

##### 6.2.3.2.1	DataReportingProvisioningSession type

The data model for the Data Reporting Provisioning Session resource with which the Ndcaf_DataReportingProvisioningSessions API is associated is specified in table 6.2.3.1.1-1 below.

Table 6.2.3.2.1-1: Definition of Data Reporting ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 0..1 | C: RO R: RO U: – | A unique identifier for this Data Reporting Provisioning Session assigned by the Data Collection AF. |
| aspId | String | 1..1 | C: RW R: RO U: – | The identity of the Application Service Provider responsible for this Data Reporting Provisioning Session, the format of which is specified in clause 5.6.2.3 of TS 29.514 [34]. |
| externalApplicationId | String | 1..1 | C: RW R: RO U: RW | The identifier to be used in reports sent to the Data Collection AF by data collection clients and in events exposed to the Application Service Provider |
| internalApplicationId | String | 1..1 | C: RO R: RO U: – | The identifier to be used by event consumers (including the NWDAF and the Event Consumer AF) when subscribing to events in the Data Collection AF. |
| eventId | Integer | 1..1 | C: RO R: RO U: – | The identifier of an AF event for exposure to subscribing event consumers as a result of the provisioning. |
| data‌Collection‌Client‌Type | Data‌Collection‌Client‌Type | 1..1 | C: RO R: RO U: – | The type of data collection client that will submit data reports to the Data Collection AF. |
| validTargets | ?? | 1..1 | C: RW R: RO U: RW | A parameter to control whether event consumers are permitted to filter events by External UE identifier or External Group Identifier when subscribing, instead of receiving events relating to all UEs. |
| reportedParameters | Array(Object) | 1..* | C: RW R: RO U: RW | The subset of domain-specific parameters associated with the specified Event ID to be reported to the Data Collection AF (subject to user consent). |
| dataExposureRestrictions | Array(Object) | 1..* | C: RW R: RO U: RW | A set of defined access restriction profiles that apply to event data exposure to consumer entities. |

#### 6.2.3.2	Enumerated data types

##### 6.2.3.2.1	DataCollectionClientType enumeration

The data model for the DataCollectionClientType enumeration which indicates the type of data collection client is specified in Table 6.2.3.2.1-1 below:

Table 6.2.3.2.1-1: Definition of DataCollectionClientType enumeration

| Enumeration value | Description |
| --- | --- |
| DIRECT | Direct Data Collection Client, as specified in TS 26.531 [7]. |
| INDIRECT | Indirect Data Collection Client, as specified in TS 26.531 [7]. |
| APPLICATION_SERVER | Application Server performing the role of data collection client, as specified in TS 26.531 [7]. |

### 6.2.4	Mediation by NEF

## 6.3	Data Reporting Configurations API

### 6.3.1	Overview

### 6.3.2	Resource structure

### 6.3.3	Data model

### 6.3.4	Mediation by NEF

END OF CHANGES