3GPP TSG SA WG4#117-e meeting	S4-220028

14th– 23rd February 2022

Agenda item: 	8.11

Source: 	Qualcomm Incorporated

Title: 	[FS_NPN4AVProd] Proposed Communication with SA2

Document for	Agreement

# Introduction

We have reviewed TR 26.805 after the submission to SA plenary. Our SA2 colleagues indicate that the descriptions related to QoS e.g. clause 6.3.3, 6.3.4 and 6.3.5 are very rudimentary. For example, they ignore support for dynamic PCC, related NEF APIs for QoS and assume pre-established QoS flows with different priorities based on static rules. They also ignore any work SA2 has done for Time Sensitve Networks (rel.16/17) and TSC for IP flows (rel.17) as option to be used to provide QoS for this kind of applications. For instance the work of rel.17 for IP flows was done to meet QoS requirements from Audio Video Production (AVProd) WI of SA1.

Generally, we believe we need to add references to SA2 specifications for the above aspects. We considered that we provide this feedback and input to SA4 directly, but the indirect communication may loose some of the essence, but we believe that this should be reviewed by SA2 collectively.  We also believe that coordination on potential normative work should be considered.

Based on this we consider sending an LS to SA2 asking for review and comments on TR 26.805 from SA4#117e is most appropriate, along with an invitation for a joint telco. A text proposal for an LS is provided in clause 2.

Qualcomm offers to prepare input in SA2 meeting in April (SA2#150) to properly respond to the LS.

# Proposed LS Text

3GPP SA4 is in the completion phase of the Feasibility Study on Media Production over 5G NPN. The current findings are documented in the latest version TR 26.805v1.1.0 and is driven by initial use cases and requirements in TR 22.827 and TS 22.263 for for Video, Imaging and Audio for Professional Applications. TR 26.805 addresses extended use cases and reviews existing service layer solutions including protocols, codecs, as well as orchestration and control solutions. Different media production scenarios are reviewed and 7 key issues are documented. Several of the issues relate to QoS and advanced timing and synchronization aspects, very much in the expertise of SA2.

Based on this, SA4 invites SA2 to review the current TR 26.805 and provide feedback on QoS and other related matters. If SA2 experts agree, we also offer to invite SA2 delegate to dedicated MBS Adhoc telco after your SA2#150 and during or after our SA4#118-e meeting.

Actions:

- TO SA2: SA4 kindly asks SA2 for feedback and comments on the latest version of TR 26.805.

# Proposal

It is proposed to

- Send an LS to SA2 from SA4#117e based on the proposed text in clause 2.

- Coordinate a telco with SA2 to review the LS either during SA4#118e or afterwards, for example on April 28, 2022.

- Await completion of FS_NPN4AVProd until feedback from SA#2 is received