3GPP TSG SA WG4#117e	S4-220029

E-meeting, 14th – 23rd February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.805 | CR | pseudo | rev |  | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_NPN4AVProd] mmWAVE for Media Production |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NPN4AVProd |  |  |  |  |  | Date: |  |  | 07/02/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.5 (new), 6.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[4]	3GPP TS 22.827: "Study on Audio-Visual Service Production".

[5]	M.P. Sharabayko, M.A. Sharabayko, J. Dube, JS. Kim, JW. Kim: "The SRT Protocol", draft-sharabayko-mops-srt-01

[6]	VSF: "Reliable Internet Stream Transport (RIST) Activity Group",

[7]	VSF TR 06-1:2020 "Reliable Internet Stream Transport (RIST) Protocol Specification – Simple Profile", https://vsf.tv/download/technical_recommendations/VSF_TR-06-1_2020_06_25.pdf

[8]	VSF TR 06-2, "Reliable Internet Stream Transport (RIST) Protocol Specification – Main Profile", https://www.vsf.tv/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

[9]	NewTek: "NDI Encoding/Decoding", https://support.newtek.com/hc/en-us/articles/218109667-NDI-Encoding-Decoding

[10]	NewTek: "NDI Network Bandwidth, https://support.newtek.com/hc/en-us/articles/217662708-NDI-Network-Bandwidth

[11]	David Aleksandersen: "What is NDI® (Network Device Interface)?", https://newsandviews.dataton.com/what-is-ndi-network-device-interface

[12]	Kieran Kunhya and Ciro Noronha: "RIST and SRT: What’s the difference?", https://www.tvbeurope.com/ip-migration/rist-and-srt-whats-the-difference

[13]	Tofik Sonono: "Interoperable Retransmission Protocols with Low Latency and Constrained Delay: A Performance Evaluation of RIST and SRT", Masters Thesis, KTH Stockholm, 2019,

[14]	EBU: "Minimum User Requirements to Build and Manage an IP-Based Media Facility", 15 July 2020, https://tech.ebu.ch/files/live/sites/tech/files/shared/tech/tech3371.pdf.

[15]	AMWA: "NMOS Overview", https://www.amwa.tv/nmos-overview.

[16]	EBU: "The Technology Pyramid For Media Nodes", .

[17]	EBU: "Technology Pyramid Media Node Maturity Checklist", September 2021, https://tech.ebu.ch/publications/technology-pyramid-media-node-maturity-checklist?rec=1.

[18]	AMWA: "NMOS Technical Overview", https://specs.amwa.tv/nmos/branches/main/docs/2.0._Technical_Overview.html.

[19]	AMWA: "Networked Media Systems – the Big Picture",
https://static.amwa.tv/networked-media-systems-big-picture-2021-03-05.pdf.

[20]	AMWA: "NMOS specification repository", https://specs.amwa.tv/nmos.

[21]	SMPTE ST 2110: "Professional Media over Managed IP".

[22]	IEEE 1588-2008: "Precision Time Protocol".

[23]	SMPTE ST 2022-1:2007: "Forward Error Correction for Real-Time Video/Audio Transport Over IP Networks".

[24]	SMPTE ST 2022-6:2012: "Transport of High Bit Rate Media Signals over IP Networks (HBRMT)",

[25]	SMPTE ST 2022-7:2019: "Seamless Protection Switching of RTP Datagrams".

[26]	SMPTE ST 2059-2:2015: "SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications".

[27]	SMPTE ST 2110-10:2017: "Professional Media Over Managed IP Networks: System Timing and Definitions".

[28]	SMPTE ST 2110-20:2017: "Professional Media Over Managed IP Networks: Uncompressed Active Video".

[29]	SMPTE ST 2110-22:2019: "Professional Media Over Managed IP Networks: Constant Bit-Rate Compressed Video".

[30]	SMPTE ST 2110-30:2017: "Professional Media Over Managed IP Networks: PCM Digital Audio".

[31]	SMPTE ST 2110-31:2018: "Professional Media Over Managed IP Networks: AES3 Transparent Transport".

[32]	IETF RFC 4585: "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[33]	IETF RFC 8086: "GRE-in-UDP Encapsulation".

[34]	Ember+ control protocol,

[35]	SMPTE ST 259:2008: "For Television — SDTV Digital Signal/Data — Serial Digital Interface".

[36]	SMPTE ST 292-1:2012: "1.5 Gb/s Signal/Data Serial Interface".

[37]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[38]	Wikipedia: "MADI", last modified 19th April 2021, https://en.wikipedia.org/wiki/MADI

[39]	Wikipedia: "Time-Sensitive Networking", last modified 23rd June 2021, https://en.wikipedia.org/wiki/Time-Sensitive_Networking

[40]	AES67 / SMPTE ST 2110: "COMMONALITIES AND CONSTRAINTS", https://aimsalliance.org/wp-content/uploads/2019/04/AES67-SMPTE-ST-2110-Commonalities-and-Constraints-Updated-April-2019.pdf

[41]	IETF RFC 5104: "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[42]	IETF RFC 4585: "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[43]	ISO/IEC 13818-1: "Information technology — Generic coding of moving pictures and associated audio information — Part 1: Systems".

[44]	IETF RFC 3550|STD 64: "RTP: A Transport Protocol for Real-Time Applications".

[45]	IETF RFC 8086: "GRE-in-UDP Encapsulation".

[46]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[47]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)".

[48]	OASIS: "MQTT Version 5.0", 7th March 2019, https://docs.oasis-open.org/mqtt/mqtt/v5.0/mqtt-v5.0.html

[49]	IETF RFC 6416: "RTP Payload Format for MPEG-4 Audio/Visual Streams".

[50]	IETF RFC 9134: "RTP Payload Format for ISO/IEC 21122 (JPEG XS)".

[51] 	GSA, mmWave Bands 24.25 GHz: May 2021, https://gsacom.com/paper/mmwave-bands-24-25-ghz-may-2021-executive-summary/

[52]	Li Nian, “The Next Journey for 5G: The Standardization and Application of mmWAVE,”, https://www.gsma.com/greater-china/wp-content/uploads/2020/09/%E6%AF%AB%E7%B1%B3%E6%B3%A2%E6%A0%87%E5%87%86%E5%8C%96%E5%92%8C%E8%AF%95%E9%AA%8C%E8%BF%9B%E5%B1%95_%E4%B8%AD%E5%9B%BD%E7%A7%BB%E5%8A%A8_%E6%9D%8E%E7%94%B7-1.pdf

[53]	GSMA Intelligence, “The economics of mmWave 5G”, January 2021, https://data.gsmaintelligence.com/research/research/research-2021/the-economics-of-mmwave-5g

[54]	3GPP TR 38.900, “Study on channel model for frequency spectrum above 6 GHz”

[55]	Yiqing Cao: "Live 8K production using 5G mmWave", accessible here: https://www.5g-mag.com/post/follow-up-workshop-media-production-over-5g-npn-deep-dive-into-protocols

=====  CHANGE  =====

## 5.5	Low-Latency Production with 5G mmWAVE

mmWAVE is also known as Frequency Range 2 defined in RAN specifications, which covers the spectrum above 24GHz. At WRC-2019 in November, several new frequency ranges were identified for IMT, to be used by IMT-2020 (5G). These encompassed many of the existing 3GPP-bands plus some new spectrum ranges:

- 	24.25–27.5 GHz

-	37–43.5 GHz

-	45.5–47 GHz

-	47.2–48.2 GHz

-	66–71 GHz.

Since Release 15, 5G mmWAVE technology, as part of 5G NR specifications, is well defined in 3GPP. It’s allowing users to access the full potential of 5G by utilizing untapped frequency bands above 24 GHz. This abundant spectrum can deliver the fastest available speeds, extreme capacity and low latency.

3GPP has done several studies on the channel model for frequency spectrum above 6 GHz, e.g. in TR 38.900 [54]. According to [52], extreme higher propagation loss and penetration loss in mmWave spectrum is expected, and the frequency is sensitive to blockage, e.g. foliage, human body. However, again according to [52], performance tests for mmWAVE provide extraordinary KPIs:

- 	14.7/3 Gbps cell peak throughout (DL/UL) using an 800 MHz spectrum

-	One-way user plane latency between 1 to 1.5ms

-	Farthest access distance: 2.6 km in line-of-sight with few small trees.

According to [55], commercial 5G modems support mmWAVE as well dual connectivity and upload speeds of 2.2 Gbps can be achieved in FR2 400 MHz (on n261) and FR1 100 MHz (n77). According to [54], mmWave bands can accommodate more capacity and bandwidth than any other band. And since spectrum in these bands is abundant, mmWave spectrum is ideally placed to deliver high speeds, low latency and high capacity, all at the same time. The short wavelength of mmWave allows for very small antennas, which helps with beam forming for enhanced coverage and spectral efficiency. Promoted by the whole industry, 5G mmWAVE commercialization grows rapidly. Until May 2021, GSA’s report [51] indicated that twenty-eight operators in 16 countries/territories are known to be already deploying 5G networks using mmWave spectrum on 24GHz.

Based on this analysis, mmWAVE is an attractive technology for production scenarios that typically rely on fully wired data rates. In particular, interactive video production scenarios require extreme low latency, as they allow video directors controlling cameras in almost realtime video. Typically, light weight compression codecs (e.g. JPEG-XS [50]) are used. In some field test, JPEG-XS could achieve less than 3ms camera-to-screen latency with PTP time synchronisation. Even without PTP time reference, it only requires 1 frame (25ms for 50 FPS) buffered for timing and the camera-to-screen latency is less than 28ms.

To achieve “Master Copy” quality, the preferred compression rate is between 1/8 – 1/12 which requires 1Gbps – 1.5Gbps transmission capacity for UHD video. Realistically, according to [55], for UHD at least 150 MBs are needed and for 8K more than 800 Mbps are needed. Due to large payload, light weight compression is transmitted via SDI cable and Ethernet before 5G commercialization. 5G mmWAVE is capable to provide several Giga bits-per-seconds data transmission with large bandwidth, and this could be leveraged to replace the Ethernet to transmit light weight compressed video signals. An example setup for mobile tele-studio is shown in Figure 5.5-1.

Figure 5.5-1 Mobile telestudio scenario with 5G mmWAVE

=====  CHANGE  =====

## 6.9	Key Issue #8: Usage of mmWAVE for Low-Latency Production

As introduced in clause 5.5, a lightweight compression codec is typically used to achieve Low-Latency Production and is currently transmitted via SDI cable and Ethernet. 5G mmWAVE is capable to provide the sufficient large data rate, but the wireless channel fluctuation may challenge service quality in particular mmWAVE spectrum is prone and sensitive to propagation & penetration loss as well as to blockage by objects and human bodies. To better utilize the wireless channel reflection and scattering, 3GPP defines advanced beam management as well as multi-carrier operation [52].

Nevertheless, to operate production scenarios at several 100 Mbit/s on top of mmWAVE, content delivery protocols and oodecs are needed that can compensate the mmWAVE channel characteristics, support high reliability and achieve the high bitrate as shown in Figure 6.9-1.

Figure 6.9-1 Protocol architecture – Mobile Tele-studio

At least the following techniques should be considered for mmWAVE based content delivery protocols:

- Adaptive bitrate encoding. Dynamically adjusting bitrate encoding could change the quality of encoded bit quality according to the channel variantion.

- Packet reordering. Wireless channels change rapidly and mobile system usual employs HARQ at physically layer to retransmit the missing data blocks. The packets sent earlier may arrive after the packets sent late due to different retransmission times. This might lead to wrong orders of packets and further errors at decoder. The decoder could buffer the received packets and reorder them in case of missing original orders.

- Error correction. FEC, ARQ, and Frame repairing are usually used to compensate the transmission errors. FEC transmits additional redundant coded bits and would require extra channel bandwidth and memories for error corrections. ARQ would introduce latency by retransmission which is not suitable for Low-Latency Production. Frame repairing is to repair the error packets by interplorating the neighboring correct packets, which doesn’t require additional bandwidth or latency. Among these three techniques, if the errors happen occasionally, Frame repairing can maintain the similar bit rate and latency performance as Ethernet.