3GPP TSG-SA WG4 Meeting #118-e	S4-220454

# E-Meeting, 6th – 14th April 2022

# 3GPP TSG-RAN WG3 Meeting #115-e	R3-222890

# Online, Feb 21 – Mar 3 2022

Title:	LS on RAN3 agreements for NR QoE

Response to:	R3-221672/R2-2202026

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	RAN2, SA4, SA5

Cc:	-

Contact Person:

Name: 		Pei Li

E-mail Address:	lip104@chinaunicom.cn

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 agreements in #115e meeting which impact may incur RAN2/SA4/SA5 impact:

Agreement about the assumptions in R2-2202026 (LS in from RAN2):

- RAN3 approves the RAN2 Assumptions 1a, 2a, 2b, 3, 4a, 4b, 5 and 6 in the LS R2-2202026, with the clarification on Assumption 5 that the largest value in the range, i.e., 30 seconds, should be used for all values greater than or equal to 30 seconds.

NR QoE Intra-System Intra-RAT Mobility

- RAN3 suggests that both S-based and M-based XML file could be included in RRC container, and M-based XML file will ONLY be included in RRC container, during mobility process.

- An explicit NGAP/XnAP “measurement status” IE is introduced, the code point is “ongoing”.

RAN-Visible QoE

- RAN visible QoE configuration can be transferred from the source to target node upon mobility and during context retrieval and as follows:

  - During Xn-based handover preparation: inside the RRC Context IE in the HANDOVER REQUEST message

  - During Xn-based UE context retrieval: inside the RRC Context IE (which in turn is included in the UE Context Information – Retrieve UE Context Response IE) in the RETRIEVE UE CONTEXT RESPONSE message

  - During NG-based handover preparation: inside the RRC Container IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE in the HANDOVER REQUIRED and HANDOVER REQUEST messages.

  - RAN3 understands the RRC Context IE and the RRC Container IE are referring to HandoverPreparationInformation.

- There is no need to pause/resume RVQoE reporting during/post RAN overload (pause/resume flag of regular QoE doesn’t impact RVQoE reporting)

- There is no need to support additional reporting periodicities for RAN visible QoE in Rel-17 than what was agreed in R3#114bis-e (i.e., no need to support the following reporting periodicities for RVQoE - ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, min60)

- If the reporting periodicity of RVQoE is not explicitly indicated in the RVQoE configuration, RVQoE reports can be sent together with the legacy QoE reports.

Alignment of Radio-Related Measurement and QoE Measurements

- When NG-RAN activates the associated MDT configuration is up to implementation, e.g., upon/after receiving the QoE measurement session start indication from the UE.

- NG-RAN deactivates the MDT measurement when it receives the deactivate request from OAM, which has no impact on legacy MDT mechanism.

RAN3 agreements in previous meetings which may impact SA5:

- The OAM sends a list of the available RAN visible QoE metrics to the RAN node, outside the legacy QoE configuration container.

- In case of aligned MDT/QMC (s-based activation), OAM includes Trace Reference and Trace Recording Session Reference of the MDT configuration in the QMC configuration sent to NG-RAN.

- In case of aligned MDT/QMC (m-based activation), OAM includes Trace Reference of the MDT configuration in the QMC configuration sent to NG-RAN.

- In case of aligned MDT/QMC, NG-RAN includes Trace Reference and Trace Recording Session Reference in the QoE report sent to MCE.

- To enable time alignment between an already ongoing Immediate MDT and a QoE measurement started later, the start time and end time of the QoE measurement, in addition to the Trace Reference and Trace Recording Session ID, needs to be added to the QoE measurement report at the NG-RAN node.

- NG-RAN can add a coarse QoE measurement session start/end timestamp autonomously in the QoE report sent to MCE based on QoE measurement session start/end indication from UE. If accurate timestamp information is desired, startTime / stopTime already included by UE in the QoE report can be used for correlating MDT-QoE at MCE.

2. Actions:

To RAN2, SA4 and SA5 group.

ACTION: 	RAN3 respectfully asks RAN2, SA4 and SA5 to consider RAN3 agreements and response to RAN3 if necessary. RAN3 kindly ask RAN2, SA4 and SA5 to update the specifications accordingly.

3. Date of Next TSG-RAN WG3 Meetings:

# RAN3#116-e                                                         9th - 20th May 2022                                  Online

# RAN3#117                                                         22th - 26th Aug 2022                                  Toulouse, FR