3GPP TSG-WG SA4 Meeting #118E e-meeting  	S4-220424

Elbonia, April 6th – 14th, 2022

# Title:	[DRAFT] LS Reply on QoS support with PDU Set granularity

# Response to:	LS (S2-2201803/S4-220XXX)

# Release:	Rel-18

# Work Item:	FS_XRM

Source:	SA4

To:	SA2

Cc:	RAN1, RAN2, RAN3

Contact Person:

Name:

Tel. Number:	Qi Pan

E-mail Address:	panqi8@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

3GPP TSG SA WG4 (SA4) would like to thank 3GPP TSG SA WG2 (SA2) on the LS on QoS support with PDU Set granularity.

For your questions, SA4 would like to provide the answers as following.

- Any feedback/guidance on the definition of PDU Sets and video slices in the attached document S2-2201851.

Answer#1: For definition of PDU Set, SA4 understands the PDU Sets represents the set of packets transmitted within the 5G system and defined to facilitate SA2 study on QoS enhancement. It is mainly a SA2 concept and SA4 is wondering, why the number of “information unit” (like NAL Units) is restricted to one. An application should be allowed to place multiple information units within one PDU Set, such as Parameter Sets (like Sequence or Picture Parameter Sets) together with an IDR frame.

About the video slice, there exists no absolute definition of it but each video coding specification has a specific definition such as in the H.264/AVC and H.265/HEVC specs. However, it is not necessary to understand the media codec specific details of a video slices from SA2 perspective. SA2 should not define video slices as it is not a network concept. In addition, the definition in itself is misleading.

- Any input on traffic characteristics of typical media services that rely on e.g. PDU Sets, as necessary.

Answer#2:  It is very difficult to identify a common “traffic characteristic”, since it often depends on the application choices. In particular individualized video services such as XR, Split-Rendering or Cloud Gaming do not use the traditional coding structure with a fixed IDR-Frame interval. Such structures are typically only used, when multiple receivers consume the same video.

Further, the traffic characteristic such as burstiness of the traffic can be influenced by network element such as traffic shapers. It is expected, that the XR Application Servers are deployed in a different domain and traffic conditioning may be applied at domain boundaries. <Maybe some hints to TR 26.926>

- Clarify what, if any, dependency there is between the IP packets that make up PDU Sets, e.g. a frame/”slice”.

Answer#3: As stated in TR 26.926, when one IP Packet of a PDU set is missing, the potentially the entire PDU Set is discarded. However, modern error concealment methods may still recover parts of the content. From the transmission requirement perspective, there is dependency between the IP packets carrying a video slice/frame	.

- Clarify what, if any, dependency there is between PDU Sets that may carry, e.g. different frames/”slices”.

Answer#4: In the video decoding phase, some frames/slices need to refer to other frames/slices based on the video coding configuration. From network transmission perspective, the PDU Sets carrying different frames/slices with reference relationship shall have dependency. As consequence, a PDU Set may depend on previously received PDU Sets.

- Provide feedback about the detailed traffic characteristics of XR media services in “Annex X (informative): Traffic characteristics of XR and media services” in the attached document S2-2201807.

Answer#5: SA4 has reviewed the Annex and has some comments on the description about the periodicity, jitter, frame importance, PDU Set granularity and traffic correlation. SA4 will keep SA2 informed if any issue identified in the future SA4 study. Findings will be documented in TR 26.926.

2. Actions:

To SA2 group.

ACTION: 	SA4 asks SA2 group to take the above reply into account and provide feedback if any.

3. Date of Next TSG SA WG4 Meetings:

SA4#119-e	11 – 20 May 2022		E-Meeting

SA4#120	22 – 26 August 2022		Malaga