Source:	NTT

Title:	Correction for FS_eiRTCW Permanent Document

Version: 	0.1.1

Document for:	Agreement

Agenda Item:	10.6

In the last SA4 meeting in April, the protocol stack for iRTCW was proposed in S4-220377, which had nothing to do with WebRTC IMS access specified in TS 24.371. On the other hand, the initial permanent document for FS_eiRTCW assumed that the iRTCW uses TS 24.371.

This contribution proposes the correction to get rid of the implication that iRTCW uses WebRTC IMS access.

- Scope

The present document extends immersive Real-time Communication for WebRTC (iRTCW) and introduces a new concept called native WebRTC signalling.

In 3GPP, the use of WebRTC technology has been investigated since Rel-12 (around 2014) and the specifications are available in TS 24.371 and TS 26.114. TS 24.371 and TS 26.114  enable WebRTC clients to access 5G systems. The C-plane signalling of those applications are SIP-like or SIP-aware over WebSocket considering the easy support by IMS core network. Although SIP satisfies almost all conversational applications, it is somewhat over-engineered or too strict to extend. Another method which is flexible, extensible, and can be optimized for new XR conversational applications, therefore, should be investigated. These requirements remind us of the original design principle of WebRTC. WebRTC, by its inherent characteristics, does not regulate C-plane signalling and allow a wide range of C-plane signalling. This study looks over this design principle again and investigates a new SIP-decoupled C-plane signalling, called native WebRTC.

Regarding the level of signalling details, TS 24.371 does not specify enough details of C-plane signalling itself. Each service provider (e.g., operator) develops its own application by following the guidelines in TS 24.371. Its subscriber downloads the application and connects to the service and other subscribers only within the same service. Detailed C-plane signalling is left open to each operator’s design. In contrast, this study tries to identify a new C-plane signalling in detail (as an interface specification) to the extent that client implementations based on it have enough interoperability. This realizes connectivity to any operators or roaming services for new XR real-time communications. Operators can provide the interface common to them according to well-defined C-plane signalling specifications. Clients can connect to any operators via the interface (see Figure1).

NOTE: TS 24.371, Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3; Protocol specification.

TS 26.114, IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction

Figure 1. Two approaches for defining specifications and their application connectivity

2	Motivations for Native WebRTC Signalling

2.1 	C-plane Signalling comparison

The C-plane signalling can be expressed as follows. Now, there are roughly four possible methods, classified in terms of their protocol stacks (see Figure 2).

Figure 2. Comparison of protocol stacks

The first method is MTSI-based, using SIP and SDP. General C-plane signalling requirements for conversational services can be covered by SIP. Interoperability is fine with the existing 5G core network. It is to be treated in IMS-based AR Conversational Services (IBACS).

The second is the method specified in TS 24.371. It enables the WebRTC clients to communicate over an IMS-based core network; only the interfaces for downloading dedicated applications and the signalling path using WebSocket are specified for C-plane signalling. Ordinary implementations adopt SIP-like protocols over WebSocket. In most cases, it is partially SIP-compliant or tightly coupled with SIP to adapt WebRTC clients in 5G systems.

The third method is an alternative to the second type that uses SIP-like protocol over WebSocket. The third method uses another signalling protocol over WebSocket, but SIP-decoupled approaches are investigated. It can be more lightweight, omitting features that is not used in XR conversational. Some constraints on SDP are necessary for interoperability. Non-browser based implementations are also in the scope. This method is the main subject of this study, FS_eiRTCW.

The other is a general WebRTC protocol stack that is not specified and left open to the users (i.e., service providers). C-plane may be SIP, XMPP, http, etc. Variants of SDP seem to be used.

Editor’s Note: 
- The reason why WebRTC signalling is necessary
- Comparison interworking between WebRTC signalling and existing SIP

3	Use Cases

Editor’s Note: 
- Use cases and communication methods (P2P, SFU, MCU)

4	Gap Analysis for WebRTC Architecture

Editor’s Note: 
Analyze gaps and identify required enhancements of terminal device and network architectures including additional functional entities (e.g., WebRTC Signalling Server, ICE-STUN Server, IMS Interworking Gateway, NNI Gateway).

- Stage2 work requirements
- Necessary functional blocks
- Architectural comparison (details are in annex)

5	Requirements for C-Plane Signalling

Editor’s Note: 
Identify signalling protocol details (e.g., based on JSON) for the common WebRTC-based immersive RTC session management.
Identify information elements in the C/U-Plane signal (including NNI) to enhance connectivity of media sessions with carrier assistance for WebRTC-based applications (including OTT applications).
Identify the minimal functional capabilities needed to support the enhancements identified in above.

- Stage3 requirements
- C-plane signalling requirements
- Signalling methods comparison (details are in annex)

6	Requirements for U-plane Signalling

7	Interworking with IMS Network

8	Tethered Cases

Editor’s Note: SmarTAR-related clause;
Identify enhancements for E2E QoS realizations over 5G systems for communications between MNOs and WebRTC clients operating over non-5G links (e.g., Wi-Fi) using WebRTC-based transport.  This also includes communication between WebRTC clients operating on tethering/tethered devices.

9	Security Considerations

Editor’s Note: 
Considerations that the third-party access to the operator network need to be controlled with SLAs and with secure access to protect the underlying network resources.
- Rate limiting
- Abuse protection
- Security measures

10	Related Groups Considerations

Editor’s Note: 
Identify collaboration formation with other WGs in 3GPP and SDOs including IETF and W3C.

11	Conclusions and Recommendations

Annex <A>:
Architectural WebRTC Entity Examples

Editor’s Note: 
Architectural example of integration of WebRTC with 5G network

Annex <B>:
Protocol Stack Examples

Editor’s Note: 
Definite example of C-plane protocol stack
Reference of U-plane (other TS/TRs) and supplemental explanation

Annex <C>:
WebRTC Signalling Protocol Examples

Editor’s Note: 
Expected signalling regulation examples (Async API)

Annex <D>:
WebRTC Signalling Flow Examples

Editor’s Note: 
Sequence and message examples using Annex C

Annex <E>:
Conference Management Protocol Examples

Editor’s Note: 
Examples of conference session management (OpenAPI)

Annex <F>:
Conference Management Flow Examples

Editor’s Note: 
Sequence and message examples using Annex E