Agenda item: 	10.5

Source: 	InterDigital Communications.

Title: 	iRTCW client functional components and architecture

Document for	Discussion and Agreement

# Introduction

In this contribution, we discuss the functional components of an iRTCW client. More specifically we discuss the additional functional components that extend the architecture defined in Figure 4.2 of TS 26.114 [1] to enable WebRTC based real-time communication.

# iRTCW Client functional components

The proposed functional components of an iRTCW client using 5G System are shown in Figure 1. An iRTCW client with these functional components is capable of supporting the four collaboration scenarios discussed in the permanent document [2].

Figure 1 Functional components of an iRTCW client in UE using 5G System

WebRTC session handler:

This component in the iRTCW client exchanges the SDP information along with ICE STUN and TURN server information between iRTCW peers and setup the iRTC media channels and Data channels. This module also manages the WebRTC session and QoS handling.

iRTC framework:

An open framework for an iRTC client that enables real-time communications capabilities for transport of immersive media in browser based and native webRTC applications.

iRTC media channel:

This component represents a network channel for an established media delivery session and is used for unidirectional or bidirectional media transfer between iRTCW peers using RTP/SRTP protocols. This component directly interacts with the media encoders and decoders which support 3GPP media codecs. In case of native and web-based webRTC applications, this component supports access to next generation 3GPP media codecs which are capable of encoding and decoding 3D/immersive media content.

WebRTC Data channel:

This component represents a network channel which can be used for unidirectional or bidirectional peer-to-peer transfers of arbitrary data which may include non-real time media data in some use-cases. Support for transmission of real-time media data using data channels may be further studied. Each Data channel is associated with a WebRTC session. When the SDP message contains a scene description URL, a WebRTC data channel is established to retrieve scene descriptions, including scene updates, over the network. In some use cases, the data channel may also be used for downloading some of the AR media in the scene over the network.

WebRTC Media channel:

A component in the traditional webRTC framework that is capable of encoding and decoding the real-time media and represents a network channel for an established media delivery session. This component is used for unidirectional or bidirectional media transfer between iRTCW peers using RTP/SRTP protocols.

In the case of browser-based webRTC applications, the webRTC media channels only have access to media codecs supported by the browser implementation of the WebRTC stack. To support immersive media transport, the webRTC stack implemented in the browser needs to support 3GPP media codecs. A fallback solution using WebRTC data channels and 3GPP media codecs may be further studied.

Scene Manager:

The Scene Manager is a software component that can create an immersive scene graph representation after parsing the scene description information (e.g., as described in clause 4 of TR 26.998 [2]). When an SDP message contains a URL for a scene description, the WebRTC session handler instantiates and initializes a Scene Manager.

AR media access:

A generic interface module on the client device that enables access to AR media and other related data that is needed by the scene manager or AR Runtime to provide an AR experience. The AR Media access module typically uses 5G System functionalities to access AR media and other related data.

AR Runtime:

The AR Runtime creates a local AR/MR session and performs registration with the local environment. AR runtime provides the functionalities such as allowing applications to discover capabilities of AR devices, managing an AR session and its state, receiving information about user’s actions, Spatial Computing and synchronizing the display and rendering the composited frame onto the AR displays. For spatial computing activities such as mapping, AR Runtime communicates with cameras to generate information about the world 3D space surrounding the AR user. AR Runtime also takes input from sensors for getting current position and predicting tracking positions, spatial computing activities such as relocalazation to estimate the pose when tracking is lost. The Scene Manager requests the creation of a local AR/MR session from the AR Runtime interface module present in an iRTCW client.

3D media encoder:

A generic encoder module capable of encoding the 3D media representation content in real-time. Appropriate codecs, media encoding formats and profiles will be defined by the MeCAR work item. For example, a 3D media representation may be a point cloud or a 3D mesh.

3D media decoder:

A 3D media decoder is a decoding module capable of decoding the 3D media representation in real-time.

Spatial audio decoder and encoder:

An audio decoder capable of decoding spatial audio content. An audio encoder capable of encoding multiple microphones audio signals into spatial audio content.

iRTCW client Interfaces:

iRTC-1:

This interface allows the exchange of WebRTC session information, configuration of QoS allocation and QoE metrics, collection of QoE reports and submission of consumption reports.

iRTC-2:

This interface allows the exchange of SDP information using signaling server along with ICE, STUN and TURN server information between iRTCW clients.

iRTC-3:

This interface provides communication between WebRTC Session handler and iRTC framework components and allows instantiation of iRTC media channels and Data channels for media and other related data communication between iRTCW clients.

iRTC-4:

This interface is used for transmission of media and other related data between two or more iRTCW clients using iRTC media channels and WebRTC Data channels. When the Scene description information contains a WebRTC media URL, AR media access component requests the WebRTC Session handler to instantiate required iRTC media channels and/or WebRTC Data channels for transmission of WebRTC media.

iRTC-5:

This interface is used to transmit the AR media and other related media data described in a scene description information document using any other protocols.

iRTC-6:

This interface is used to instantiate and initialize Scene Manager and AR media access component from the WebRTC session handler. When the Scene description information contains a WebRTC media URL, the AR media access component requests the WebRTC Session handler to instantiate required iRTC media channels and/or WebRTC Data channels for transmission of WebRTC media using this interface.

# Definitions

AR Media:

Media objects that are part of the scene rendered on the iRTCW client.

# Proposal

We propose to agree the baseline functional components of an iRTCW client in clause 2, definitions in clause 3 and document them in the iRTCW PD.

# References

- 3GPP TS 26.114: “IP Multimedia Subsystem (IMS) Multimedia telephony, media handling and interaction”.

- S4aR220010, “iRTCW Permanent Document”