3GPP TSG-S4   Meeting #121	S4-221251

# Toulouse, France, 14th–18th November 2022	revision of S4aI221397

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0028 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS3] Rel-16 clarifications and corrections |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-17 |
| Reason for change: |  | Responding to feedback from 5G-MAG. |  |  |  |  |  |  |  |
| Summary of change: |  | Text corrections: - 4.3.3.2: Clarification on which parties nominate the base URLs in the Content Hosting Configuration for push- and pull-based ingest. - 7.6.3.1: ingestConfigurations.path replaced by baseURL. - 7.6.3.1: distributionConfigurations.canonicalDomainName declared with wrong cardinality. - 7.6.3.1: distributionConfigurations.domainNameAlias declared with wrong cardinality. - 8: Clarification on the translation of M4d request URL into M2d URL. - 13.2.1: Replacement figure. - B: Adjustments to Content Hosting Configuration examples. OpenAPI corrections (clause C.3.5): - Bump OpenAPI version numbers. - urlPatternFilter declared in wrong scope in Content Hosting Provisioning API. Make Rel-16 match Rel-17 fix in CR0025. - ingestConfigurations.path property replaced by baseURL. - Correct distributionConfigurations.‌pathRewriteRules.‌requestPattern as requestPathPattern to match table 7.6.3.1-1. - distributionConfigurations.canonicalDomainName no longer a required property. - distributionConfigurations.domainNameAlias no longer a required property. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OpenAPI is not correctly defined. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.2, 7.6.3.1, 8, Figure 13.2.1-1, B, C.3.5 |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  | See also Rel-16 counterpart. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

# Proposal for simplifying the Content Hosting Configuration

Based on implementation of the 5GMS AS as part of the 5G-MAG Reference Tools initiative, the following simplification to the Content Hosting Configuration is proposed.

- An M2 base URL is declared in the ingest configuration by the 5GMS Application Provider (pull ingest) or by the 5GMS AF (push ingest).

  - All resources are fetched (pull ingest) or pushed (push ingest) at M2 relative to this base URL.

- The 5GMS AF chooses an M4 base URL for each distribution configuration.

  - All resources are fetched (downlink) or pushed (uplink streaming) at M4 relative to this base URL.

  - A read-only Distribution‌Configuration.‌baseURL property is added so that the chosen value can be passed back to the 5GMS Application Provider for inclusion in any M8 Service Announcement.

- There is now a very straightforward mapping between M4 request paths and M2 resource paths simply by substituting one base URL for the other.

  - Any path rewrite rules are overlaid on this basic mapping when configuring the 5GMS AS.

- The 5GMS Application Provider may optionally nominate a media entry point path suffix (e.g. a relative path to an MPD) in the top level of the Content Hosting Configuration.

  - When prefixed with the M2 base URL, this forms a fully-resolved URL to the media entry point on the 5GMS Application Provider side (pull ingest) or on the 5GMS AF (push ingest).

  - When prefixed with the M4 base URL, this forms a fully-resolved URL to the media entry point on the 5GMS AS accessible to the 5GMS Client.

  - Used to populate the optional ServiceAccessInformation.‌streamingAccess.‌mediaPlayerEntry at M5.

  - 5GMS AS may trigger start of (or reset the timeout counter for) content preparation associated with the Content Hosting Configuration when a request with this URL is received at M4.

- When URL signing is provisioned for a distribution configuration, the (initially valid) URL signing token needs to be passed back to the 5GMS Application Provider via the M1 response for inclusion in any M8 Service Announcement so that the 5GMS-Aware Application can present a valid token at M4 when required to do so.

  - (The 5GMS Application Provider already knows which M4 URLs require a signature because it provisioned urlSignature.urlPattern.)

  - Clause 7.6.4.5 specifies that "some elements of the M4d request URL" are signed, but nothing in clause 7.6 specified exactly which ones are to be included in the calculation of the signature token.

    - The description of urlSignature.urlPattern is extended so that this is also used to provision which part of the URL is to be signed.

Example Content Hosting Configuration

{ "name": "Big Buck Bunny", "entryPointPath": "BigBuckBunny/4sec/BigBuckBunny_4s_onDemand_2014_05_09.mpd" "ingestConfiguration": { "pull": true, "protocol": "urn:3gpp:5gms:content-protocol:http-pull-ingest", "baseURL": "https://ftp.itec.aau.at/datasets/DASHDataset2014/" }, "distributionConfigurations": [ { "canonicalDomainName": "m4d.mno.net", "domainNameAlias": "content.provider.com", "baseURL": "http://content.provider.com/d54a1fcc-d411-4e32-807b-2c60dbaeaf5f/", "pathRewriteRules": [], "urlSignature": { "urlPattern": "BigBuckBunny/4sec", "tokenName": "auth", "passphraseName": "passphrase", "passphrase": "sophia-antipolis", "tokenExpiryName": "expiry", "useIPAddress": true, "ipAddressName": "client-ip" } } ] }

FIRST CHANGE

#### 4.3.3.2	Create Content Hosting Configuration

This procedure is used by the 5GMSd Application Provider to create a new Content Hosting Configuration. The 5GMSd Application Provider shall use the HTTP POST method for this purpose and the request message body shall include a ContentHostingConfiguration resource, as specified in clause 7.6.3.1.

-	If the Content Hosting Configuration uses the Push-based content ingest method, i.e. the pull attribute is set to True, then the IngestConfiguration.baseURL property shall be nominated by the 5GMSd Application Provider in the request message body.

-	If the Content Hosting Configuration uses the Push-based content ingest method, i.e. the pull attribute is set to False, then the IngestConfiguration.baseURL property shall be nominated by the 5GMSd AF and returned in the response message body. It shall not be set by the 5GMSd Application Provider in the request message body.

In all cases, the DistributionConfiguration.baseURL property is read-only: it shall be omitted from the creation request and shall be assigned by the 5GMSd AF, allowing the value to be inspected by the 5GMSd Application Provider in the returned ContentHostingConfiguration resource representation, or by using the procedure specified in clause 4.3.3.3 below.

If the procedure is successful, the 5GMSd AF shall generate a resource identifier representing the new Content Hosting Configuration. In this case, the 5GMSd AF shall respond with a 201 (Created) HTTP response message and shall provide the URL to the newly created resource in the Location header field. The response message body may include a ContentHostingConfiguration resource (see clause 7.6.3.1) that represents the current state of the Content Hosting Configuration, including any fields set by the 5GMSd AF.

If the procedure is not successful, the 5GMSd AF shall provide a response code as defined in clause 6.3.

NEXT CHANGE

#### 7.6.3.1	ContentHostingConfiguration resource

The data model for the ContentHostingConfiguration resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of ContentHostingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| name | String | 1..1 | A name for this Content Hosting Configuration. |
| entryPointPath | Url | 0..1 | An entry point for a single content item expressed as a relative path (i.e. without a scheme or any leading forward slash characters). The semantics of the entry point are dependent on the value of ingestConfiguration.protocol, as specified in clause 8. If provided, the path shall be valid at reference point M2d when appended to the ingest base URL and at reference point M4d when appended to all distribution base URLs. |
| ingestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider's origin server from which media resources will be ingested via interface M2d. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | Uri | 1..1 | A fully-qualified term identifier allocated in the name space urn:3gpp:5gms:content-protocol that identifies the content ingest protocol. The set of supported protocols is defined in clause 8. |
| baseURL | Url | 0..1 | A base URL (i.e. one that includes a scheme, authority and, optionally, path segments) from which content is ingested at reference point M2d for this ingest configuration. In the case of Pull-based content ingest (pull flag is set to True), the base URL shall be provided to the 5GMSd AF to indicate the location from which content is to be pulled for this Content Hosting Configuration. A request received at reference point M4d is mapped by the 5GMSd AS to a URL at reference point M2d whose base is the value of this property. In the case of Push-based content ingest (pull flag is set to False), this property is populated by the 5GMSd AF and returned to the 5GMSd Application Provider to indicate the base URL to which content for this Content Hosting Configuration is to be published. |
| distributionConfigurations | Array(Object) | 1..1 | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| contentPreparationTemplateId | ResourceId | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4 |
| canonicalDomainName | String | 0..1 | All resources of the current distribution shall be accessible through this default Fully-Qualified Domain Name assigned by the 5GMSd AF. |
| supplementary‌Distribution‌Networks | Array(<Distribution‌NetworkType, DistributionMode>) | 0..1 | Specifies that the content for this distribution configuration is to be distributed via one of more supplementary networks. Each member of the array is a duple mapping a type of distribution network to a mode of distribution. The same DistributionNetworkType value shall appear at most once in the array. |
| domainNameAlias | String | 0..1 | The 5GMSd Application Provider may assign another Fully-Qualified Domain Name through which media resources are additionally accessible at M4d. This domain name is used by the 5GMSd AS to select an appropriate Server Certificate to present at M4d, and to set appropriate CORS HTTP response headers at M4d. If this property is present, the 5GMSd Application Provider is responsible for providing in the DNS a CNAME record that resolves domainNameAlias to canonicalDomainName. |
| baseURL | Url | 0..1 | A base URL (i.e. one that includes a scheme, authority and, optionally, path segments) from which content is made available to 5GMS Clients at reference point M4d for this distribution configuration. The value is chosen by the 5GMSd AF when the Content Hosting Configuration is provisioned. It is an error for the 5GMSd Application Provider to set this. |
| pathRewriteRules | Array(Object) | 0..1 | An ordered list of rules for rewriting the request URL paths of media resource requests handled by the 5GMSd AS. If multiple rules match a particular resource's path, only the first matching rule, in order of appearance in this array, shall be applied. |
| requestPathPattern | String | 1..1 | A regular expression [5] against which the path part of each 5GMSd AS request URL, including the leading "/", and up to and including the final "/", shall be compared. (Any leaf path element following the final "/" shall be excluded from this comparison.) In the case of Pull-based ingest, the M4d download request path is used in the comparison. In the case of Push-based ingest, the M2d upload request path is used in the comparison. In either case, if the request path matches this pattern, the path mapping specified in the corresponding mappedPath shall be applied. |
| mappedPath | String | 1..1 | A replacement for the portion of the 5GMSd AS request path that matches requestPathPattern. In the case of Pull-based ingest, ingestConfiguration.entryPoint is concatenated with the mapped path and any leaf path element from the original M4d download request to form the M2d origin request URL. In the case of Push-based ingest, canonicalDomainName (and, optionally, domainNameAlias) are concatenated with the mapped path and any leaf path element from the original M2d upload request to form the distribution URL(s) exposed over M4d. |
| cachingConfigurations | Array(Object) | 0..1 | Defines a configuration of the 5GMSd AS cache for a matching subset of media resources ingested in relation to this Content Hosting Configuration. |
| urlPatternFilter | String | 1..1 | A pattern that will be used to match media resource URLs to determine whether a given media resource is eligible for caching by the 5GMSd AS. The format of the pattern shall be a regular expression as specified in [5]. |
| cachingDirectives | Object | 1..1 | If a urlPatternFilter applies to a resource, then the provided cachingDirectives shall be applied by the 5GMSd AS at M4d, potentially overwriting any origin caching directives ingested at M2d. |
| statusCodeFilters | Array(Integer) | 0..1 | The set of HTTP origin response status codes to which these cachingDirectives apply. The filter shall be provided as a regular expression as specified in [5]. If the list is empty, the cachingDirectives shall apply to all HTTP origin response status codes at M2d. |
| noCache | Boolean | 1..1 | If set to True, this indicates that the media resources matching the filters shall not be cached by the 5GMSd AS and shall be marked as not to be cached when served by the 5GMSd AS at M4d. |
| maxAge | Integer | 0..1 | The caching time-to-live period that shall be set on ingested media resources matching the filters. This determines the minimum period for which the 5GMSd AS shall cache matching media resources as well as the time-to-live period signalled by the 5GMSd AS at interface M4d when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested. |
| geoFencing | Object | 0..N | Limit access to the content to the indicated geographic areas. |
| locatorType | Uri | 1..1 | The type of the locators shall be indicated using a fully-qualified term identifier URI from the controlled vocabulary urn:3gpp:5gms:‌locator-type, as specified in clause 7.6.4.6, or else from a vendor-specific vocabulary. |
| locators | Array(String) | 1..1 | Array of locators from which access to the resources is to be allowed. The format of the locator strings shall be determined by the value of locatorType, as specified in clause 7.6.4.6. |
| urlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content resource at M4d. |
| urlPattern | String | 1..1 | A pattern that shall be used by the 5GMSd AS to match M4d media resource URLs. The 5GMSd AS shall not serve a matching media resource at M4d unless it includes a valid authentication token calculated over the portion of the M4d request URL that matches this pattern. The format of the pattern shall be a regular expression as specified in [5]. |
| tokenName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the authentication token when required to do so. |
| passphraseName | String | 1..1 | The name of the query parameter that is used to refer to the passphrase when constructing the authentication token. Note that the token is not included in the cleartext part of the M4d URL query component. |
| passphrase | String | 1..1 | The shared secret between the 5GMSd Application Provider and the 5GMSd AS for this distributionConfiguration. The passphrase is used in the computation and verification of the M4d authentication token but is never sent in-the-clear over that interface. |
| tokenExpiryName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the token expiry field. |
| useIPAddress | Boolean | 1..1 | If set to True, the IP address of the UE is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the 5GMSd AF only when the M4d request is made from a UE with this IP address. |
| ipAddressName | String | 0..1 | The name of the M4d request query parameter that is encoded as part of the authentication token if the useIPAddress flag is set to True. Note that the IP address is not passed in the cleartext part of the M4d URL query component. |
| certificateId | ResourceId | 0..1 | When content is distributed using TLS [16], the X.509 [8] certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M4d. This attribute indicates the identifier of the certificate to use. |

NEXT CHANGE

# 8	Media Ingest and Publish (M2) protocols

## 8.1	General

The set of content protocols supported by the 5GMS AS is listed in table 8.1-1 below:

Table 8.1-1: Supported content protocols

| Description | Term identifier | Clause |
| --- | --- | --- |
| Content ingest protocols at interface M2d |  |  |
| HTTP pull-based content ingest protocol | urn:3gpp:5gms:content-protocol:http-pull-ingest | 8.2 |
| DASH-IF push-based content ingest protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 8.3 |
| Content egest protocols at interface M2u |  |  |

## 8.2	HTTP pull-based content ingest protocol

If IngestConfiguration.protocol is set to urn:3gpp:5gms:content-protocol:http-pull-ingest in the Content Hosting Configuration, media resources shall be ingested by the 5GMSd AS using HTTP [9]. The IngestConfiguration.pull property shall be set to True, indicating that a Pull-based protocol is used. The IngestConfiguration.baseURL property shall point at the 5GMSd Application Provider's origin server, as specified in table 7.6.3.1-1, and may indicate the use of HTTPS [16].

When the 5GMSd AS receives a request for a media resource at interface M4d that cannot be satisfied from its content cache, the request shall be transformed into a corresponding HTTP GET request directed to the 5GMSd Application Provider's origin server via interface M2d as follows:

1.	The prefix of the request URL indicated in the Distribution‌Configuration.‌baseURL of the applicable Content Hosting Configuration is replaced with that of the corresponding Ingest‌Configuration‌.baseURL.

NOTE:	It is the responsibility of the 5GMSd AF to assign unique M4d base URLs to each provisioned Content Hosting Configuration so as to ensure that this substitution is unambiguous.

2.	The path rewrite rules (if provisioned in DistributionConfiguration.PathRewriteRules) are applied in strict order to the remainder of the request URL (i.e., the path segments following Distribution‌Configuration.‌baseURL). The requestPathPattern of the first matching path rewrite rule is replaced with the corresponding mappedPath.

## 8.3	DASH-IF push-based content ingest protocol

If IngestConfiguration.protocol is set to urn:3gpp:5gms:content-protocol:dash-if-ingest in the Content Hosting Configuration, media resources shall be ingested by the 5GMSd AS as specified by the DASH-IF Live Media Ingest specification [3]. The IngestConfiguration.pull property shall be set to False, indicating that a Push-based protocol is used. The IngestConfiguration.baseURL property shall be set by the 5GMSd AF to the base URL that is to be used by the 5GMSd Application Provider to upload the DASH segments and MPD(s) to the 5GMSd AS at reference point M2d.

NEXT CHANGE

Figure 13.2.1-1: DASH Client Architecture

NEXT CHANGE

Annex B (informative):
Content Hosting Configuration examples

# B.1	Pull-based content ingest example

## B.1.1	Overview

1.	The 5GMSd Client on the UE requests a media resource via M4d.

2.	The 5GMSd AS determines that it does not have a cached copy of the requested media resource.

3.	The 5GMSd AS transforms the M4d request URL into a request to the 5GMSd Application Provider's origin server via M2d.

## B.1.2	Desired URL mapping

In the example shown in table B.1.2-1 below, media resources are exposed at M4d from a default canonical domain 5gmsd-as.mno.net determined by the 5GMSd System operator, and also from a custom domain name alias mno-cdn.5gmsd-ap.com that has been configured by the 5GMSd Application Provider.

Table B.1.2-1: Example URL mapping for pull-based ingest

| M4d request from 5GMSd Client | Mapped M2d request to origin server on 5GMSd AS cache miss |
| --- | --- |
| https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 | https://origin.5gmsd-ap.com/‌media/‌asset123456/video1/segment1000.mp4 |
| https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 |  |
| https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 | https://origin.5gmsd-ap.com/‌media/‌asset123456/video2/segment1000.mp4 |
| https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 |  |
| https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 | https://origin.5gmsd-ap.com/‌media/‌asset123456/audio1/segment1000.mp4 |
| https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 |  |

## B.1.3	Content Hosting Configuration

Table B.1.3-1 below shows the relevant Content Hosting Configuration parameters needed to achieve the example mapping described in table B.1.2-1 above.

Table B.1.3-1: Content Hosting Configuration properties relevant to pull-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| IngestConfiguration |  |  |
| protocol | urn:3gpp:5gms:content-protocol:http-pull-ingest | 5GMSd Application Provider |
| pull | true |  |
| baseURL | https://origin.5gmsd-ap.com/media |  |
| DistributionConfiguration |  |  |
| canonicalDomainName | 5gmsd-as.mno.net | 5GMSd AF (M1d response) |
| domainNameAlias | mno-cdn.5gmsd-ap.com | 5GMSd Application Provider |
| baseURL | https://mno-cdn.5gmsd-ap.com/m4d/‌provisioning-session9876/ | 5GMSd AF (M1d response) |
| ‌ |  |  |
| ‌ |  |  |

# B.2	Push-based content ingest example

## B.2.0	Overview

1.	The 5GMSd Application Provider uploads content to the 5GMSd AS via M2d.

2.	The 5GMSd AS rewrites the M2d upload URL to an M4d downlink URL that is exposed to the 5GMSd Client on the UE.

## B.2.1	Desired URL mapping

In the example shown in table B.2.1-1, media resources are pushed into the 5GMSd AS at M2d by the 5GMSd Application Provider and exposed to the 5GMSd Client at M4d using the canonical name of the 5GMSd AF 5gmsd-as.mno.net and an additional domain name alias mno-cdn.5gmsd-ap.com configured by the 5GMSd Application Provider.

Table B.2.1-1: Example URL mapping for pull-based ingest

| M2d ingest URL pushed to 5GMSd AS | M4d URL exposed to 5GMSd Client |
| --- | --- |
| https://5gmsd-as.mno.net/m2d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 | https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 |
|  | https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 |
| https://5gmsd-as.mno.net/m2d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 | https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 |
|  | https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 |
| https://5gmsd-as.mno.net/m2d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 | https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 |
|  | https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 |

## B.2.2	Content Hosting Configuration

Table B.2.2-1 below shows the relevant Content Hosting Configuration parameters needed to achieve the example mapping described in table B.2.1-1 above.

Table B.2.2-1: Content Hosting Configuration properties relevant to push-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| IngestConfiguration |  |  |
| protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 5GMSd Application Provider (M1d request) |
| pull | false |  |
| baseURL | https://5gmsd-as.mno.net/‌m2d/‌provisioning-session9876/ |  |
| DistributionConfiguration |  |  |
| canonicalDomainName | 5gmsd-as.mno.net | 5GMSd Application Provider (M1d response) |
| domainNameAlias | mno-cdn.5gmsd-ap.com | 5GMSd Application Provider (M1d response) |
| baseURL | https://5gmsd-as.mno.net/‌m4d/‌provisioning-session9876/ | 5GMSd Application Provider (M1d response) |
| ‌ |  |  |
| ‌ |  |  |

NEXT CHANGE

## C.3.5	M1_ContentHostingProvisioning API

openapi: 3.0.0 info: title: M1_ContentHostingProvisioning version: 2.1.0 description: ¦ 5GMS AF M1 Content Hosting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentHostingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Hosting Provisioning' externalDocs: description: 'TS 26.512 V17.3.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentHostingConfiguration summary: 'Create (and optionally upload) the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '201': description: 'Content Hosting Configuration Created' headers: Location: description: 'URL of the newly created Content Hosting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveContentHostingConfiguration summary: 'Retrieve the Content Hosting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' put: operationId: updateContentHostingConfiguration summary: 'Update the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '204': description: 'Updated Content Hosting Configuration' '404': description: 'Not Found' patch: operationId: patchContentHostingConfiguration summary: 'Patch the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' delete: operationId: destroyContentHostingConfiguration summary: 'Destroy the current Content Hosting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Hosting Configuration' '404': description: 'Not Found' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/purge: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: A unique identifier of the Provisioning post: operationId: purgeContentHostingCache summary: 'Purge the content of the cache for the Content Hosting Configuration of the specified Provisioning Session' requestBody: description: 'The regular expression pattern for resources to purge from the cache' required: true content: application/x-www-form-urlencoded: schema: properties: pattern: description: 'Keyword' type: string value: description: 'The regular expression' type: string responses: '200': description: 'Content Purged' components: schemas: IngestConfiguration: type: object description: 'A configuration for content ingest.' properties: pull: type: boolean protocol: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' baseURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' PathRewriteRule: type: object description: 'A rule to manipulate URL paths.' required: - requestPathPattern - mappedPath properties: requestPathPattern: type: string mappedPath: type: string CachingConfiguration: type: object description: 'A content caching configuration.' required: - urlPatternFilter properties: urlPatternFilter: type: string cachingDirectives: type: object required: - noCache properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 DistributionConfiguration: type: object description: 'A content distribution configuration.' properties: contentPreparationTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' canonicalDomainName: type: string domainNameAlias: type: string baseURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cachingConfigurations: type: array items: $ref: '#/components/schemas/CachingConfiguration' geoFencing: type: object required: - locatorType - locators properties: locatorType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' locators: type: array items: type: string description: 'Format of individual locators depends on the locatorType.' minItems: 1 urlSignature: type: object required: - urlPattern - tokenName - passphraseName - passphrase - tokenExpiryName - useIPAddress properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' supplementaryDistributionNetworks: type: array items: type: object description: "A duple tying a type of supplementary distribution network to its distribution mode." required: - distributionNetworkType - distributionMode properties: distributionNetworkType: $ref: '#/components/schemas/DistributionNetworkType' distributionMode: $ref: '#/components/schemas/DistributionMode' # Schema for the resource itself ContentHostingConfiguration: type: object description: "A representation of a Content Hosting Configuration resource." required: - name - ingestConfiguration - distributionConfigurations properties: name: type: string entryPointPath: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfigurations: type: array items: $ref: '#/components/schemas/DistributionConfiguration' DistributionNetworkType: description: "Type of distribution network." anyOf: - type: string enum: [NETWORK_EMBMS] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DistributionMode: description: "Mode of content distribution." anyOf: - type: string enum: [MODE_EXCLUSIVE, MODE_HYBRID, MODE_DYNAMIC] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

END OF CHANGES