Title: 	[IBACS] Basic AR call flow

Source: 	Nokia Corporation1

Document For: 	Agreement

Agenda item: 	10.6

# Introduction

The contribution provides a call flow for a basic AR call for IBACS.

# Basic AR call flow

The steps are as follows:

A. Call Setup

- UE1 sends an invite for an AR call with UE2 to the AS.

- The AS selects the proper DCS that will provide the AR application data channel resources.

- The AS instructs the DCS to setup the data channel resources for the AR application.

- The AS sets up the MRF for the AR call.

- The MRF sends SDP offer to UE2 to set up media flows.

- The MRF also sends SDP offer to UE1 to set up media flows.

- UE2 accepts the MRF offer; some parameters may be negotiated, and further offer/answer may be done.

- UE1 accepts the MRF offer; some parameters may be negotiated, and further offer/answer may be done.

B. Scene description distribution

- MRF prepares the scene description based on media descriptions and assets for the call. Some assets may be available on the DCS.

- MRF delivers the scene update to the UEs.

C. Scene description update

- A UE may trigger a scene update e.g., when a new object is added/removed in the scene, or a spatial information update is sent. The figure shows the update is triggered by UE1, but this can be either UE.

- The MRF will process the new information and creates a scene description update. It is also possible for the MRF to initiate an update without an update from the UEs.

- MRF distributes scene description update to all UEs.

NOTE: Spatial data related updates may be required for collaborative AR calls, e.g., when multiple users are physically collocated and also part of the same AR experience. The type of spatial description updates is FFS.

D. AR Media and Metadata Exchange

- Both UEs will send and receive the AR media required (and negotiated) for the call as part of the call setup and scene description information received. Prestored media can be fetched from the DCS. Real-time conversational media can flow via the MRF. The MRF may process the AR media before delivery.

- Related metadata (e.g. User Pose) may be needed during the AR session. The metadata may be delivered as RTP header extension, RTCP feedback or over data channel; this aspect is FFS.

NOTE: Delivery of real-time media over data channel is FFS.

# Proposal

The proposal is to include the text in section 2 to the permanent document of IBACS.