Source:	NTT

Title:	Thoughts on subjective listening tests for noisy speech

Document for:	Discussion and Agreement

Agenda Item:	7.5

Introduction

For subjective listening tests for stereo coding in the selection and qualification tests of IVAS, we assume to use DCR (Degradation Category Rating) defined in the test plan [1]. We presume the reference signal is the codec's input and the test signal is the output of the codec, assuming that the reference signal can provide the best quality.

There are some cases, however, that the codec for stereo or multi-channel signals may naturally reduce the background noise when input signals are noisy speech. Considering use case for teleconference, listeners feel preferable to less noisy speech signals than reference signals and may be confused by the guidance of the DCR.

Source will propose using CCR (Comparative Category Rating) [2] or clean speech signals for reference of DCR, if they are available.

Result of trial listening tests for noisy speech

We have checked the qualities of coded speech with noisy speech input by DCR under the following conditions.

- [REF] Reference signal: stereo clean speech

- [CuT1] test signal 1: added 15 dB environmental noise signals to the reference

- [CuT2] test signal 2: output of codec of IVAS stereo at 24.4 kbps as an input of [Cut1].

All inputs are female and male speakers without overlap recorded by 6 types of stereo microphone positions. To simulate ambient environmental noise, independent environmental noise signals were added using impulse responses from 8 different places with 1.5 m distance and (azimuth, elevation [radian]) of
{(-π, -π6), (-π3,π6 ), (-π6, 0), (0, -π6), (0, π6), (π6, 0), (π3, -π6), (π, π6)}.

Results are shown in Fig.1. We notice that there are 4 coded signals that have better qualities than noisy input speech. This is because stereo coding can naturally attenuate environmental noise and give higher preference.

Fig. 1  MOS for noisy speeches and coded noisy speeches by DCR with references of clean speeches. Error bars show 95% confidential intervals. Each score is averaged by eight votes on a scale of 1-5.

Proposal

So long as the application of teleconference, it is fair and natural that users prefer lower level environmental noise. Source would like to propose the followings:

- If there are 1) clean speech signals and 2) the synthesized signals with the clean speech signals and environmental noise signals, we should use the clean speech signals for reference of DCR, and the synthesized noisy speech signals for the input to the codec. The DCR will compare the quality between a) clean speech signals and b) their noised and then coded/decoded signals.

- If there are only naturally recorded noisy speech signals, we need to use CCR rather than DCR. The CCR will compare the quality between c) noisy speech signals and d) their coded/decoded signals.

Data Base

Source will make the following database A) – D) available for listening tests in the development of the IVAS coding.

A) Stereo speech for various microphone geometry

- Japanese, English, and some music sampled at 48 kHz

B) Stereo Environment Noise

- Bubble and street noise sampled at 48 kHz

C) Stereo Speech mixed with environmental noise and a set of impulse responses.

- Combined A) and B) with various source position of noise

D) A set of Impulse responses by FOA microphone

https://github.com/nttrd-mdlab/seld-foa-meir

A), B) and C) will be uploaded to 3GPP forge for open collaboration of developing IVAS. D) is already available under the condition of the license text.

References

[1] SA4-221102: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase

[2]  ITU-T p.800 Annex E, CCR