Agenda item: 	9.7

Source: 	vivo

Title: 	Split Point Update

Document for	Discussion and Agreement

- Introduction

In last meeting, basic AI/ML model distribution has been discussed, but the current service flow has only considered the AI model distribution at the first time, and the update of AI model during the service lifetime has not been discussed. So we propose to add the description of detail service flow of split point update.

- Proposed changes

--------------------------------------------- Start of Change ----------------------------------------------------------------------------

- 5.2.x	Split point update

Figure 5.2.x-1: Basic architecture for split point update, with media data source in the network

Figure 5.2.x-1 shows a simple basic architecture for split point update between the network and the UE, where the media data source is in the network, and the ordering of split inference is first performed in the network, then on the UE.

For the updating, additional components are required:

In the network:

- AI Model Capability Collection collects capabilities of UE which executes AI processing, such as memory, CPU, current UE load, UE location, and UE power consumption, etc. In the Splitting scenario, the capability collection function may also collect processing capabilities on the network side through network applications, for example, processing capabilities of the edge computing server or the central cloud server.

- AI Model Selection selects a new AI model or a new split point by considering the collected information in AI Model Capability Collection.  In the Split scenario, the AI model selection function needs to decide the latest model split policy, that is, decide the new Split Point.

- The network side can actively trigger the update of the AI Model. For example, when the network side (edge computing center, data center, and application server) finds that the memory occupied by the current processing AI Model exceeds a certain threshold, the AI Model Capability Collection can trigger the Model update or split point adjustment at the network layer

NOTE x: AI Model Capability Collection and AI Model Selection are logical functions, which may exist independently or be combined with other functions separately or completely.

In the UE:

- UE Capability Delivery Function is responsible for collecting UE’s capability periodically, and sends it to the network through 5GS.

- UE can actively trigger the update of AI Model. For example, when usage of UE computing capability (e.g. CPU) exceeds a certain threshold or UE performance deteriorates, UE will transmits UE computational information in UE Capability Delivery Function while also carrying AI Model update request, and actively triggers model update or split point adjustment at network layer

Figure 5.2.x-2: Basic architecture for split point update between the UE and network, with media data source in the UE

Figure 5.2.X-2 shows a simple basic architecture for split point update between the UE and the network, where the media data source is in the UE, and the ordering of split inference is first performed in the UE, then in the network. The resulting inference output data is then sent back to the UE.

--------------------------------------------- End of Change ---------------------------------------------------------------------------

- Proposal

- We propose to update clause 5 of the permanent document with the above proposed changes.