Source:	Xiaomi

Title:	Visual capabilities for MeCAR

## Document for:	Agreement

## Agenda Item:	9.5

# 1	Introduction

At the 3GPP meeting #120-e, SA4 published the Permanent Document (PD) for MeCAR v3.0 [1]. It has the first version of the Augmented Reality User Equipment (AR UE – former EDGAR-1) at 4.2 and some example media capabilities (5.2) and media profiles for video (5.4.1). In this contribution we go over some additional devices and use cases in order to further refine the media capabilities.

# 2	Media applications overview

Here, we propose a set of media applications that we consider to be relevant for AR UE. Based on these applications, we can extract requirements that can be used in a later stage to define the media capabilities.

Following is a table of media applications, with the device capabilities required and the requirements derived for the applications to be enabled:

Table 1 - Requirements form typical media applications

| Applications | Device Capabilities | Typical requirements |
| --- | --- | --- |
| Video Conferencing (human-to-human communication) | - Display - Camera - Speakers - Microphone | 1 audio/video encoding instance and 1 audio/video decoding instance +1 decoding and 1 encoding for screen sharing +n decoding for multiparty call (not p2p) |
| AR video call | - Display - Camera - Speakers - Microphone - 3D Graphics Processor | 1 encoding instance and 1 decoding instance video decoding with e.g., RGB plus depth, V-PCC, volumetric… +n video/image decoding if 3D assets present in the call |
| 3D Scene Rendering | - 3D Graphics Processor - Image/Video Texture Extraction - Speakers - Display | Image decoding Video decoding for video assets, video textures, possible video background Audio decoding |
| Augmented Reality Utilities (e.g. live translation, browsing, messaging) | - Camera - Display - Speakers | Images decoding 1 video decoding (video message, video tutorial, etc.) |
| Remote Rendering | - Display - Speakers - Camera - Speakers | 1 video decoding instance, matching display characteristics 1 audio decoding instance, matching speaker characteristics Transparency carriage desirable for better experience (blending in real-word scene) |

# 2	Media Capabilities for AR UE based on device characteristics

We consider the requirements from Table 1 and device characteristics of three state of the art standalone AR UEs [3] [4] [5] to compile the media capabilities shown below in Table 2.

Table 2 Media Capabilities for AR UE

| Media capability categories | Media capabilities to be supported | Justification |
| --- | --- | --- |
| Video Decoding | AVC, High Profile, Level 4.0 (at least) 2 video decoding instances for supporting applications | Display Resolution: (up to) 1,920x 1,080 per eye Refresh rate: (up to) 60Hz |
| Video Encoding | AVC, High Profile, Level 4.2 | Camera FullHD@60Hz |

# 6	Visual functions and capabilities

### Added text

## 6.1	Video Decoding

AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [6] bitstreams, with the chroma format being 4:2:0; and the bit depth being 8 bit.

HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[7] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, with the chroma format being 4:2:0; and the bit depth being 8 bit.

## 6.2	Video Encoding

AVC-FullHD-Enc: the capability to encode H.264 (AVC) Progressive High Profile Level 4.2 [6] bitstreams, with the chroma format being 4:2:0; and the bit depth being 8 bit.

HEVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 6.1

### End of Added text

# 9	AR device categories

## 9.1	Introduction

## 9.2	AR UE

### 9.2.1 	General

### 9.2.2	Refined architecture

### 9.2.3	General and system capabilities

### 9.2.4	Visual capabilities

### Added text

### 9.2.4.1	Video decoding

### The AR UE shall support video decoding AVC-FullHD-Dec as defined in clause 6.1.

### The AR UE should support video decoding HEVC-FullHD-Dec as defined in clause 6.1.

### 9.2.4.2	Video decoding interface

### The AR UE shall support two video decoding instances as defined in clause 9.2.4.1.

### 9.2.4.3	Video encoding

### The AR UE shall support video decoding AVC-FullHD-Enc as defined in clause 6.2.

### The AR UE should support video decoding HEVC-FullHD-Enc as defined in clause 6.2.

### End of Added text

### 9.2.5	Audio capabilities

# 4	Conclusion

We propose adding Sections 3 and 4 as new sections to the MeCAR Permanent Document.

# References

- S4-221150, MeCAR Permanent Document v3.0, 3GPP TSG SA WG4 120-e Meeting, August 2022

- 3GPP TR 26.998, “Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices”

- Epson Moverio BT-40s. https://www.epson.ie/en_IE/products/smart-glasses/see-through-mobile-viewer/moverio-bt-40/p/31095

- Vuzix Blade. https://www.vuzix.com/products/vuzix-blade-smart-glasses-upgraded

- Xiaomi Smartglasses. https://einfoldtech.com/2021/09/xiaomi-unveils-xiaomi-smart-glasses/

- ITU-T Recommendation H.264 (06/2019): "Advanced video coding for generic audiovisual services".

- ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".