Agenda item: 	5.2

Source: 	Ericsson LM

Title: 	Work on SA2 PDU Set Concept in SA4

Document for	Discussion and Agreement

# Introduction

3GPP SA2 has sent a liaison to SA4 on the topic of XR PDU Sets (S4-221244/S2-2209905 ‘LS on N6 PDU Set identification’) asking SA4 to investigate and, if necessary, define new mechanisms to identify PDU Sets between the AS an the UPF. SA4 is also asked to provide timeline information for such work, to enable SA2 to decide if it can be considered within SA2 Rel-18 normative work (Q1/Q2 2023).

The purpose for such work is to simplify the extraction of PDU Set-related information.

# SA2 Proposed Requirements

- Define new protocol (e.g., RTP/SRTP) header extensions to identify PDU Sets in downlink traffic in a codec agnostic and forward-compatible way.

- Identify PDU Sets in terms of, e.g. (to be discussed and confirmed with SA2):

  - PDU Set sequence number

  - PDU Set size in bits

  - PDU Set length in number of PDUs

  - PDU sequence number within the PDU Set

# Proposal

We propose that:

- Work on including PDU Set information for RTP/SRTP traffic is included as part of the Rel-18 5G_RTP WI, led by RTC SWG in collaboration with other SWGs as needed.

- Work on including PDU Set information for HTTP/HTTPS traffic is discussed and considered to be started as a new SA4 Rel-18 Study Item, led by MBS SWG in collaboration with other SWGs as needed.

- Notify SA2 on SA4 intentions, indicate that further contact between SA4 and SA2 is needed on the PDU Set information details to include in protocol header extensions, and indicate a timeline for the work.