Agenda Item:		10.5

Source:		Facebook Japan K.K. (Rapporteur)

Title:			iRTCW Permanent Document

Version:		0.30

Document for:	Discussion & Agreement

This document includes texts, figures, or other information that may complement TS 26.113 or be included later. Sections 4-8 of this document are related to clauses 4-8 of TS 26.113 respectively.

Introduction

4.	System description

4.1	High-level architecture

4.2	iRTC client in terminal

4.3	Web real-time communication

5.	Functional components

5.1	Audio

5.1.1	Microphone description

A microphone array description is provided in [10] and further extended below as an example. The sending or receiving iRTC client, or audio infra may identify the direction of each microphone from a set of angles and a type (e.g., pick up pattern). Alternatively, they may identify the position of each microphone from a set of cartesian coordinates.

In many spatial audio formats, a microphone processing block converts raw signals from microphones into a device-independent representation.

The angles are expressed in units of 1/10000 radians. For example, 3.1416 () radians is expressed as 31416.

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| Yaw (𝛹, psi) | int | Direction angle | -31416 < 𝛹 ≦ 31416 |
| Pitch (𝛳, theta) | int | Elevation angle | -31416 < 𝛳 ≦ 31416 |
| Roll (𝛷, phi) | int | Rotation angle | -31416 < 𝛷 ≦ 31416 |
| MicrophoneType | int | A number that uniquely identifies microphone type | May be used for indicating vendor-defined types |

These parameters can describe the orientation of each microphone relative to a camera, and its type. The rotation angle, not defined in [10], can be used to indicate the rotation of a directional microphone whose pick up pattern is not axisymmetric. It is assumed that the RGB or D cameras for capturing 2/3D video are located on a plane parallel to the ZY plane, pointing objects in the direction of X axis. This information on the direction of microphones (with respect to cameras) can be used for improved spatial capture of the sound field and subsequent post processing in terms of noise reduction or sound source decomposition.

Although typical UEs (at the time of writing this contribution) have 2-3 microphones, they are rarely located on the same plane. Usually, a microphone is located on top of a display while another is located at the bottom of UE. Sometimes a microphone is located on the other side of display (with main cameras). Therefore, the Linear or Planar mode [10], designated with the wMicArrayType parameter, would not be applicable.

In some contemporary UEs, e.g., foldable devices, the direction or elevation angle may be different in each session or even vary during a session, and the sending or receiving iRTC client, or audio infra may need to be informed of such changes for proper rendering. In addition to the geometric description, if applicable, other or additional information may be beneficial to the microphone processing, e.g.:

- acoustic overload point (AOP) for estimating the loudness (pressure level) of sounds in recorded audio signals

- total harmonic distortion (THD) for understanding nonlinearity in captured audio signals

- microphone sensitivity response for potential post equalisation.

Editor’s Note: Need for transmitting this information will be discussed and studied further.

5.2	Video

5.2.1	3D video capture

5.2.1.1	Depth formats & point cloud generation

Resolutions and rates of RGBD frames would influence the overall quality and their minimum required (or recommended) values may be specified for iRTC & other RTC applications. FoVs (horizontal and vertical) of RGBD frames, typically depending on the number and location (distance) of sensors, may also be specified.

5.2.1.2	Depth formats & point cloud generation

Depth information is typically stored in 16-bit unsigned integer format, which indicates the distance between each point and a vertical plane including the depth sensors, which typically consist of an (IR) transmitter and one or more receivers, in millimeter [1], [2]. Each point in 3D space is mapped onto a 2D image plane via a series of transforms illustrated below.

Mapping of 3D points to 2D image plane [3]

In the figure, [R t]T represents the rotation and translation from a 3D world coordinate system to a 3D camera’s coordinate system, whose parameters can be supplied by UE’s motion sensors. K is the camera intrinsic matrix defined as

where

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| fx | float | X-axis focal length (in pixel) |  |
| fy | float | Y-axis focal length (in pixel) |  |
| cx | float | X-axis principle point (in pixel) |  |
| cy | float | Y-axis principle point (in pixel) |  |
| s | float | Skew coefficient | Zero if image axes are perpendicular |

Depending on notation, transpose of K is also used as a camera intrinsic matrix. These device-specific information is supplied by typical OSs used in UEs [4], [5], which assume s=0. Therefore, a point cloud in e.g., PLY format can be generated from a pair of RGB and D frames by reversing the transforms, with parameters available to applications in UEs. The FoVs of RGB and D cameras will be in general different and how to align them is left to the discretion of the implementation.

5.2.2	Size measurement of 3D Objects

In [6], the needs for scaling 3D objects were illustrated for several scenarios. The conference managing server or receiving iRTC client needs the size information of 3D objects for a proper scaling. The size of a 3D object captured with a visual sensor (i.e., video camera), can be achieved with the help of the capture device information (focal length and sensor size) and the (estimated) distance to the subject.

This means to estimating a physical size of an object (or user), first the image size of the object is determined in the captured image data, and secondly the relation between the image size and the physical size is determined with the help of the camera metadata (i.e., focal length) and the objects distance to the capture device (e.g., based on a depth sensor or machine learning estimate). The resulting object size metadata comprises the size of the object to enable a rendering device or server to establish the “actual” size of the virtual object in the virtual environment in accordance with its physical size of the object in physical space.

Additionally, there are smartphone applications that estimate the length, width and height, or volume of objects seen through the viewfinder. The measurement in general depends on the types of cameras used. These applications typically place a box or cube around an object and show the measured values. If the cube around a 3D object is set to a minimum, then the size of the cube may be used as an estimate of the object’s size, as illustrated below:

Camera-based measurement applications [7], [8]

Considering that typical depth cameras represent the depth information in millimeter [9], the size information may be similarly represented as

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| w | Int | Width of cube (in mm) |  |
| h | Int | Height of cube (in mm) |  |
| d | int | Depth of cube (in mm) |  |

The size information can be signalled to a far-end iRTC client or conference managing server for scaling the 3D object to other objects or backgrounds. The size information may be transmitted periodically or in an on-demand fashion, depending on applications, and may also be used locally.

The error in the size estimation may be reduced by following common rules for placing boxes or cubes during initial measurements. A cube may not always be an optimal form to contain a point cloud captured with a limited number of RGB or D sensors of UEs, which may also depend on the FoVs of the cameras.

Editor’s Note: This will be discussed also with Video SWG.

5.2.3	Use cases and requirements for volumetric video dynamic 3D representation

Use cases 19 and 22 in TR 26.998 provide an AR conferencing experience. In both cases, participants can capture their 3D dynamic representation in real-time and share it with others in a shared AR experience. In this clause we state the requirements related to capturing and transporting the dynamic 3D representation of an object under conversational constraints. Dynamic 3D representation in this context is a format for defining the 3D model of an object (e.g., a caller) that is captured in real-time using one or more cameras. A dynamic 3D representation can be a point cloud, 3D mesh, or similar representation (e.g., RGB and depth).

TR 26.998 section 4.4.5 defines some existing formats used for dynamic 3D representations. The Virtual Reality Industry Forum (VRIF) has recently issued their first Volumetric Video Guidelines, addressing volumetric video production workflows and media profile standards for volumetric media distribution [11]. Based on the requirements for encoding, transporting, and decoding volumetric media, the following sub-categories are defined for the different use cases and their associated requirements.

5.2.3.1	AR two-party calls over IMS

The proposed solution uses RTP for low-latency delivery of dynamic 3D representations.

The use case in the above figure [xx] establishes a bidirectional AR two-party call, which may or may not use the MCU (MRF for IMS). It should be possible to combine other functions, e.g., 2D video, 360-degree video, images, etc., which are not shown in the figure for simplicity.

The dynamic 3D representations delivery in this case is over RTP. It is bidirectional with conversational latency requirements. The dynamic 3D representation can be delivered over one of the two paths shown in the figure. Path A goes through the MCU (MRF for IMS) and Path B is point-to-point.

Non-real-time 3D representations can be delivered via the data channel as shown in the figure.; the term 3D representation here includes both dynamic and static 3D representations, which are not captured and delivered under conversational latency requirements. The data channel is a WebRTC data channel or an IMS data channel. The IMS data channel used is as defined by TS 26.114. Further requirements, if needed, can be defined for using the data channel to transport 3D representations.

The following set of requirements relate to the bidirectional conversational dynamic 3D representation:

1) Call setup and control: this building block covers

a) signaling to setup a call or a conference – basic functions already provided by MTSI and will be covered also in IRTCW.

b) fetching of the entry point for the AR experience. The protocol needs to support upgrading and downgrading to/from an AR experience. Dependency on MeCAR to define device types.

2) Formats: The media and metadata types and formats include in addition to the ones already covered by MTSI, volumetric media. Format properties and codecs need to be defined for dynamic 3D representations along with appropriate RTP payload formats and functions. Appropriate codecs need to be defined by MeCAR for encoding, decoding and rendering dynamic 3D representations.

3) Real-time encoding and decoding with latency requirements for conversational media.

4) Enhancements to SDP, scene description to support AR telephony.

5) For AR telephony media types (e.g., dynamic 3D representation), the necessary QoS characteristics need to be defined.

6) Support for AR media processing in the MCU (MRF for IMS).

7) 5G system integration: offering the appropriate support by the 5G system to AR telephony includes:

a) discovery and setup of MCU (MRF) resources to process AR telephony media types.

b) defining the necessary QoS characteristics for AR telephony media types.

c) data collection and reporting.

5.2.3.2	AR multi-party calls

The use case in the above figure establishes a multiparty call. The call may be unidirectional (one dynamic 3D representation sender and multiple receivers, 1:N) or bidirectional (multiple dynamic 3D representation senders and multiple dynamic 3D representation receivers, N:N). The 1:N case can be addressed first as part of this work as it is simpler. The N:N case can be addressed later.

In addition to the requirements of use case 2.1, the following requirements need to be considered for AR multi-party calls:

• Signalling for establishing a multiparty call. This may be done in a similar way as for traditional MTSI/WebRTC calls.

• Expanding scene description to address the case of multiple senders and multiple receivers; defining appropriate procedures to maintain position of all participants in the rendered space for each participant.

• Mixing/transcoding in the MCU (MRF for IMS) to combine content from multiple participants. This may include, e.g., scaling and placement of 3D representations in a virtual room. Other requirements can also be studied.

More advanced requirements may also be considered based on the existing use cases in FS_5GXR and FS_5GSTAR

• Integration with other 5G services such as 5GMS for DASH delivery of AR media (that is not used for delivering conversational AR media but possibly video streams for a shared experience) along with conversational media.

• Maintaining consistent head motion and eye-contact in a multiparty call with 3D avatars.

Editor’s Note: The discussion here uses entities, such as MRF, from the IMS architecture. The figures can be modified once the architecture for WebRTC has been defined.

5.2.3.3	AR conferencing

The proposed solution uses cloud/SFU architecture for AR calling (e.g., AR conferencing use-case 19 in TR 26.998). The solution should use the media capabilities / APIs defined in AR device architecture in TR 26.998 and TS 26.119.

The workflow is described in the following steps:

- Capturing device is equipped with 3D video capture capabilities e.g., RGB and depth sensors.

- Captured media streams, e.g., compressed RGB and depth, are transmitted via secure WebRTC channel to the cloud.

- The cloud media processing unit processes the received streams to generate a mesh or point cloud representation.

- The cloud SFU (selective forwarding unit) handles communication between multiple device instances.

- The receiving device decodes and renders the received media stream.

The architecture supports the transmission of multiple media streams such as audio, RGB and depth streams (human representation), and non-dynamic 3D object representation.

The media transport between sender UE and cloud, and the cloud and receiver UE can use WebRTC data and/or media channel.

The solution uses WebRTC signalling and discovery mechanisms for establishing communication between different peers.

5.2.4	3D video compression

The 3D video data can be directly compressed, or analysed and converted to codes for animating an avatar [6]. As an avatar trained for a person can represent only the very person, volumetric-types can be used to represent generic 3D objects or arbitrary people.

5.2.4.1	Volumetric-type

There are standards such as V3C that can be used for compressing point cloud [21].

5.2.4.2	Avatar-type

Use of an avatar needs prior work for generating a 3D facial model of a person to represent.

There are many approaches to generate and train 3D avatars. From a large amount of image or video data, a 3D model for facial areas or heads can be defined, which is trained with neutral (no expression) and a diversity of expression data captured from a person.

3D avatar generation process [22]

A set of data professionally-captured for building 3D facial models can be downloaded from [23]. In [22], the neutral and expression data were captured with a smartphone having an RGBD camera. Note that the display is used to guide the series of expressions to make.

Once an avatar is trained, it is shared by the sender and receiver devices. During conversations or conferences, movement of facial areas are captured and converted to codes that are to animate a 3D avatar, and transmitted with compressed audio. It was shown that a VR headset could drive multiple avatars simultaneously [24].

A movie explaining the generation and operation of avatars based on [22] can be watched in [25].

5.3	Sensor

5.4	Transport protocols

5.4.1	Real-time interaction metadata transport over data channel

The interaction metadata may be carried in WebRTC data channel [15] given the data size and low-latency requirements.

5.4.1.1	Real-time interaction metadata relevant use cases

TS 26.928 [20] defines core use cases and scenarios for XR, and the interaction metadata relevant use cases are as followings:

- Real-time XR sharing such as real-time 3D communication, AR animated avatar call, and 5G shared spatial data. The relevant metadata such as gesture, position, FOV, viewport, facial expression may be used to render the view, control the animated avatar visual and auditory appearance, The XR clients may also continuously send sensing data to a cloud service which provides the map back to client.

- XR multimedia Streaming such as immersive 6DoF streaming and emotional streaming. The relevant metadata such as user pose, FOV, viewport and head or body motion may be used to control immersive 3DoF+ or 6DoF views from different position and angles and help the users to navigate through the scene.

- Online XR gaming, the relevant metadata such as body movement, viewport and gaming control metadata are essential for the rendering and reaction to enable presence.

- XR conference such as 360-degree conference meeting, 3D shared experience, 6DoF VR conferencing and XR meeting. The relevant metadata such as pose, FOV, viewport are used to control the view or poster rendering in 360-degree or 6DoF conferencing.

5.4.1.2	WebRTC data channel

In the WebRTC framework, communication between the parties consists of media (for example, audio and video) and non-media data. Media is sent using the SRTP, and non-media data is handled by using the Stream Control Transmission Protocol (SCTP) [16].

The priority associated with a media flow or data flow is classified as “very-low”, “low”, “medium”, or “high” in the API. WebRTC implementations may attempt to set QoS on the packet sent according to the guidelines in [17].

The SCTP provides the following features for transporting non-media data between browsers:

- Support of multiple unidirectional streams

- Ordered and unordered delivery of user messages

- Reliable and partial reliable transport of user message

An SCTP packet is composed of a common header and chunks. A chunk contains either control information or user data. The SCTP packet format is shown below:

SCTP packet format

The common header field includes a source port number, a destination port number, a verification tag and checksum, as shown in the following figure.

SCTP common header format

Each chunk is formatted with a chunk type field, a chunk-specific Flag field, a Chunk Length field, and a value field as shown below.

SCTP chunk field format

SCTP defines a number of chunk types, and one of the types is Payload Data (DATA) when chunk type is 0. The payload data format is shown in the following figure. The U bit indicates unordered data chunk; B and E bit indicate the fragmented user message; the payload protocol identifier represents an application specific protocol identifier and is passed to SCTP by its upper layer and sent to its peer.

SCTP chunk payload data type format

5.4.1.3	Potential solution for using data channel for interaction metadata

The WebRTC data channel may be used to convey real-time interactive metadata.

[Editor’s Note: carrying interaction metadata over the Data Channel using this approach may be useful but further study of use cases and other types of interaction metadata will be investigated to identify other potential  solutions.]

It is proposed to use registered SCTP PPID 53, “WebRTC Binary” [15], to carry the interaction metadata. A generic payload format is shown in the following figure.

Generic data channel payload format for the timed metadata

The 32-bits Subprotocol payload ID indicates the subprotocol or specifications used for the metadata payload data format, such as OpenXR.

The 16-bits Metadata type field may indicate the metadata type specified in the payload subprotocol.

[Editor’s note: the data length may be extended if 16-bits is not sufficient to accommodate the potential metadata type indication such as URN]

The 16-bit Metadata attributes field may indicate the metadata attributes such as time synchronization and reliability.

A WebRTC data channel may be optionally mapped by means of an SDP offer/answer [18] or DCEP [19] procedure to a “subprotocol” parameter indicating which protocol the client expects to exchange data via the channel. This “subprotocol” parameter value is registered for WebRTC data channels as per [19] Section 9.1, or equivalently [19], Section 5.1. If the “subprotocol” parameter is not present, then its value defaults to an empty string.

For the interaction metadata specified in MeCAR, the subprotocol could be defined as “3gpp-timedmetadata”, and the message format would be binary. The specific syntax and semantics of the interaction metadata are indicated by the 32-bits subprotocol payload ID field in the User Data of the SCTP packet.

The following figure is an example of an SDP offer that adds 1 timed metadata data channel stream (stream id=110).

Example SDP offer with timed metadata signalling

The following figure is an example considering further down the syntax and semantics of the generic candidate fields introduced in the figure for the case of an OpenXR XR_EXT_hand_tracking metadata:

- 32-bits subprotocol payload ID is set to an OpenXR ID value.

- 16-bits metadata type field is set to 52 which is the registered extension number for XR_EXT_hand_tracking in OpenXR.

- 16-bits metadata attributes field may indicate the metadata attributes such as time synchronization and reliability.

- OpenXR defines a 64-bit signed integer representing nanoseconds (XrTime) and associates it to the APIs. Yet, SCTP does not carry timestamp information. For the payload protocol that does not define timestamp, a timestamp field may be added to the chunk user data section when a metadata timestamp attribute bit, T, is set to 1 as below.

- OpenXR (XrTime) 64-bit signed timestamp in nanoseconds.

Example of OpenXR XR_EXT_hand_tracking metadata payload format

5.4.1.4	Security considerations

The interaction class real-time metadata can contain sensitive information tracking the interactions of an end user, e.g., elements of pose, tracking information of palm, hand, or face, as well as controller inputs. Therefore, the integrity and confidentiality of metadata in transit is in some scenarios, depending on the application requirements, necessary.

The transport of the real-time interaction class metadata over the WebRTC data channel and in particular over SCTP as of clause 2.3 ensures by default, [15], integrity and confidentiality given the underlying DTLS over UDP secure transport.

6.	Session management

6.1	WebRTC functions in 5GS

7.	Inter-working

8.	Packet-loss handling

9.	Architecture and function

9.1	WebRTC QoS architecture

Collaboration scenarios.

9.1.1	5G support for OTT WebRTC

In this scenario, the application provider offers a WebRTC service to their customers and is responsible for the security and privacy of the data exchanged over their service. The application provider desires to improve the quality of the service for mobile users.

The collaboration scenario is similar to the 5GMS streaming model, where all involved Application Servers (AS) are in non-trusted domain.

The following figure depicts this collaboration scenario:

    - MNO-provided trusted WebRTC functions

In this collaboration scenario, the user utilizes a variety of WebRTC-based conferencing services for personal and work purposes. The user is concerned about their privacy, e.g., through threats from man-in-the-middle attacks that result from usage of untrustworthy ICE functions. the user opts for using a trusted WebRTC configuration, that is provided by their MNO, which offers ICE functions such as STUN, TURN, MCU, etc.

In addition, the MNO will offer the required traffic handling to the WebRTC sessions, from some or all application providers, based on existing SLAs or based on user contractual agreements.

The following figure depicts this collaboration scenario:

    - MNO-facilitated WebRTC services

The MNO may offer several WebRTC-based services such as remote gaming, AR telephony, etc. These services are facilitated and managed by the MNO and are offered to the MNO’s customers exclusively.

All WebRTC functionality is hosted by the MNO, which ensures the quality of the services through appropriate network assistance, such as QoS allocation.

The following figure depicts this collaboration scenario:

    - Inter-operable WebRTC services

As an extension of previous collaboration scenario, a globally inter-operable WebRTC service is provided, where mobile users from different MNOs are able to join the same service and benefit from the 5G system support for better end-to-end quality of service. In this collaboration scenario, the WebRTC functions are hosted by one or more MNOs.

The following figure depicts this collaboration scenario:

    - WebRTC functions in the context of iRTCW

## 9.2.1		General

We have agreed on 4 different collaboration scenarios for iRTCW at the SA4-118e meeting. The collaboration scenarios are listed here for convenience:

- 5G support for OTT WebRTC: in this scenario the WebRTC session runs completely over the top. However, the MNO may offer support in form of QoS allocation, bitrate recommendations, and QoE report collection based on request by the UE.

- MNO-provided trusted WebRTC functions: in this scenario the MNO offers trusted support functions such as ICE servers to the WebRTC application on the UE.

- MNO-facilitated WebRTC services: the MNO may host and facilitate WebRTC sessions by providing a trusted WebRTC signaling server, which may also offer 5G network assistance.

- Inter-operable WebRTC services: collaboration scenario 3 is extended with functions to support MNO to MNO inter-operability.

Based on the documented collaboration scenarios, we identify the following functions and describe their roles.

## 9.2.2		Potential 3GPP-defined functions

9.2.2.1	General

These functions will be discussed and possibly confirmed in the context of the architecture that will be defined by the 5G_AREA WI.

9.2.2.2	Provisioning server

The provisioning server may enable an application provider to perform provisioning of the following functionalities:

- QoS support provisioning for WebRTC sessions

- Charging provisioning for WebRTC sessions

- Collection of consumption and QoE metrics data provisioning related to WebRTC sessions

- Offering ICE functionality provisioning such as STUN and TURN servers

- Offering WebRTC signaling servers provisioning, potentially with interoperability to other signaling servers.

The provisioning server may not be relevant to all collaboration scenarios and some of the 5G support functionality may be offered without application provider provisioning.

### 9.2.2.3	Configuration server

The configuration server stores WebRTC-related configuration information and makes them accessible to the UE. It stores information and recommendations to operate network-assisted WebRTC sessions over 5G.

The configuration information may consist of static information such as the following:

- Recommendations for media configurations

- Configurations of STUN and TURN server locations

- Configuration about consumption and QoE reporting

- Discovery information for WebRTC signaling and data channel servers and their capabilities.

### 9.2.2.4	Media session handler (MSH)

The MSH is an entity running on the UE, which assists with the 5G integration of the WebRTC application. It exchanges, on behalf of the application, information about the WebRTC sessions with the network.

The MSH receives information about a new WebRTC session from the application. It relays the information to the Support Function. It also receives events and other network information about the WebRTC session from the Support Function, which it may relay to the application.

### 9.2.2.5	Network support function

The support functionality includes the following:

- Network Support Function receives information about a WebRTC session and its state

- Network Support Function requests QoS allocation for a starting or modified session

- Network Support Function receives notification about changes to the QoS allocation for the ongoing WebRTC session

- Network Support Function exchanges information about the WebRTC session with the trusted STUN/TURN/Signaling Server, e.g. to identify a WebRTC session and associate it with a QoS template.

## 9.2.3		WebRTC functions

### 9.2.3.1	Trusted ICE functions

The MNO may offer trusted ICE functions to the WebRTC application to be used during the WebRTC ICE gathering phase. These functions may be STUN and TURN servers that facilitate NAT and Firewall traversal.

The MNO-operated trusted ICE functions may assist with the 5G integration of the WebRTC application. This could be done by triggering network assistance to starting or ongoing WebRTC sessions.

## 9.2.4		iRTCW-defined functions

### 9.2.4.1	Trusted WebRTC signaling server

The trusted WebRTC signaling server is used to setup and manage MNO-operated WebRTC applications. They offer a standardized signaling protocol for the session setup to both parties of the WebRTC session. The WebRTC signaling server will handle the offer/answer exchange and will have access to the SDP in both directions.

The WebRTC signaling server may use that knowledge to offer network assistance and other 5G features to the endpoints of the WebRTC session.

9.2.4.2	Inter-working function

This function provides inter-working functionality to enable MNO-facilitated WebRTC sessions that involve end-points across different MNOs. They may for example provide cross-network signaling functionality to allow WebRTC signaling server that are hosted in different networks to communicate, in order to establish and manage the WebRTC sessions.

### 9.2.4.3	Trusted media server

A media server may be offered by the MNO to support WebRTC sessions. It may offer a wide range of functionality such as:

- a content server that serves content to the WebRTC application, e.g. through a data channel

- media processing functionality: may be used by the WebRTC application as a relay that performs some media processing function such as transcoding, recording, 3D reconstruction, etc.

- scene composition functionality: the server may compose a 3D scene and distribute it to several point-to-point WebRTC sessions

- MCU functionality: the server may offer multi-party conferencing functionality to merge a number of point-to-point WebRTC sessions

- SFU (Selective Forwarding Unit) functionality: the server may offer the selection, copy, and forwarding functionality of IP steams produced by multiple WebRTC endpoints (i.e., participants).

# Mapping to collaboration scenarios

The following table provides an initial mapping of the identified functions to the collaboration scenarios:

| Functions/CS | Collaboration scenario 1 | Collaboration scenario 2 | Collaboration scenario 3 | Collaboration scenario 4 |
| --- | --- | --- | --- | --- |
| Provisioning server | Optional | Optional | Optional | Optional |
| Configuration server | Optional | Required | Required | Required |
| MSH | Required | Optional | Optional | Optional |
| Network support function | Required | Required | Optional (maybe fulfilled by WebRTC signaling server) | Optional |
| Trusted ICE function | N/A | Required | Optional | Optional |
| Trusted WebRTC signaling server | N/A | N/A | Required | Required |
| Trusted media server | N/A | Optional | Optional | Optional |

Annex

A.1	Connection models to be used in iRTCW

It is assumed that the services to be supported by iRTCW are real time, with multiple parties, and with some media handling (e.g., VR conferencing, a live performance with a large audience, remote operation supported by multiple experts). The number of UEs can be from two to a large number, e.g., several hundreds. The number of UEs may change during the service, e.g., from two to ten, and to several hundreds. The discussion is what is the best connection model to meet this assumption.

There should be at least four connection models to be considered.

Connection model #1: Multiple UEs are connected via a media server.

Connection model #2: Two UEs are connected via a media server.

Connection model #3: Multiple UEs are connected directly (not via any media servers).

Connection model #4: Two UEs are connected directly (not via any media servers).

It should be noted that Connection models #2 and #4 are introduced for the sake of argument. Connection model #2 can be seen as a subset of Connection model #1. Connection model #4 can be a subset of Connection model #3.

Regarding the necessary network functions, Connection models #1 and #2 need a media server as an extra entry, while Connection models #3 and #4 do not.

When it comes to a conference service, the media server in Connection models #1 and #2 can serve as a conference room, which can be seen by a UE as an intermediate target destination (as a rendezvous point) instead of the real peer UEs. The conference room provided by the media server allows a calling UE to stay there, does not require immediate discovery of the real target UEs, and thus does not require the registration of the peer UEs. In Connection models #3 and #4, the conference session should be set up among all UEs by exchanging information (by SDP offer/answer) among them. Exchanging the information, which is performed by control plane signalling, needs to resolve the exact destination of the peer UEs, which should be registered in advance. In other words, Connection models #3 and #4 require a network entity for UE registration.

The iRTC service may need connections between a wide range of the numbers of UEs (from two parties to multiple parties, even scalable to hundreds of parties). Considering the scalability, a large number of direct connections between multiple UEs (by Connection model #3) have a severe issue for network performance. The number of media paths increases by the square of the number of UEs, which easily leads to network congestion.

The number of the parities may change during a call/service. Direct connection model (by Connection model #4) may be best for a two-party call, but how to support additional UEs during a call should be considered. Connection model #4 can easily transit to Connection model #3, which leads to the network congestion issue. If Connection model #4 transits to Connection model #1, it involves a complex process for transition. For supporting modification of a call by increasing and decreasing the number of UEs, using a media server (by Connection models #1 or #2) is simple and scalable.

Regarding the communication delay, connections through a media server have a certain additional delay and direct connections have a shorter delay.

In some services, such as conferencing [Conferencing-oriented], the basic connection model is #1 or #2 (connection via media server). In other services and purposes, such as split rendering, the basic connection model is #3 or #4 (direct connection between endpoints).

|  | Multiple UEs connected via a media server (Connection model #1) | Two UEs connected via a media Server (Connection model #2) | Multiple UEs connected directly (Connection model #3) | Two UEs connected directly (Connection model #4) |
| --- | --- | --- | --- | --- |
| NW/UE functions specific to the connection model | Media server |  | - UE registration to receive incoming calls - Direct reachability between UEs (i.e., IPv6 global unicast address or IPv4 and Full-cone NAT) |  |
| Responsiveness of call modification (increase/decrease the number of UEs) | High (UEs can join and leave the call while maintaining the same connection model) |  | Low (especially for increasing the number of UEs, full-mesh connection has severe network performance issue for scalability) | Low (when additional UEs joins the call, the connection model may move to Connection model #1 or #3) |
| Communication Delay | Ordinary RTT using intermediate server for conferencing |  | Shorter delay than with a media server |  |

Connection model #1

Connection model #2

Connection model #3

Connection model #4

References

[1] Azure Kinect DK hardware specifications, Microsoft

[2] Intel RealSense D400 series product family datasheet, Intel

[3] Camera calibration parameters, MathWorks

[4] intrinsicMatrix, Apple

[5] CameraIntrinsics, Google

[6] S4aR220046 3D video types for iRTC client in the terminal, Meta

[7] Quick measure, Samsung

[8] Measure app, Apple

[9] S4aR220014, 3D video capture description for iRTC client in the terminal, Meta

[10] Windows microphone array geometry descriptor format, Microsoft

[11] VRIF volumetric video guidelines v0.95, https://www.vr-if.org/guidelines/

[12] 3GPP TSG SA WG4 S4-221150, “MeCAR Permanent Document v0.3.0”, August 2022

[13] 3GPP TSG SA WG4 S4-221209, “5G_RTP Permanent Document v0.0.2”, August 2022

[14] IETF RFC 4566, “SDP: Session Description Protocol”, July 2008

[15] IETF RFC 8831, "WebRTC Data Channels", January 2021

[16] IETF RFC 4960, “Stream Control Transmission Protocol”, September 2007

[17] IETF RFC 8837, “Differentiated Services Code Point (DSCP) Packet Markings for WebRTC QoS"

[18] IETF RFC 8864, “Negotiation Data Channels Using the Session Description Protocol (SDP)”, January 2021

[19] IETF RFC 8832, “WebRTC Data Channel Establishment Protocol”, January 2021

[20] 3GPP TS 26.928, “Extended Reality (XR) in 5G”, April 2022

[21] ISO/IEC 23090-5:2021: “Information technology — Coded representation of immersive media — Part 5: Visual volumetric video-based coding (V3C) and video-based point cloud compression (V-PCC)”

[22] C. Cao, T. Simon, J. Kim, G. Schwartz, M. Zollhoefer, S. Saito, S. Lombardi, S. Wei, D. Belko,

S. Yu, Y. Sheikh, J. Saragih, Authentic volumetric avatars from a phone scan, ACM Transactions on Graphics, Volume 41, Issue 4, July 2022

[23] Multiface dataset, Meta

[24] S. Ma, T. Simon, J. Saragih, D. Wang, Y. Li, F. De La Torre, and Y. Sheikh, Pixel codec avatars, IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), June 2021

[25] https://www.youtube.com/watch?v=t7_TMD7v0Xs

# Revision history

| Date | Meeting | Subject / Comment | Old | New |
| --- | --- | --- | --- | --- |
| 2022-05-20 | SA4#119-e | - PD skeleton (S4-220769) | 0.10 | 0.11 |
| 2022-11-14 | SA4#121 | - 3D video capture (S4aR220014) - Microphone description (S4aR220043) - 3D size measurement and scaling (S4aR220023) - WebRTC QoS architecture (S4aR220010) - Dynamic 3D representation use cases and requirements (S4-221193) | 0.11 | 0.20 |
| 2023-02-20 | SA4#122 | - Real-time metadata transport over data channel (S4-221557) - Additions to size measurement of 3D objects in iRTCW (S4-221546) - iRTCW architecture for AR conferencing (S4-221547) - Proposal for connection models to be used in iRTCW (S4-221549) - 3D avatar generation & operation for iRTC client in the terminal (S4aR230011) | 0.20 | 0.30 |