Source:	Intel, Nokia Corporation

Title:	use case of Multiple Video Sources in UE

Agenda Item:	10.10

Document for:	Discussion and Agreement

- Introduction

In this contribution, we would like to introduce a use case where a UE has multiple video sources.

It covers the cases where WebRTC-enabled UE has Multiple Video Sources covering different areas such as 360 degrees of view. In some cases, not all of the current UE’s field-of-view (FoV) is necessary to be streamed or the receiver-side UE does not care what’s behind source’s current FoV. This contribution was previously discussed in the SA4#121 meeting.

- Multiple Video Sources and zone allocations

Figure. X-2.1 illustrates a common use case when a UE has multiple video sources (e.g. 2D/3D-capable). Each camera may have a fixed Field-of-View (FoV) or varied FoV. Each camera may support the same set of video capabilities such as codec support.

A camera ID or zone ID is assigned for each camera. By assigning a zone number, the UE has the flexibility to signal each source by its source ID (e.g. SSRC in case of RTP) or zone/camera ID.

The zone ID may be assigned with a priority based on the areas it covers and may consist of one or more cameras. For example, the area covered by cameras in zone-1 may be more important than the ones located in zone-2 and zone-3 since it covers the front FoV of the UE. This is important information since it enables UE to signal the essential zone areas or high-priority zones. In some cases, all of the zones have to be treated equally, then all the zone will have the same priority assignment.

Figure. X-2.1 Multiple Video Sources With different zone allocations in UE

Note: the number of cameras and size of UE can be varied.

For such a UE to be able to support signaling multiple video sources over the network, UE may have a choice of signaling an SDP by a) combining multiple zones into one or b) sending SDP from the individual zone.

In summary, in this use case, there are following possible scenarios:

1) Each media source is able to signal individual control message and send individual media stream

2) Media sources inside of each zone may get combined together, send combined control message and media stream per zone.

3) Media sources from each zone may get combined together and send a combined control message and media stream as a whole.

4) It may be further possible to associate the zone IDs or camera IDs to particular pose information when the UE is creating or sending immersive content. Streams from individual camera or cameras in certain zones can be paused/resumed depending on the viewing orientation of the receiver UE (i.e., for viewport-dependent media).

- Proposal

It is proposed d to add clause 2 into C.x of the FS_eiRTCW permanent document and work on session management-related procedures for this use case.