3GPP TSG SA WG4 Meeting #123	TDoc S4-230695

Electronic, 17th–21st April 2023	revision of S4-230578

Title:	LS on Object Acquisition Method

Response to:	—

Release:	Rel-17, Rel-18

Work Item:	5MBS, 5MBUSA

Source:	3GPP SA4

To:	3GPP CT4

Cc:	3GPP CT3

Contact person:	Richard Bradbury

richard dot bradbury at bbc dot co dot uk

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA4 has identified a small gap in the Nmbstf_MBSDistributionSession service API specified in TS 29.581 and would like to draw this to CT4's attention.

## 1.1	Stage 2 requirements in TS 26.502

Table 4.5.6-2 of TS 26.502 defines the Object acquisition method of an MBS Distribution Session as follows:

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| Object acquisition method | 1..1 | MBS Application Provider | Indicates whether the objects(s) are to be pushed into the MBSTF by the MBS Application Provider or whether they are to be pulled from the MBS Application Provider by the MBSTF as part of the corresponding MBS User Data Ingest Session. In the latter case, the Object acquisition method indicates whether the object(s) are to be retrieved once from the MBS Application Provider at the start of each active period of the corresponding MBS User Data Ingest Session, or whether the MBSTF is required to check their validity periodically, for example once per rotation of an object carousel. |

## 1.2	Stage 3 API specification in TS 29.581

The data type ObjDistributionData in table 6.1.6.2.5-1 of TS 29.581 specifies the objDistributionOperatingMode attribute in terms of an enumerated data type ObjDistributionOperatingMode and specifies the objAcquisitionMethod attribute in terms of an enumerated data type ObjAcquisitionMethod:

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objDistributionOperatingMode | ObjDistributionOperatingMode | M | 1 | Operating Mode for the Object distribution method |  |
| objAcquisitionMethod | ObjAcquisitionMethod | M | 1 | Indicates whether the objects(s) are to be pushed into the MBSTF by the MBS Application Provider or whether they are to be pulled from the MBS Application Provider by the MBSTF |  |

The enumerated data type ObjDistributionOperatingMode is specified as follows in table 6.1.6.3.4-1 of TS 29.581:

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SINGLE" | Each object ingested by the MBSTF is distributed once |  |
| "COLLECTION" | A set of objects described by a manifest is ingested by the MBSTF and distributed once Not used with Push-based ingest |  |
| "CAROUSEL" | A set of one or more objects described by a manifest is ingested by the MBSTF and distributed according to a repetition pattern specified in the manifest Not used with Push-based ingest |  |
| "STREAMING" | A sequence of objects is ingested by the MBSTF and streamed in real time |  |

Furthermore, the enumerated data type ObjAcquisitionMethod is specified as follows in table 6.1.6.3.5-1 of TS 29.581:

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "PULL" | MBSTF Pulls objects from AF/AS |  |
| "PUSH" | AF/AS Pushes objects into MBSTF |  |

SA4 observes that the current API specification in TS 29.581 does not satisfy the requirement highlighted in section 1.1 for the MBSF (acting on behalf of the MBS Application Provider) to be able to unambiguously indicate the two distinct subcases of pull-based object acquisition highlighted above to the MBSTF at reference point Nmb2.

A potential remedy to satisfy the stage-2 requirement in TS 26.502 would be to add an independent Boolean parameter (e.g., objAcquisitionKeepUpdated) to the ObjDistributionData type defined in table 6.1.6.2.5-1 of TS 26.581 indicating the need for the MBSTF to check for updates to the ingested acquisition objects. In Release 17, this applies only to the carousel operating mode provisioned to use the pull-based object acquisition method, but it may additionally apply to operating modes defined in subsequent releases.

# 2	Actions

To CT4

ACTION: 	SA4 kindly asks CT4 to take the above information into account.

ACTION: 	SA4 kindly asks CT4 to advise of any resulting change to TS 29.581 by LS response.

# 3	Dates of next TSG SA WG 4 meetings

SA4#124	22nd–26th May 2023		Berlin, Germany

SA4#125	21st–25th August 2023		Gothenburg, Sweden