Source: 	Nokia Corporation, Interdigital, Sony Group Corporation, Intel, Philips, Samsung

Title: 	[MeCAR] Volumetric video operation points

Document for	Agreement

Agenda item: 	9.5

# Introduction

During the SA4#122 a new section on volumetric video support was added as background to the MeCAR Permanent document with an indication from proponents to work toward defining the required media capabilities to support volumetric video coding for different MeCAR device types, one possible codec for volumetric video being the Visual Volumetric Video-based Coding (V3C) standard.

Before starting work on defining the required media capabilities related to volumetic video, the contribution proposes how to specify the volumetric video codec in TS 26.119. The changes are proposed for the MeCAR Permanent document v 5.1 (S4aV230017).

Note that in this document we mainly consider IF-4 (the interface between the Media Access Function and the 5G System for user plane data) as defined in section 4.1 of the Permanent document and presented on Figure 1. The client architecture figure is shown here for reference only.

Additionally, the contribution provides information on the coding capabilities of Visual Volumetric Video-based Coding (V3C).

Figure 1 Client architecture from MeCAR Permanent document

# Pseudo Change Request

## 2.2	Abbreviations

PLR - Point Local Reconstruction

EOM - Enhanced Occupancy Mode

VUI - Volumetric Usability Information (when used in reference to V3C)

## 9.X	3D Media capabilities

## 9.X.1	V3C coding capabilities

## 9.X.1.1 General

ISO/IEC 23090-5 specifies a generic mechanism for volumetric video coding, i.e., visual volumetric video-based coding (V3C), that may be used by applications targeting volumetric content, such as point clouds or immersive video with depth. The applications of V3C target point cloud representations of volumetric frames in Annex H of ISO/IEC 23090-5 (V-PCC). ISO/IEC 23090-12  specifies the applications of V3C targeting Immersive Video (MIV).

Unlike two dimensional image or video coding standards, the V3C standard specifies profiles as a combination of three components, namely CodecGroup, Toolset, and Reconstruction components.

The CodecGroup component indicates video decoding specifications and their profiles, e.g., Main 10 as specified in ISO/IEC 23008-2, that is used for encoding V3C video components. The CodeGroup profile component affects only V3C video components and does not have impact on V3C atlas component. The Toolset profile component indicates the coding tools that could be used to generate the conformant bitstream and shall be supported by a decoder. The Toolset profile component affects only the V3C atlas components and does not have impact on the V3C video component. The CodecGroup and Toolset components specify conformance point A in ISO/IEC 23090-5.

The Reconstruction profile component describes conformance point B in ISO/IEC 23090-5 and it specifies the pre-reconstruction, reconstruction, post-reconstruction, and adaptation tools supported or recommended to achieve conformance in terms of 3D reconstruction. A decoder may support and may be able to conform to multiple reconstruction profiles specified in ISO/IEC 23090-5. However, the Reconstruction profile that a decoder selects to operate in, is outside the scope of ISO/IEC 23090-5. When a decoder claims to be operating at a particular conformance point B, it shall be capable of reconstructing a volumetric frame according to the reconstruction recommendations specified by that profile.

The following subsections further summarize profile, tier and level structure for V3C codecs.

## 9.X.1.2 V3C decoder conformance points

V3C profiles follow a structured and flexible definition to allow for clearly identifying two distinct conformance points, Conformance Point A and Conformance Point B. These conformance points are illustrated in the decoding block diagram shown in Figure X.

Figure X V3C decoding block diagram with decoding conformance points A and B

The first conformance point, point A, covers the decoded video sub-bitstreams and atlas sub-bitstream. It also covers the derived block to patch map information. The second conformance point, point B, covers the reconstruction process.

In order to signal the required decoding capability at a given conformance point V3C utilizes a combination of syntax elements ptl_tier_flag, ptl_level_idc, ptl_profile_codec_group_idc, ptl_profile_toolset_idc, and ptl_profile_reconstruction_idc that are present in the V3C bitstream. A mapping of the syntax elements to a given conformance point is shown in Figure X2.

Figure X2 - V3C profile, level, and tier structure

V3C bitstreams conforming to a specified tier and level obey the following constraints for each atlas bitstream, where tiers, main (MT) and high (HT) relate directly to the video bitstream level limits.

- V3C bitstream level

  - Maximum of PROJECTED, EOM, and RAW points per second

  - Maximum of PROJECTED, EOM, and RAW points per atlas frame

  - Maximum number of maps

  - Maximum number of attributes

  - Maximum number of attribute dimensions (channels)

- Atlas bitstream level

  - Maximum of PROJECTED, EOM, and RAW patches per atlas frame

  - Maximum of PROJECTED, EOM, and RAW patches per second

  - Maximum number of tiles per atlas

  - Maximum atlas size

  - Maximum atlas bitrate

- Video bitstream level

  - Maximum luma picture size

  - Maximum aggregated luma sample rate

  - Maximum bitrate per video stream

  - Maximum aggregated video stream bitrate

The CodecGroup and Toolset profile components describe conformance point A. More specifically, CodecGroup describes video decoding specifications and their profiles, and Toolset specifics tools (e.g., use of EOM and PLR etc) allowed, i.e., define constrains on the atlas bitstream syntax structures.

- CodecGroup

  - AVC Progressive High

  - HEVC Main10

  - HEVC444

  - VVC Main 10

  - HEVC Main

  - MP4RA (codec specified by a fourcc registered at mp4ra.org)

- Toolset

  - V-PCC Basic (specified Annex H of ISO/IEC 23090-5)

  - V-PCC Basic Still (specified Annex H of ISO/IEC 23090-5)

  - V-PCC Extended (specified Annex H of ISO/IEC 23090-5)

  - V-PCC Extended Still (specified Annex H of ISO/IEC 23090-5)

  - MIV Main (specified Annex A of ISO/IEC 23090-12)

  - MIV Extended (specified Annex A of ISO/IEC 23090-12)

  - MIV Geometry Absent (specified Annex A of ISO/IEC 23090-12)

The V3C bitstream profile, level and tier syntax further indicates the number of maximum required video decoder instantiations, from 0 to a maximum of 32 video decoder instances.

To indicate profile conformance of a bitstream or decoder at conformance point A, it is only needed to consider the CodecGroup and Toolset profile components. Decoders conforming to a V3C profile at conformance point A at a specific level of a specific tier shall be capable of decoding all V3C bitstreams or collection of V3C sub-bitstreams, for which all of the following conditions apply:

- The V3C bitstream or the collection of V3C sub-bitstreams are indicated to conform to the supported CodecGroup and Toolset profile components.

- The V3C bitstream or the collection of V3C sub-bitstreams are indicated to conform to a level that is lower than or equal to the specified level.

- The V3C bitstream or the collection of V3C sub-bitstreams are indicated to conform to a tier that is lower than or equal to the specified tier.

The Reconstruction profile component describes conformance point B, specifying the pre-reconstruction, reconstruction, post-reconstruction, and adaptation tools supported or recommended to achieve conformance in terms of 3D reconstruction.

A decoder may support and may be able to conform to multiple reconstruction profiles as specified. When a decoder claims to be operating at a particular conformance point B, it shall be capable of reconstructing a volumetric frame according to the reconstruction recommendations specified by that profile.

# X.X.  Working assumption on operation points for MeCAR

Before starting work on the defining required media capabilities related to 3D video, Operations Points are to be considered, where Operation Point is aligned with the definition in TS26.118 as "a collection of discrete combinations of different content formats including spatial and temporal resolutions, colour mapping, transfer functions, rendering metadata or the encoding format."

The MeCAR specification would define operation points for immersive video in IF-4, i.e. the interface between the Media Access Function and the 5G System for user plane data as defined in section 4.1.

However, in contrast to TS26.118, where the bitstreams conforming to Operation Points were defined to be self-contained omnidirectional experiences, in TS26.119, the bitstreams may be added to a scene that renders and composes data contained in multiple bitstreams.

Hence, carrying forward the definition from TS 26.118 into TS 26.119 may not be straightforward. We may need to differentiate three cases:

- The operation point is such that no action or pose-based rendering is needed, i.e. that operation point is directly consumable by the XR Runtime as the media is already projected (left/right eye), equirectangular, etc., possibly with some simple colour conversion. Note that this may require some amount of metadata processing.

- The operation point describes an asset and includes asset metadata that requires a GPU-based rendering taking into account the viewer pose, but can be projected into a composition layer to be consumed by the run-time

- The operation point is a scene that requires a GPU-based rendering taking into account the viewer pose and also address local input an actions, but then is projected into a composition layer to be consumed by the run-time.

The above operation point may require different capabilities on the device:

  - Video decoding capabilities: needed by all three cases

  - Rendering capabilities: needed by 2 and 3

  - Scene composition capabilities: needed by 3

Hence, the MeCAR specification would define at least three capabilities:

- video decoding capability

- rendering capabilities based on asset metadata information, describing the mapping from 3D space to the 2D video frame (texture, depth, etc).

- rendering and scene composition capabilities to render scenes and included assets

Video decoding capability would define the requirements as per section xxx of the PD.

Rendering capabilities for asset metadata information may require the ability to render information provided by the video decoder output combined with asset metadata this could include. Asset metadata may for example be:

- Certain SEI messages,

- Atlas metadata as defined in V3C profiles, or

Finally, scene rendering includes the ability to interpret a scene and render the scene and the referenced objects. The may for example

- A gltf scene

- A gltf scene with MPEG-I Scene Description extensions.

For the asset metadata information, MeCAR may evaluate, for example,

- Relevant rendering metadata in SEI messages

- MIV and V-PCC profiles of V3C,.

NOTE: Further clarifications need to be added on 3D video and asset metadata. The terminology ‘operation points’ can be further refined to match the terminology agreed in MeCAR. Rendering capabilities for devices need to be further studied.

- End of Changes

# Proposal

The proposal is to agree on defining three operation points, video and rendering metadata operation points, in IF-4 of the MeCAR architecture and include the changes from Section 2 to the MeCAR Permanent document to record the agreement. Changes 1 and 2 are informative and Change 3 is to be used as basis for future work.

Contact: Lukasz Kondrad, Saba Ahsan, Igor Curcio, Nokia Technologies, Finland. Emails: ífirstname. lastnameý@nokia.com.