3GPPSA4 #123	S4-230726

Athens, 17-21 April 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | pseudo | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Improvements and Corrections to edge and dynamic policy procedures in SR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 2023-4-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document 1. improves the architecture, 2. adds an overview call flow, and 3. correct the use of edge processing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 5.1.3Reference architecture - 5.2.1 a new section on overall call flow - 7: correct the use of edge provisioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of adequate specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1. The architecture can benefits from a description of SR-5 or equivalent. 2. Since we added the edge processing, the use of edge processing and split rendering needed to be demonstrated. The new call flow ties all call flows of the TS together. 3. The edge provisioning can be client or network driven. Also other edge provisioning can be used with the split-rendering. The text is corrected accordingly. |  |  |  |  |  |  |  |  |

CHANGE

5.1.3	End-to-End Architecture

Figure 5.1-3 – Split management architecture

As shown in Figure 5.1.3:

1. The 5G Application Providers (AP) provisions the split-rendering through SR-1.

2. In the use cases in which the AP is involved in the media delivey, the SR-2 interface is used for this purpose.

3.The communication between AF and SRS is through SR-3.  This interface is out of the scope of this document. This interface may for instance include the EDGE-3 interface.

4. The signaling as well as the media delivery between SRC and SRS is though SR-4.

5. The AF may provide the split-rendering information to the Media Session Handler defined by the corresponding parent interface (RTC-5), defined in  TS26.506.

6. The SRC discovers the client media capabilities through the SR-7 interface. This interface is out of the scope of this document.

7. The 5G Application and AP interact through SR-8. This interface is out of the scope of this document.

CHANGE 2

### 5.2.1	Call flow for edge server and split rendering session setup

Figure 5.2.4-1 demonstrates a general call flow for split-rendering.

Figure 5.2.1-1: High-level call flow for split-rendering

Steps:

- In this optional step, the Application Provider requests and sets up the edge server(s) used for the split-rendering as described in TS 26.501 clauses 8.1 or 8.2. The Application provider may use any other method to allocation edge servers, or leave it to the MNO to set up appropriate edge servers to run the split-rendering process.

- The Application Provider provisions the split-rendering session using SR-1 and SR-3, as defined in call flows of clauses 5.2.1.1 or 5.2.1.2. If the edge servers were provisioned in step 1, the edge servers ids are provided in this session to employ them for split-rendering.

NOTE: In the case of the client-driven edge management (TS 26.501 8.1), only the client-driven split-rendering (5.2.1.1) is applicable.

- The split-rendering session is set up according to clause 5.2.2.

CHANGE 3

### Split-Rendering Provisioning API

### 7.2.2.1	Overview

This clause specifies the API that the AP uses at interface SR-1 to create and manage Provisioning sessions for media services that use split rendering. Each split rendering configuration is represented by a SplitRenderingConfiguration, for which the resource structure is specified in 7.2.2.2 and the data model is specified in clause 7.2.2.3.

### 7.2.2.2	Resource structure

The Split-Rendering Provisioning API is accessible through this URL base path:

{apiRoot}/3gpp-rtc1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

Table 7.2.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.2.2-1: Operations supported by the Split-Rendering Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Split-Rendering Configuration | split-rendering-configuration | POST | Used to create a Split-Rendering Configuration resource. |
| Retrieve Split-Rendering Configuration |  | GET | Used to retrieve an existing Split-Rendering Configuration. |
| Update Split-Rendering Configuration |  | PUT, PATCH | Used to modify an existing Split-Rendering Configuration. |
| Delete Split-Rendering Configuration |  | DELETE | Used to delete an existing Split-Rendering Configuration. |
| Purge Split-Rendering Configuration cache | split-rendering-configuration/purge | POST | This operation is used to invalidate some or all cached media resources associated with this Split-Rendering Configuration. |

### 7.2.2.3	Data model

#### 7.2.2.3.1	SplitRenderingConfiguration resource

The data model for the SplitRenderingConfiguration resource is specified in table 7.2.2-2 below:

Table 7.2.2-2: Definition of SplitRenderingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| Name | String | 1..1 | A name for this Split Rendering Configuration. |
| Status | Boolean | 1..1 | Indicates whether to this split rendering configuration is active |
| edgeResourceConfigurationId | ResourceId | 0..1 | The identifier of the edge resource configuration that will be used for sessions of this split rendering configuration. |
| policyTemplateId | ResourceId | 1..1 | The identifier of the policy template that will be applied to the sessions of this split rendering configuration. |
| Configuration | Object | 1..1 | Describes the split-rendering configuration currently used by the SRS. Editor’s Note: The syntax and semantics of this element are TBD. |

### 7.2.2.4	Configuration Guidelines for Split Rendering

#### 7.2.2.4.1	Guidelines on Provisioning Session

The ProvisioningSessionType shall be set to “BIDIRECTIONAL”.

The aspId shall be configured and shall be a unique identifier for the Application Service Provider that offers split rendering.

The externalApplicationId shall be a URN that uniquely identifies the application and shall be terminated by the sub-string “+3gpp-sr”. An examples is as follows: “urn:com:example:game+3gpp-sr”.

#### 7.2.2.4.2	Guidelines on Edge Resource Configuration

A split rendering application may use the procedures defined in TS26.512 clause 7.10 to define an edge resource configuration to be used for split-rendering  session. In this case:

- The eligibilityCriteria shall be present and shall have appRequest set to true.

- The easRequirements shall indicate “SR” as the easType and shall include “3gpp-sr” among the easFeatures. The serviceKpi shall be present and indicate the SRS processing and networking capabilities and requirements.

- The easRelocationRequirements shall indicate “RELOCATION_INTOLERANT” in the tolerance field.

#### 7.2.2.4.3	Guidelines on Policy Template

A policy template for the split rendering shall be associated with the split rendering configuration. The QoS specification should include at least the following sub-streams:

- 2 configurations for left and right eye buffer streams

- 1 optional configuration for a depth buffer stream

- 1 configuration for an audio stream

### Dynamic Policy API for Split Rendering

The WebRTC Signaling Server (which potentially maybe part of the Split Rendering Server) shall support the dynamic policy API as defined in X.

Furthermore, the Split Rendering Server shall support the PDU Set marking and should support the End of Burst marking for the RTP streams that are generated by the Split Rendering Server.