3GPP TSG-SA WG4 Meeting #123	S4-230489

Online, 17th April 2023 - 21th April 2023

Source:	China Mobile Com. Corporation

Title:	Colour Conversion Module for Image Processing

Agenda item:	9.5

Document for:	Discussion and Agreement

- Introduction

In this contribution, we proposed an colour conversion module for processing the rendered frames

- Colour Conversion Module

- The rendered frames acquired from XR Runtime are in RGBA formats and includes textures information, which may not be directly accepted by the encoders. In cases, where encoders can’t directly on RGBA, the extended colour conversion module in combination with available acceleration frameworks (e.g., GPU, NDK) is needed to convert from RGBA to YUV.

- Depending on different planar and storage formats, YUV can be further subdivided into YUV I420, YUV420sp (NV12), YUV420sp (NV21) and so on.

A generic YUV format, capable of describing any 4:2:0 chroma subsampled planar or semi planar buffer (but not fully interleaved), is an 8 bits per colour sample. The conversion between RGB and YUV color spaces can be carried out based on the following formula in order to calculate each YUV values, and the alpha_channel can be support as additional transparency information.

Y =  0.299R + 0.587G + 0.114B

U = -0.147R - 0.289G + 0.436B

V =  0.615R - 0.515G - 0.100B

- Due to the size of the rendered frames, this conversion may not be fast enough on the CPU. This module is therefore suggested to be implemented on the GPU.

- An example of color conversion procedures for smartphones or tablets (defined as Device design type 4 in MeCAR)

- descriptions as follows:

- The XR Runtime sends the rendered frame to the color conversion module.

- The color conversion module gets the pixel data of the rendered frame by copying it into a provide bitmap.

- The color conversion module uses the RGB to YUV conversion formula to convert the R, G, B value into the Y, U, V value. The alpha_channel value is set to be 0 as a typical LCD screen of current smartphone devices, used as an optical transmitter, is based on a RGB_888 color space system.

- Finally, the color conversion module follows the YUV storage format to store the Y, U, V values. For example, YUV420sp (NV21) has one luminance plane Y and one plane with V and U values interleaved. Every four Y pixels there is one V and one U. The sampling is every other pixel and every other scan line.

- Proposal

We propose to include section 2 of this document into the MeCAR permanent document as a colour conversion module for image processing.

- References

- 3GPP TR 26.928: "Extended Reality (XR) in 5G".

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

- [3]	3GPP TSG SA WG4 S4-230307, “MeCAR Permanent Document v5.0.0”, February 2023.