Source:	Audio SWG Co-Chair

Title:	Draft Report on Audio SWG Call on 17 March 2023

Agenda Item:	5.1

Executive Summary

The Audio SWG (19 participants, see Annex B) met on 17 March 2023, 14:00 – 17:00 CET.

This meeting was on IVAS.

The meeting outcome is summarized below:

- IVAS

- The contribution S4aA220027 contains proposed updates for IVAS-8a from the Editor; during on-line editing, the revised version 0.7.2 was produced and agreed, as the basis of further work.

  - The contribution S4aA220028 identifies open issues relating to IVAS-8a; it was agreed to perform HL, CL, and MC tasks on voluntary basis without the need for a contract; further input is expected on other specific items (GAL, stereo material, selection of model parameters, sound material, clarification on section 4.5.4).

  - The contribution S4aA220029 contains indicative price information for running subjective listening experiments from the labs; in on-line editing, a column was added with the price information to the table of experiments in IVAS-8a (while the Excel sheet will be kept for eventually modifying the table in future).

  - The contribution S4aA220030 proposed instructions to listeners in DCR tests; the proposed DCR scale will be implemented in IVAS-8a with the Editor’s Note expressing this is a preliminary decision subject of cross-check / verification.

  - The contribution S4aA220031 discussed the use of HP-filter in IVAS-7a; the proposal was agreed and IVAS-7a will be updated accordingly.

  - The contribution S4aA220032 proposed to use WAVE format for collected audio material which was agreed.

  - The contribution S4aA220033 discussed on spatial room impulse recordings, comments / opinions were collected.

- AOB

1.  Opening of the Session

The Audio SWG Co-Chairman, Mr. Imre Varga (Qualcomm), opened the call on 17 March 2023, 14:00 CET. Minutes were taken by the Chairman.

2.   Approval of Agenda and Registration of Documents

The Chairman presented a draft agenda which was agreed (see the agenda in Annex A of the present report).

3.   IVAS

S4aA230027

Presenter: Mr. Milan Jelinek

Discussion:

- On-line editing

Decision: S4aA230027 is revised on-line and agreed version 0.7.2 as the basis of further editing

S4aA230028

Presenter: Mr. Milan Jelinek

Discussion:

- Price of GAL: J. Reimes will check the tasks and come back on this at the next call

- Stereo material: J. Holub stated Mesaqin will not provide stereo material; other labs will check for the next call

- Performing HL, CL and MC tasks on voluntary basis without need for a contract: agreed

- Selection of model parameters, sound material, and clarification to section 4.5.4 of IVAS-8a: to simplify the process, volunteering organizations provide immersive audio material and model parameters to MC; there was a support for this, more details are expected at the next call

- Conclusion: further input is expected on specific items

Decision: S4aA230028 is noted

S4aA230029

Presenter: Mr. Jan Holub

Discussion:

- S. Bruhn: what is the basis of price on MUSHRA tests with headphones

- J. Holub, J. Reimes, T. Stegenborg: main difference between MUSHRA rel DCR is using experienced listeners (more expensive); setup time is significant

- S. Bruhn: do the lab have access to trained listeners or new persons will be needed

- J. Holub, T. Stegenborg: yes they are available but new listeners are possibly needed (more listeners are needed or the already trained ones are not available at that point in time etc); J. Reimes: sufficient number of experienced listeners is available

- M. Jelinek: prepared the updated table (excel sheet) with pricing information

- S. Döhla: how is the price justified as compared to the EVS figures

- T. Stegenborg: EVS tests were 9 years ago, inflation in the meantime; MUSHRA tests are at relatively low cost, DCR will somewhat compensate for this

- Conclusion: we add a column with the price information to the table of experiments in IVAS-8a (possible keeping the excel sheet for further modification separately)

Decision: S4aA230029 is noted

S4aA230030

Presenter: Mr. Markus Multrus

Discussion:

- M. Jelinek: can live with the modified instruction set

- S. Bruhn: in general agreeable; P.800-suppl allows a certain degree of freedom, so we may be able to find some language to be added to express this freedom; also translation to other languages than English is included here

- M. Jelinek: but the meaning should not change; minimize the degrees of freedom at least for English tests; a formulation will be needed

- L. Laaksonen: reservation on the added two sentences (mainly the second one) in the sample instructions; they put a lot of focus on alteration; rendering aspect has to be clarified in parallel, otherwise quite dangerous

- T. Moriya: situation in the presence of background noise – degradation in the speech part should be meant only (not in background noise)

- M. Jelinek: ref to Lasse’s comment, we should use the same renderer (for selection testing)

- E. Norvell: “future immersive telecommunications services using spatial audio” – is “future” needed?

- I. Varga: the proposed addition in the instructions “The reference audio sample defines the expected quality. Any alteration to this expected quality shall be considered as a degradation, even if you personally would prefer the test sample over the reference sample.” may confuse listeners to vote an improvement as degradation and proposed to cross-check / verify before finally agreeing

- S. Bruhn: it may be better to use the original P.800-suppl formulation

- Conclusion: we decide that we will use the same renderer (at least in selection testing); the proposed DCR scale will be implemented in IVAS-8a with the Editor’s Note expressing this is a preliminary decision subject of cross-check / verification – everyone especially test labs are requested to provide feedback whether it will work well, according to their experience

Decision: S4aA230030 is agreed (preliminary decision, subject to cross-check/verification)

S4aA230031

Presenter: Mr. Markus Multrus

Discussion:

- M. Jelinek: did we consider 50 Hz cut-off

- S. Döhla: EVS used 50 Hz

- S. Ragot: for NB-WB-SWB not for FB

- S. Bruhn: we don’t know which HP filters UEs will use; should we rather rely on codec design than on assumptions on how UEs will do capturing

- T. Toftgard: right but we also have references

- M. Multrus: right we don’t know how UEs will do capturing

- T. Toftgard: have the HP filter as optional and collect further input

- Conclusion: the Editor of IVAS-7a will update it accordingly

Decision: S4aA230031 is agreed

S4aA230032

Presenter: Ms. Eleni Fotopoulou

Discussion:

- S. Bruhn: 16 bit file is used

- S. Döhla: be careful, but this is still open

- M. Jelinek: keep things simple – 16 bits

- E. Novell: good idea with WAVE format

- E. Fotopoulou: yes let’s keep things simple

- Conclusion: the proposal is agreed and the Editor is requested to implement it into IVAS-8a

Decision: S4aA230032 is agreed

S4aA230033

Presenter: Mr. Stefan Bruhn

Discussion:

- L. Laaksonen: useful for our work; could you make IRs available

- S. Bruhn: yes we can share them (under NDA)

- J. Reimes: ref 3 is the Farina paper, did you measure with speech signal?

- S. Bruhn: no, ref 2 was followed (noise sequence); concerned about the sine sweep method

- A. Rämö: which kind of noise was used

- S. Bruhn: pink noise which randomizes the phase (constant envelope)

- A. Rämö: we work on this aspect as well, which signal is the best, all of them provide similarly good results

Decision: S4aA230033 is noted

4. Any Other Business

None.

5. Close of the Session

The Chairman thanked the participants for their contributions.

The meeting was closed on 17 March 2023, 17:00 CET.

Annex A (Audio SWG Call Agenda)

- Opening of the conference call

- Approval of the agenda and allocation of documents

- Progress work on IVAS

  - S4aA230027

  - S4aA230028

  - S4aA230029

  - S4aA230030

  - S4aA230031

  - S4aA230032

  - S3aA230033

- AoB

- Close of the conference call

Annex B

Participants

| FIRST NAME | FAMILY NAME | COMPANY |
| --- | --- | --- |
| Imre | Varga | Qualcomm |
| Andrijana | Brekalo | ETSI MCC |
| Eleni | Fotopoulou | FhG IIS |
| Stefan | Bruhn | Dolby |
| Brian | Lee | Dolby |
| Anssi | Rämö | Nokia |
| Hiroyuki | Ehara | Panasonic |
| Huan-yu | Su | Huawei |
| Lasse | Laaksonen | Nokia |
| Milan | Jelinek | VoiceAge |
| Markus | Multrus | FhG IIS |
| Stefan | Döhla | FhG IIS |
| sAnssi | Rämö | Nokia |
| Takehiro | Moriya | NTT |
| Noboru | Harada | NTT |
| Stephane | Ragot | Orange |
| Marek | Szczerba | Philips |
| Frans | De Bont | Philips |
| Vaclav | Eksler | VoiceAge |
| Tommy | Vaillancourt | VoiceAge |
| Erik | Norvell | Ericsson |
| Naotaka | Morita | NTT |
| Jan | Reimes | HEAD acoustics |
| Nien | Wu | Xiaomi |
| Wang | Bin | Xiaomi |
| Tore | Stegenborg-Andersen | Force Tech. |
| Jan | Holub | Mesaqin.com |

Imre Varga, Email: ivarga@qti.qualcomm.com