Source:	VoiceAge Corporation, Fraunhofer IIS

Title:	Test plan proposal for IVAS Selection experiment P800-6, 1 Object

## Document for:	Discussion and Agreeement

## Agenda Item:	7.5

Introduction

EVS-8b SWB clean speech experiment [1] was used as a template for this Experiment setup proposal, which was adapted to reflect the IVAS Test Plan [2].

Proposal

Include the text below into Annex E.6 of IVAS-8a.

References

S4-141319: EVS Permanent Document EVS-8b: Test plans for selection phase including lab task specification, v.1.2.

S4-230453: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4.

S4aA230050: IVAS-7a: Processing plan for selection phase, v.0.8.1.

Practical procedures for subjective testing, ITU-T Handbook, 2011.

---------------

Annex E: P.SUPPL800 Experiments

E.6	Experiment P800-6: 1-Object clean speech test

The purpose of this experiment is to evaluate the performances of the IVAS candidate for encoding, decoding, and rendering of a single object with metadata. The metadata determines the position of the object around the listener in a 3-dimensional space as described in the Scene description section below. The audio is rendered binaurally via headphones.

E.6.1	Database

- 48 kHz sampled speech.

- P.800 sentence pairs, 8 s long.

- Note: P.800 sentence pairs following [4] are assumed in this experiment, including the leading and trailing silence of minimum of 0.5 s. The metadata corresponds to the whole duration of 8 s. This means that for moving objects, only a part of the trajectory corresponds to active speech.

- Artificially created spatial samples following the Scene descriptions below.

- Level adjusted as specified in IVAS-7a (Processing Plan) [3].

E.6.2	Descriptions of different scenes for metadata creation

    - Talker sitting at a table (elevation 0°), at different azimuths:

| Panel | Talker | Azimuth |
| --- | --- | --- |
| 1 | G1 | 0° |
| 2 | G6 | 60° |
| 3 | G5 | 120° |
| 4 | G4 | 180° |
| 5 | G3 | 240° |
| 6 | G2 | 300° |

    - Standing talker (elevation 35°), at different azimuths:

| Panel | Talker | Azimuth |
| --- | --- | --- |
| 1 | G2 | 120° |
| 2 | G1 | 180° |
| 3 | G6 | 240° |
| 4 | G5 | 300° |
| 5 | G4 | 0° |
| 6 | G3 | 60° |

    - Smaller talker (child) walking around a table in the positive sense (counter clockwise), elevation 0°. Azimuth varies continuously for the sentence pair to cover the whole circle starting at:

| Panel | Talker | Starting Azimuth |
| --- | --- | --- |
| 1 | G3 | 0° |
| 2 | G2 | 60° |
| 3 | G1 | 120° |
| 4 | G6 | 180° |
| 5 | G5 | 240° |
| 6 | G3 | 300° |

    - Adult talker walking around a table in the negative sense (clockwise), elevation 35°. Azimuth varies continuously for the sentence pair to cover the whole circle starting at:

| Panel | Talker | Starting Azimuth |
| --- | --- | --- |
| 1 | G4 | 0° |
| 2 | G3 | 60° |
| 3 | G2 | 120° |
| 4 | G1 | 180° |
| 5 | G6 | 240° |
| 6 | G5 | 300° |

    - Elevation displacement: Elevation varies continuously for the sentence pair to cover the interval of -90° to 90°. Azimuth is constant for a sentence pair, but different for different for each sentence pair:

| Panel | Talker | Azimuth |
| --- | --- | --- |
| 1 | G5 | 240° |
| 2 | G4 | 300° |
| 3 | G3 | 0° |
| 4 | G2 | 60° |
| 5 | G1 | 120° |
| 6 | G6 | 180° |

    - Azimuth and elevation displacement. Azimuth and elevation vary continuously. Azimuth varies in positive sense (counter clockwise) to cover the range of 180°, with different starting azimuth for each sentence pair. Elevation varies in negative sense, from 35° to -35°:

| Panel | Talker | Starting Azimuth |
| --- | --- | --- |
| 1 | G6 | 60° |
| 2 | G5 | 120° |
| 3 | G4 | 180° |
| 4 | G3 | 240° |
| 5 | G2 | 300° |
| 6 | G1 | 0° |

E.6.3	Allocation of scenes to talkers to each listening panel

The following table assumes that test Categories correspond to different talkers. Each of the sentences uttered by a certain talker is encoded using different scene. To balance the test, in addition to listeners of each panel listening to all talkers (Categories), and all scenes are also covered in each panel. An example of allocation of sentences to each panel is given in Table below:

Table E.6.1: Allocation of scenes to talkers to each listening panel

| Panel | G1 (M1) | G2 (F1) | G3 (M2) | G4 (F2) | G5 (M3) | G6 (F3) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | S1 (Sc a) | S1 (Sc b) | S1 (Sc c) | S1 (Sc d) | S1 (Sc e) | S1 (Sc f) |
| 2 | S2 (Sc b) | S2 (Sc c) | S2 (Sc d) | S2 (Sc e) | S2 (Sc f) | S2 (Sc a) |
| 3 | S3 (Sc c) | S3 (Sc d) | S3 (Sc e) | S3 (Sc f) | S3 (Sc a) | S3 (Sc b) |
| 4 | S4 (Sc d) | S4 (Sc e) | S4 (Sc f) | S4 (Sc a) | S4 (Sc b) | S4 (Sc c) |
| 5 | S5 (Sc e) | S5 (Sc f) | S5 (Sc a) | S5 (Sc b) | S5 (Sc c) | S5 (Sc d) |
| 6 | S6 (Sc f) | S6 (Sc a) | S6 (Sc b) | S6 (Sc c) | S6 (Sc d) | S6 (Sc e) |

E.6.4	Experiment setup

Tables E.1 to E.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

The test Categories correspond to different talkers.

Table E.6.2: Factors and conditions for Experiment P800-6

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Bitrates | 3 | 13.2, 16.4, 24.4 and 32 kbps |
| DTX | 2 | DTX ON and OFF (32 kbps DTX OFF only) |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Codec references |  |  |
| Codec references | 1 | EVS |
| Bitrates | 3 | 13.2, 16.4, 24.4 and 32 kbps |
| DTX | 2 | DTX ON and OFF (32 kbps DTX OFF only) |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| MNRU | 5 | Q= 15, 23, 31, 39, 47 dB |
| ESDRU | 4 | 0.1, 0.3, 0.5, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Number of talkers (categories) | 6 | 3 male and 3 female |
| Number of speech samples | 7 | 6 + 1 (preliminaries) sentence pairs per talker. |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations (panels) | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | P.SUPPL800 |
| Listening System | 1 | Headphones |

Table E.6.3: Preliminaries for Experiment P800-6

| Trial | Label | Sample | Condition | Bitrate [kb/s] | DTX | FER [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | EVS | 24.4 | Off | 3 |
| 2 |  |  | MNRU 31 | - | - | - |
| 3 |  |  | EVS | 13.2 | On | 0 |
| 4 |  |  | ESDRU 0.5 | - | - | - |
| 5 |  |  | EVS | 32 | Off | 0 |
| 6 |  |  | MNRU 15 | - | - | - |
| 7 |  |  | EVS | 16.4 | Off | 0 |
| 8 |  |  | Direct | - | - | - |
| 9 |  |  | EVS | 13.2 | Off | 3 |
| 10 |  |  | MNRU 47 | - | - | - |
| 11 |  |  | EVS | 24.4 | On | 0 |
| 12 |  |  | ESDRU 0.1 | - | - | - |

Table E.6.4 Test conditions for Experiment P800-6:

| Label | Condition | Bitrate [kb/s] | DTX | FER [%] | Ref condition | ToR |
| --- | --- | --- | --- | --- | --- | --- |
| C01 | Direct |  | - | - | - | - |
| C02 | MNRU 15 |  | - | - | - | - |
| C03 | MNRU 23 |  | - | - | - | - |
| C04 | MNRU 31 |  | - | - | - | - |
| C05 | MNRU 39 |  | - | - | - | - |
| C06 | MNRU 47 |  | - | - | - | - |
| C07 | ESDRU 0.1 |  | - | - | - | - |
| C08 | ESDRU 0.3 |  | - | - | - | - |
| C09 | ESDRU 0.5 |  | - | - | - | - |
| C10 | ESDRU 0.7 |  | - | - | - | - |
| C11 | EVS | 13.2 | Off | 0 | - | - |
| C12 | EVS | 16.4 | Off | 0 | - | - |
| C13 | EVS | 24.4 | Off | 0 | - | - |
| C14 | EVS | 32 | Off | 0 | - | - |
| C15 | EVS | 13.2 | Off | 3 | - | - |
| C16 | EVS | 16.4 | Off | 3 | - | - |
| C17 | EVS | 24.4 | Off | 3 | - | - |
| C18 | EVS | 13.2 | On | 0 | - | - |
| C19 | EVS | 16.4 | On | 0 | - | - |
| C20 | EVS | 24.4 | On | 0 | - | - |
| C21 | CuT | 13.2 | Off | 0 | C11 | NWT |
| C22 | CuT | 16.4 | Off | 0 | C12 | NWT |
| C23 | CuT | 24.4 | Off | 0 | C13 | NWT |
| C24 | CuT | 32 | Off | 0 | C14 | NWT |
| C25 | CuT | 13.2 | Off | 3 | C15 | NWT |
| C26 | CuT | 16.4 | Off | 3 | C16 | NWT |
| C27 | CuT | 24.4 | Off | 3 | C17 | NWT |
| C28 | CuT | 13.2 | On | 0 | C18 | NWT |
| C29 | CuT | 16.4 | On | 0 | C19 | NWT |
| C30 | CuT | 24.4 | On | 0 | C20 | NWT |