Source:	Fraunhofer IIS

Title:	Proposed experiments for IVAS Codec selection testing with Multi-Channel 7.1+4 content and BS.1543 MUSHRA tests

Document for:	Discussion & Agreement

Agenda Item:	7.5

Introduction

In Table 5 of Section 5 in the IVAS-8a Test Plan [1] there is a high-level overview of the experiments to be held using BS.1543 MUSHRA tests. Experiments BS1534-3a and BS1534-3b give an overview of the experiments for Multi-Channel 7.1.4 input as can be seen in the respective rows from Table 5 below.

| Exp | Input format | Source material | Listening environment | Bitrates kbps | FER/jitter | DTX | Headtracking |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS1534-3a | 7.1.4 | Generic Audio | 7.1 + 4 |  | ≤ x% | N | No |
| BS1534-3b | 7.1.4 | Generic Audio | 7.1 + 4 |  | ≤ x% | N | No |

This contribution aims to give a detailed description of the experiments to be held for the selection phase for this input format.

Multi-Channel BS.1543 MUSHRA experiments BS1534-3a, BS1534-3b

Experimental general considerations

Below are proposed outlines for experiments BS1534-3a, BS1534-3b with multi-channel 7.1+4 input/source material. Main factors to be varied in the tests are:

Source material

The source material should meet the requirements defined in Section 4.5.4 for critical generic audio material in [1].

Bit rates

The bitrate range for the 7.1+4 input material is 13.2-512 kbps. According to IVAS-8a [1], there are no P.SUPPL800 tests envisioned for multi-channel input format, since this format is suitable for generic audio material rather than conversational scenarios. On the other hand, as can be seen in the respective table in IVAS-3 [2] there are no suitable reference for comparison in the lower end of the bitrate range. Therefore, the source finds that the two tests allocated for this input format should cover the mid-bitrates and the higher bitrates. For the mid-bitrates it is suggested to cover the first bitrates where EVS SWB is available, namely 128 and 160kbps. For the higher bitrates is suggested to test the highest end of the range namely 384 and 512kbps.

Audio bandwidth

IVAS operation is required for WB, SWB and FB audio input-output. It is not possible to cover all audio bandwidths in the selection tests. In line with a corresponding editor’s note in Pdoc IVAS-3 [2], it is rather suggested to test at the maximum available bandwidth, which is FB.

Reference condition

For the experiments BS1534-3a/BS1534-3b with multi-channel 7.1+4 input format, apart from the reference and hidden reference, one 3.5kHz low-pass anchor condition is considered to be sufficient.

Specifics for Experiments BS-1534-3a: Multi-Channel 7.1+4

Factors and conditions (BS1534-3a Generic Audio)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated with multi-channel 7.1+4 input at 128 and 160 kbps DTX off at 0% FER |
| Codec references |  |  |
| Codec references | 3 | Multi-mono EVS coding multi-channel 7.1+4 input at 11*9.6 kbps, 11*13.2 kbps, 11*16.4 kbps DTX off at 0% FER LFE coded with 9.6kbps NB [2] |
| Other references |  |  |
| Reference | 1 | Direct signal, Nominal input level |
| Hidden Reference | 1 | Direct signal, Nominal input level |
| LP3.5 anchor | 1 | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests. |
| Audio sampling frequency/bandwidth | 2 | 48 kHz/FB |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12 - 16] | Experienced Listeners |
| Randomizations | 10 | Individual per listeners |
| Rating Scale | 1 | Continuous MUSHRA scale from 0-100 [ITU-R BS.1534] |
| Replications | 2 |  |
| Listening System | 1 | High-quality loudspeaker: 7.1+4 overhead speaker setup with the CICP19 configuration |
| Listening Environment | 1 | No room noise |

Specifics for Experiments BS-1534-3b: Multi-Channel 7.1+4 Input

Factors and conditions (BS1534-3b Generic Audio)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated with multi-channel 7.1+4 audio input at 384 and 512 kbps DTX off at 0% FER |
| Codec references |  |  |
| Codec references | 3 | Multi-mono EVS coding multi-channel 7.1+4 input at 11*32 kbps, 11*48 kbps, 11*64 kbps DTX off at 0% FER LFE coded with 9.6kbps NB [2] |
| Other references |  |  |
| Reference | 1 | Direct signal, Nominal input level |
| Hidden Reference | 1 | Direct signal, Nominal input level |
| LP3.5 anchor | 1 | 3.5 kHz lowpass filtered signal, nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests. |
| Audio sampling frequency/bandwidth | 2 | 48 kHz/FB |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12 - 16] | Experienced Listeners |
| Randomizations | 10 | Individual per listeners |
| Rating Scale | 1 | Continuous MUSHRA scale from 0-100 [ITU-R BS.1534] |
| Replications | 2 |  |
| Listening System | 1 | High-quality loudspeaker: 7.1+4 overhead speaker setup with the CICP19 configuration |
| Listening Environment | 1 | No room noise |

# References

S4-230025: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4.

S4-230368: IVAS Permanent Document IVAS-3: Performance Requirements, v.1.0.0.