3GPP TSG-WG SA4 Meeting #123-e	S4-230586

# Online, 17th – 21st April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev |  | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW] Dynamic Policy Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 11th April 2023 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 6.3	Dynamic Policy API

### 6.3.1	Overview

A 5G-RTC AF shall support the Dynamic Policy API over the RTC-3 and RTC-5 interfaces, as defined in the present sub-clause. The Dynamic Policy API allows both the MSH and the trusted ICE or WebRTC Signaling Server AS to request a specific QoS and charging policy to be applied to the data flows of an RTC session. The API defines a set of data models, resources, and the related procedures for the creation and management of the dynamic policy request.

The 5G-RTC AF uses either the Npcf_PolicyAuthorization API over N5, or the Nnef_AFSessionWithQoS over N33 interface to request the allocation of the dynamic policy with the PCF.

### 6.3.2	Resource structure

The Dynamic Policies API is accessible through the following URL base path:

{apiRoot}/3gpp-rtc5/{apiVersion}/dynamic-policies/

The table below specifies the operations and the corresponding HTTP methods that are supported by this API. The sub-resource path specified in the second column shall be appended to the URL base path.

Table 6.3-1: Operations supported by the Dynamic Policies API

| Resource name | Sub-resource path | Allowed HTTP methods | Description |
| --- | --- | --- | --- |
| Dynamic Policies |  | POST | Create a new Dynamic Policy resource. If the operation succeeds, the URL of the created Dynamic Policy Instance resource shall be returned in the Location header of the response. |
| Dynamic Policy | {dynamicPolicyId} | GET | Read a Dynamic Policy resource. |
|  |  | PUT | Replace an existing Dynamic Policy resource. |
|  |  | PATCH | Modify an existing Dynamic Policy resource. |
|  |  | DELETE | Delete an existing Dynamic Policy resource. |

### 6.3.3	Data model

#### 6.3.3.1	DynamicPolicy resource

The DynamicPolicy resource for 5G-RTC is define in the following table.

Table 6.3-2: Definition of Dynamic Policy resource

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| dynamicPolicyId | ResourceId | 1..1 | RO | Unique identifier for this Dynamic Policy. |
| policyTemplateId | ResourceId | 1..1 | C: RW R: RO U: RW | Identifies the Policy Template which should be applied to the application flow(s). |
| provisioningSessionId | ResourceId | 1..1 | C: RW R: RO U: RW | Uniquely identifies Provisioning Session, which is linked to the Application Service Provider. |
| qosSpecification | RTCQoSSpecification | 0..1 | C: RW R: RO U: RW | Describes the network Quality of Service properties of this Dynamic Policy. |

The RTCQoSSpecification object is defined in the following table.

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| mediaIdentifier | String | 1..1 |  | Provides an identifier for the media stream to associate with the corresponding service component in the QoS Policy. |
| marBwDlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Downlink. |
| marBwUlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Uplink. |
| minDesBwDlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Downlink. |
| minDesBwUlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Uplink. |
| mirBwDlBitRate | BitRate | 1..1 |  | Minimum requested bit rate for the Downlink. |
| mirBwUlBitRate | BitRate | 1..1 |  | Minimum requested bandwidth for the Uplink. |
| desLatency | Integer | 0..1 |  | Desire Latency. |
| desLoss | Integer | 0..1 |  | Desired Loss Rate. |
| pduSetMarking | PDUSetMarking | 0..1 |  | A description of the PDU Set and End of Burst marking configuration for the session. In release-18, this shall only be present in the case of an RTP stream. |

The PDUSetMarking object is defined in the following table:

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| headerExtensionVersion | Integer | 1..1 |  | The RTP header extension version. |
| localIdentifier | Integer | 1..1 |  | A unique identifier of the RTP header extension in the scope of the media session. |
| format | Boolean | 0..1 |  | Indicates if a short or a long header extension format is used. When set to false, a short 1-byte header extension format is being used. |
| pduSetSizeActive | Boolean | 0..1 |  | A flag to indicate if the PDU Set size in bytes is present in the RTP header extension. |
| eobMarkingActive | Boolean | 0..1 |  | A flag to indicate if the End of Burst signaling is activated for this RTP stream. |

### 11.5.4	Operations

The policyTemplateId uniquely identifies the Policy Template, to which the Dynamic Policy Instance is associated. An  RTC session is composed of multiple components that correspond to RTP streams or data channels. Multiple RTP media sessions may be multiplexed into a single RTP stream, thus producing a single QoS flow.

The Dynamic Policy resource contains a serviceDataFlowDescription property which contains the service data flow template according to TS 23.503. The ServiceDataFlowDescription shall contain a flowDescription object (including 5-Tuples, Type of Service, Security Parameter Index, etc.).

The qosSpecifcation object shall be present and it shall contain the following properties:

-	marBwDlBitRate or marBwUlBitRate, indicating the maximum requested bit rate by the Media Session Handler.

-	mirBwDlBitRate or mirBwUlBitRate, indicating the minimum requested bit rate by the Media Session Handler.

-	minDesBwDlBitRate or minDesBwUlBitrate, indicating the minimum bit rate desired by the Media Session Handler.

The requested QoS parameters shall be within the limits of the provisioning QoS Policy Tempalte.