## Source:	Intel Corporation

## Title:	[5G_RTP] Understanding of “End of Data Burst indication” (EOBI) for PDU Set

## Agenda Item:	10.8

## Document for:	Discussion

- Introduction

# This contribution tries to understand the requirement of adding “EoBI” to 5G_RTP new header extension.

# Per SA2 agreed on CR3875R14, UPF shall report the QoS flow N6 Jitter measurement report for the UE power saving management.

# Based on the definition from TS23.700-60, the EOB is defined as follows:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time. NOTE 3: A Data Burst can be composed by one or multiple PDU Sets. Burst size (number of bytes), i.e. size of the 'data burst' that a PDU Set a part of. Rationale: It is generally beneficial for NG-RAN to be aware of the amount of data that needs to be transferred up-front as it aids the RRM and can be used to improve the capacity.

# The Table 6.29.3-1 also defines the “default maximum data burst volume (MDBV)” for each 5QI value (Ex. 63000 bytes for visual content, etc). The usage of EOB is indicated by UPF such as:

# “inserting the EOBI (End of Burst indication) in the GTP-U header in the last PDU of the last PDU set based on implementation-specific mechanisms to detect the end of the data burst, or based on explicit indications from the AS. The indication from the AS can be standardized or non-standardized.”

# Observation 1: How to define the EOBI from RTP header extension? Each codec has various profile and layer configurations, it would be required for the RTP packetization to extract high-level syntax parameters to signal the end of PDUs/PDU set. But yet, we have not concluded how PDU/PDU set boundary will be signaled from the new Header extension (per slice/frame? Or dynamic)

# Observation 2: MDBV is the Maximum data burst volume. It is not clear how UPF decides data burst per MDBV for each 5QI.

# Observation 3: Recommendation from RTP payload format (AVC/HEVC/VVC) regarding packetization rule is that non-vcl NAL unit should be using aggregated packet (AP) to include its related VCL NAL units. When an aggregated RTP packet includes a PDU belonging to the end of data burst in a PDU set, the question is how would UPF decide when the data burst starts. For example, an AP includes 2 NAL units, the first NAL unit is the end PDU in a PDU set but belongs to the same RTP packet which also includes the beginning of another PDU set.

In the previous RTC pre-123 offline telco, S4aR230046 proposed the signalling of EOB by adding few bits in the new RTP header extension.

# Observation 3: One confusion is how to group “small” data burst into a “bigger” data burst to be “qualified” as an EOBI. Is it defined only by the size of the “data burst” or is it defined by the specific time period such as a few milliseconds?

# Observation 4: IHMO, data burst shall be detected/reported/identified based on a period of the traffic pattern. That might require the RTP source to do extra computing work before assigning a value in the new RTP header extension.

# 2. Proposal

# It is proposed 5G_RTP should take the above discussion into consideration when fulfilling SA2’s requirement for adding EOBI.