Agenda item: 	10.8

Source: 	InterDigital Communications

Title: 	[5G_RTP] Signaling PDU set importance in RTP HE

Document for:	Discussion and Agreement

# Introduction

In S4aR230057 the RTC working group has agreed that the semantics of the importance/priority field needs to be defined based on the 3GPP codecs (existing and new ones that are included as part of Release 18 work in SA4) in an RTP header extension.

In this contribution, we discuss the syntax and semantics of PDU set importance field signaling in the RTP HE and the guidelines on how to set this field in the RTP HE at the application level.

# Introduction

SA4 agreed to work on the RTP HE for PDU set marking providing the following PDU set information as part of the RTP HE fields as:

- PDU set sequence number (PSSN)

- PDU sequence number (PSN) within a PDU set

- End PDU of a PDU set indication (E)

- PDU set importance (PSI)

- PDU end-of-data burst (EoB) indication

- Optionally, PDU set size (PSS) in bytes

# PDU set importance field

The PDU set importance can be signalled in the PDU Set information RTP header extension.

The syntax and semantics for the importance/priority field (3 bits in length) in the PDU Set information header extension is as follows:

    - Syntax

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=6   |E|EOB| PRI | R |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      PSSize                                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1. RTP header extension using one-byte header format.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x10    |    0x00       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID           |    L=6        |E|EOB| PRI | R |      PSSN

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   PSN     |                      PSSize                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2. RTP header extension using two-byte header format.

    - Semantics

PRI (3 bits): Priority field indication. This field indicates the priority of a PDU set compared with other PDU sets with in the same stream. The lower the value of the priority field the higher the importance. For example, a PDU set with priority value 0 is more important compared to a PDU set with importance value 1.

    - Guidelines

The PDU sets that contain audio data are set with highest importance compared with other media PDU sets. PDU sets that belong to audio media are set with a priority value 0.

The PDU sets that contains the reference frames present in the video bitstream are set with higher priority compared with non-reference frames present in the video bitstream. The Intra Random Access Pictures (IRAP) pictures such as Instantaneous Decoder Refresh (IDR) frames, Clean Random Access (CRA) frames, Broken Link Access (BLA) frames and Gradual decoding refresh (GDR) frames are set very important in a video stream and are set with higher priority. The priority value for such pictures can be set to 0x01.

In video coding, temporal scalability is the option to decode only some of the frames in a video stream instead of the whole stream. This enables a media server to reduce the bitrate sent towards viewers who doesn’t have enough bitrate or CPU to handle the whole stream. Pictures with lowest temporal identifier value as shown in Figure 3 are used as reference pictures in the bitstream and are important for decoding the dependent frames.

Hence PDU sets with TID value 1 (lowest possible value) are set with higher priority. The priority value for such pictures may be set to 0x01 (for IRAP pictures) or 0x02 (for non-IRAP pictures).

PDU sets with higher TID values in the bitstream are set with lower priority compared with the PDU sets with lower TID values.

PDU sets with the highest TID value in the bitstream are set with lowest priority.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|   Type     |   LayerId  | TID |

+-------------+-----------------+

Figure 3. The Structure of the HEVC NAL Unit Header

The following clauses provides the guidelines on setting the priority field in a PDU set RTP header extension for various video codecs.

      - H.264 Codec

In an H.264 bitstream, NAL units with the nal_unit_type field assigned the value 5 (refer to Table 7.1 in AVC specification [1]) are Intra Random Access Pictures (IRAP) pictures. When the Type field value in the NAL Unit header of an RTP packet is 5, then the corresponding PDUs in that PDU set are set with higher priority value for example with 0x01.

The parameter set NAL units such as Sequence Parameter Set (SPS) and Picture Parameter Set (PPS) are important for decoding the bitstream. Therefore, PDU sets with a payload Type field value equal to 7, 8, 13 or 15 (refer to Table 7.1 in AVC specification [1]) in the NAL Unit header of the RTP packet are set with higher priority. The priority value for such PDU sets is set to 0x01.

+---------------+

|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+

|F|NRI|  Type    |

+---------------+

Figure 4. NAL unit type octet in an RTP packet payload

      - HEVC Codec

In an HEVC bitstream, NAL units with the nal_unit_type field assigned a value in the rang 16 to 23 (inclusive) (refer to Table 7.1 in HEVC specification [2]) are Intra Random Access Pictures (IRAP) pictures. When the Type field value in the NAL Unit header of RTP packet is in the range 16 to 23 (inclusive), then the corresponding PDUs in that PDU set are set with higher priority value for example with 0x01.

The parameter set NAL units such as Sequence Parameter Set (SPS), Picture Parameter Set (PPS), Video Parameter Set (VPS) are important for decoding the bitstream. Therefore, PDU sets with payload Type field value in the NAL Unit header of RTP packet in the range 32 to 34 (inclusive) are set with higher priority. The priority value for such PDU sets is set to 0x01.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|   Type      |  LayerId  | TID |

+-------------+-----------------+

Figure 5. The Structure of the HEVC NAL Unit Header

# Proposal

We propose to agree the proposed syntax, semantics, and guidelines for importance field indication to the PDU set information RTP header extension in 5G_RTP permanent document.

# References

- H.264: Advanced video coding specification. https://www.itu.int/rec/T-REC-H.264-202108-I/en

- H.265: High efficiency video coding specification. https://www.itu.int/rec/T-REC-H.265-202108-I/en