Agenda item: 	10.8

Source: 	InterDigital Communications

Title: 	[5G_RTP] Signaling discardability in the PDU Set information RTP HE

Document for:	Discussion and Agreement

# Introduction

In S4aR230057 the RTC working group has agreed that the semantics of the importance/priority field needs to be defined based on the 3GPP codecs (existing and new ones that are included as part of Release 18 work in SA4) in an RTP header extension.

In this contribution, we discuss the syntax and semantics of the PDU set dicardability flag signaling in the RTP HE and the guidelines on how to set this field in the RTP HE at the application level.

# Introduction

SA4 agreed to work on the RTP HE for PDU set marking providing the following PDU set information as part of the RTP HE fields as:

- PDU set sequence number (PSSN)

- PDU sequence number (PSN) within a PDU set

- End PDU of a PDU set indication (E)

- PDU set importance (PSI)

- PDU end-of-data burst (EoB) indication

- Optionally, PDU set size (PSS) in bytes

S2-2303879 [1] defines PDU set importance as the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. The definition also includes a note that states “whether and how PDU Set Importance can span across QoS Flows is FFS”. RAN2 also agreed that it may use the priority or importance indication at a PDU set level for packet discarding in the presence of congestion. To support this, we propose to split the PDU set importance field into two fields:

- PDU set discardability flag (1 bit)

- PDU set priority field (3 bits)

The discardability flag indicates whether to discard a PDU set or not during the congestion time. The PDU set priority field indicates the importance of a PDU set compared to other PDU sets within a QoS flow. The syntax, semantics, and guidelines for the priority field are discussed in another document S4-230612.

# PDU set discardability flag

The PDU set discardability flag can be signalled in the PDU Set information RTP header extension.

The syntax and semantics for the discardability flag (1-bit in length) in the PDU Set information header extension is as follows:

  - Syntax

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=6   |E|EOB| PRI |D|R|      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      PSSize                                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1. RTP header extension using one-byte header format.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x10    |    0x00       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID           |    L=6        |E|EOB| PRI |D|R|      PSSN

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   PSN     |                      PSSize                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2. RTP header extension using two-byte header format.

  - Semantics

D (1 bit): Discardability flag indication. This flag indicates if the PDU set can be discarded without significant impact on the decoding of the media. When the flag is set to 1, the PDU set can be discarded without significant impact on the decoding and reconstruction. When the flag is set to 0, the PDU set should not be discarded.

  - Guidelines

The discardability flag can be signaled in an RTP header extension based on the presence of temporal scalability support in the video bitstream and the NAL unit types present in the bitstream.

    - Temporal scalability

AVC and HEVC codecs includes an improved support of temporal scalability, by inclusion of the signaling of TemporalId in the NAL unit header. The support of temporal scalability comes with the restriction that pictures of a particular temporal sub-layer cannot be used for inter-prediction reference by pictures of a lower temporal sub-layer.

Hence, pictures with highest TemporalId cannot be used as reference pictures and can be discarded at the network level when the throughput is not good, or network conditions are unstable.

The NAL unit header in an RTP packet payload is as shown in Figure 3. TID field is known as nuh_temporal_id_plus1. This field specifies the temporal identifier of the NAL unit plus 1. The value of TemporalId is equal to TID minus 1.

PDU sets with highest TID field value in the NAL Unit header of RTP packet are discardable. The discardability flag in the PDU set header extension for such PDU sets is set to true or 0x1.

PDU sets with any other TID field value in the NAL Unit header of RTP packet are not discardable. The discardability flag for such PDU sets is set to false or 0x0.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|   Type      |  LayerId   | TID |

+-------------+-----------------+

Figure 3. The Structure of the HEVC NAL Unit Header

    - NAL unit types

      - H.264

The NAL unit type octet contains the NRI (nal_ref_idc) field highlighted in Figure 4. A value of 0x00 indicates that the content of the NAL unit is not used to reconstruct reference pictures for inter picture prediction. Such NAL units can be discarded without risking the integrity of the reference pictures. Values greater than 0x00 indicate that the decoding of the NAL unit is required to maintain the integrity of the reference pictures.

+---------------+

|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+

|F|NRI|  Type    |

+---------------+

Figure 4. NAL unit type octet in an RTP packet payload

PDU sets with an NRI field value 0x00 in the NAL Unit header of RTP packet are discardable. The discardability flag in the PDU set header extension for such PDU sets is set to true or 0x1.

PDU sets with an NRI field value greater than zero in the NAL Unit header of RTP packet are not discardable. The discardability flag for such PDU sets is set to false or 0x0.

      - HEVC

For HEVC streams, RASL pictures can be discarded. HEVC provides mechanisms to enable specifying the conformance of a bitstream wherein the originally present RASL pictures have been discarded. Consequently, system components can discard RASL pictures, when needed, without worrying about causing the bitstream to become non-compliant.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|   Type      |  LayerId  | TID |

+-------------+-----------------+

Figure 4. The Structure of the HEVC NAL Unit Header

PDU sets with Type field value equal to 8 or 9 (refer to Table 7.1 in HEVC specification [3]) in the NAL Unit header of RTP packet are RASL pictures and they are discardable. The discardability flag in the PDU set header extension for such PDU sets is set to true or 0x1.

PDU sets with any other Type field value in the NAL Unit header of RTP packet are non-RASL pictures and they are not discardable. The discardability flag for such PDU sets is set to false or 0x0.

# Proposal

We propose to agree the proposed syntax, semantics, and guidelines for discardability flag signaling to the PDU set information RTP header extension in 5G_RTP permanent document.

# References

- S2-2303879, 23.501 CR3759R6 (Rel-18, 'B'): Support of XR and Media Services, Tencent, et al.

- H.264: Advanced video coding specification. https://www.itu.int/rec/T-REC-H.264-202108-I/en

- H.265: High efficiency video coding specification. https://www.itu.int/rec/T-REC-H.265-202108-I/en